3GPP TSG-RAN WG2 #101	Tdoc R2-1802367

Athens, Greece, 26th February – 2nd March 2018	(Resubmission of R2-1800425)

Agenda Item:	10.4.1.3.4

Source:	Ericsson

Title:	Timer-based transition to inactive

Document for:	Discussion

# Introduction

This contribution is a resubmission of R2-1800425.

At the previous meetings there was some progress on the state modelling and state transitions for NR. This contribution attempts to further progress the area of state transition from RRC_CONNECTED to RRC_INACTIVE addressing the aspects related to the usage of a timer for the implicit state transition.

# Discussion

In the past meeting, it has been proposed the support of implicit state transition from RRC_CONNECTED to RRC_INACTIVE by using an inactivity timer (in UE and in gNB): if data is not received/sent for certain period, the timer expires and the UE and gNB state transits from RRC_CONNECTED to RRC_INACTIVE ([1], [2], [3]).

Figure 1: Timer-based transition to RRC_INACTIVE

This approach has some drawbacks:

- It could lead to temporary misalignment between RRC states in UE and the gNB, because the two timers are independent each other and there is no way to synchronize them: gNB and UE might decide differently on when the timer is started depending if the last transmission was heard or not.

During temporary state misalignment the UE might be not reachable.

Four cases are expected:

  - UE in RRC_CONNECTED – gNB in RRC_INACTIVE:

    - DL data: gNB sends RAN paging, but it is not received by UE since it is already in RRC_CONNECTED. gNB must repeat the RAN paging to resume the connection and send data.

    - UL data: data packets are sent to gNB, but they cannot be received because gNB is in RRC_INACTIVE.

  - UE in RRC_INACTIVE – gNB in RRC_CONNECTED:

    - DL data: data packets are directly sent to UE, but they cannot be received because UE is in RRC_INACTIVE

    - UL data: UE resumes the connection (unneeded exchange of RRC messages) and data can be sent

- It requires pre-configuration of the RRC_INACTIVE parameters (e.g. RAN notification area, RAN configured DRX cycle) via an early RRC reconfiguration message, but such parameters could be out-to-date when the UE enters RRC_INACTIVE.

The implicit state transition from RRC_CONNECTED to RRC_INACTIVE by using an inactivity timer could lead to temporary states misalignment between UE and gNB, making the UE unreachable

The RRC_INACTIVE parameters sent via pre-configuration could be out-to-dated when the UE enters RRC_INACTIVE

For these reasons, it is proposed to use the explicit state transition from RRC_CONNECTED to RRC_INACTIVE (i.e. initiated by gNB by sending RRCConnectionRelease kind of message).

Explicit state transition from RRC_CONNECTED to RRC_INACTIVE should be used for the following reasons:
	- The network controls when sending the UE to RRC_INACTIVE, avoiding states misalignment and UE unreachability
- RRC_INACTIVE parameters are always updated because are sent to the UE when the state transition occurs (no earlier reconfigurations needed)

It follows that no timer should be used to move the UE from RRC_CONNECTED to RRC_INACTIVE

No timer should be used to move the UE from RRC_CONNECTED to RRC_INACTIVE

The usage explicit state transition requires sending of the RRCConnectionRelease kind of message whenever the gNB decides to move the UE to RRC_INACTIVE state. This is not expected to be a problem from the efficiency point of view, even in case of frequent transitions, because the size of the RRCConnectionRelease kind of message is small.

Sending the RRCConnectionRelease kind of message for explicit transition to RRC_INACTIVE state is not expected to be a problem from the efficiency point of view, even in case of frequent transitions, because the size of the message is small

We should clarify that the abovementioned proposals about not defining a timer-based solution are about the normal procedure to move UEs from connected to inactive. In another contribution (R2-1802592 – Handling of suspend failure [4]) we have addressed the failure case where network tris tries to send a Suspend message that is not received by the UE, which could cause a state mismatch. Therein, as in LTE, we have proposed to use an inactivity timer that when expired makes the UE notify the higher layer which will trigger a NAS recovery.

# Conclusion

In section 2 we made the following observations:

- The implicit state transition from RRC_CONNECTED to RRC_INACTIVE by using an inactivity timer could lead to temporary states misalignment between UE and gNB, making the UE unreachable

- The RRC_INACTIVE parameters sent via pre-configuration could be out-to-dated when the UE enters RRC_INACTIVE

- Sending the RRCConnectionRelease kind of message for explicit transition to RRC_INACTIVE state is not expected to be a problem from the efficiency point of view, even in case of frequent transitions, because the size of the message is small

Based on the discussion in section 2 we propose the following:

- Explicit state transition from RRC_CONNECTED to RRC_INACTIVE should be used for the following reasons:
- The network controls when sending the UE to RRC_INACTIVE, avoiding states misalignment and UE unreachability
- RRC_INACTIVE parameters are always updated because when sent to the UE at state transition (no earlier reconfigurations needed)

No timer should be used to move the UE from RRC_CONNECTED to RRC_INACTIVE

# References

R2-1712208, “State transition from RRC_CONNECTED to RRC_INACTIVE”, ASUSTeK, Reno, Nevada, USA, 27th November – 1st December 2017

R2-1712567, “Timer based state transmission from CONNECTED to inactive”, Huawei, HiSilicon, Reno, Nevada, USA, 27th November – 1st December 2017

R2-1712797, “Timer-based Inactivation for NR”, InterDigital, Reno, Nevada, USA, 27th November – 1st December 2017

R2-1802592, “Handling of suspend failure”, Ericsson, Athens, Greece, 26th February – 2nd March 2018