3GPP TSG-RAN WG2 #101	Tdoc R2-1802377

Athens, Greece, 26th Feb – 2nd Mar, 2018	Revision of R2-1800325

Agenda Item:	10.2.14

Source:	Ericsson

Title:	TP on 38.300 for RNA Configuration

Document for:	Decision

# Introduction

In RAN2 #99bis a response LS to a RAN3 LS on definition of RAN Notification Area in inactive state was agreed ([1],[2]). In the response, RAN2 indicated endorsement of that RNA should be possible to be defined as either a list of cells, either through RAN Area Codes or via a list of TAI’s.

The current text in section 9.2.2.3 for RAN-based Notification Area (RNA) do not match the agreements obtained during LS exchange between RAN3 and RAN2 in R2-1710036 and R2-1712006.

This contribution include a text proposal to 3GPP TS 38.300 to reflect these three methods.

The contribution is revised into a discussion paper (from a CR).

# Discussion

Adopt the text proposal for inclusion in 3GPP TS 38.300

# Text Proposal

#### 9.2.2.3	RAN-Based Notification Area (RNA)

A UE in the RRC_INACTIVE state can be configured with an RNA, where:

-	the RNA can cover a single or multiple cells, and can be smaller than or equal to the CN registration area;

-	a RAN-based notification area update (RNAU) is periodically sent by the UE and is also sent when the cell reselection procedure of the UE selects a cell that does not belong to the configured RNA.

The RNA for a UE shall only include cells that are also part of the UE CN Registration Area.

There are several different alternatives on how the RNA can be configured:

-	List of cells:

-	A UE is provided an explicit list of cells (one or more) that constitute the RNA.

-	List of RAN areas:

-	A UE is provided (at least one) RAN area ID, where a RAN area is a subset of a CN Tracking Area. A RAN area ID includes a TAI + a RAN Area Code, RANAC;

-	A cell broadcasts a TAI and a RANAC in the system information so that a UE knows which area the cell belongs to.

-	List of TAI’s:

-	A UE is provided with at least one TAI from the TAI list of the CN registration area

NG RAN may provide different RNA definitions to different UEs but not mix different definitions to the same UE. UE shall support all options.

# Conclusion

Based on the discussion in section 2 and 3 we propose the following:

Proposal 1	Adopt the text proposal for inclusion in 3GPP TS 38.300

# References

R2-1712006, Reply LS on definition of RAN Notification Area in inactive state, RAN2, 3GPP TSG RAN WG2#99bis, Prague, Czech Republic, 9 - 13 October, 2017

R2-1710036, LS on definition of RAN Notification Area in inactive state (R3-173427), RAN3, 3GPP TSG RAN WG2#99bis, Prague, Czech Republic, 9 - 13 October, 2017