3GPP TSG-RAN WG2 #101	Tdoc R2-1802718

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	9.15  Highly Reliable Low Latency Communication for LTE

Source:	Ericsson

Title:	Support periodic URLLC traffic with SPS

Document for:	Discussion, Decision

# Introduction

In the LS from RAN1 [1], for URLLC, “Reliability” is defined as the success probability R of transmitting X bits within L seconds, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface, at a certain channel quality Q (e.g., coverage-edge). Moreover, the latency bound L includes transmission latency, processing latency, retransmission latency and queuing/scheduling latency (including scheduling request and grant reception if any).

In this paper, we discuss how to support periodic URLLC traffic with SPS.

# Discussion

Example use cases of URLLC include, for example, wireless control of industrial manufacturing or production processes, remote medical surgery, distribution automation in a smart grid, transportation safety, etc. One salient difference from human-centric traffic as in eMBB data is that the URLLC traffic may be periodically generated. For example [2], in a closed-loop control application, the controller periodically receives measurement from sensors or submits instructions to actuators. For example, the controller sends the instructions to the actuators which return a response within a cycle time (i.e. after performing the instructed actions the devices respond to the controller which then needs to send corresponding feedback (e.g. user plane data to be delivered to an application server in the IP network) to the eNB using URLLC metrics). In these applications, SPS, which allocates periodic resources for an UE, can be used to send the user plane data.

Periodic URLLC traffic can be configured to be transmitted by SPS.

The question arises on how the network can configure a proper SPS allocation for the UL periodic URLLC traffic. One may argue that the periodicity of the traffic can be conveyed by QCI (QoS Class Identifier) in LTE when the data radio bearer is set up.  However, the packet arrival time is unknown i.e. there is no coordination between the uplink radio resources made available to a UE according to the SPS periodicity and when a sensor/actuator/controller has user plane data (MAC PDU) available for transmission. A misalignment between the arrival time and the allocated transmission opportunity leads to a waiting time for UL SPS resources for data transmissions. This waiting time adds to the overall packet delay even though the time taken for the actual transmission of the data may satisfy the corresponding URLLC delay attribute. The overall latency may exceed the ultra-low latency requirement for URLLC or it may leave too short remaining time for physical layer to reach the ultra-high reliability requirement, which is guaranteed by time-diversity techniques, such as HARQ-less repetitions. One example is illustrated in the below.

Figure 1 Extra latency due to: misalignment of MAC PDU arriving time and SPS configuration

A mis-alignment of SPS configuration can lead to a constant delay for periodic URLLC traffic.

In LTE release 14, an SPS enhancement for V2X was added so that the UE can provide the network the traffic pattern of a UL logical channel, see below in the UEAssistanceInformation RRC message.

sps-AssistanceInformation-r14		SEQUENCE {

trafficPatternInfoListSL-r14		TrafficPatternInfoList-r14		OPTIONAL,

trafficPatternInfoListUL-r14		TrafficPatternInfoList-r14		OPTIONAL

}			OPTIONAL,

TrafficPatternInfoList-r14 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-r14

TrafficPatternInfo-r14 ::=	SEQUENCE {

trafficPeriodicity-r14			ENUMERATED {

sf20, sf50, sf100, sf200, sf300, sf400, sf500,

sf600, sf700, sf800, sf900, sf1000},

timingOffset-r14					INTEGER (0..10239),

priorityInfoSL-r14					SL-Priority-r13								OPTIONAL,

logicalChannelIdentityUL-r14		INTEGER (3..10)								OPTIONAL,

messageSize-r14					BIT STRING (SIZE (6))

}

field descriptions

logicalChannelIdentityUL Indicates the logical channel identity associated with the reported traffic pattern in the uplink logical channel.

messageSize Indicates the maximum TB size based on the observed traffic pattern. The value refers to the index of TS 36.321 [6, table 6.1.3.1-1].

sps-AssistanceInformation Indicates the UE assistance information to assist E-UTRAN to configure SPS.

trafficPatternInfoListUL This field provides the traffic characteristics of uplink logical channel(s).

trafficPeriodicity This field indicates the estimated data arrival periodicity in a SL/UL logical channel. Value sf20 corresponds to 20 ms, sf50 corresponds to 50 ms and so on.

timingOffset This field indicates the estimated timing for a packet arrival in a SL/UL logical channel. Specifically, the value indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds.

A similar mechanism can be used but with more granular information on the traffic pattern. For example, the period of URLLC data might very well be smaller than 20 ms, and the granularity of the timing offset shall be smaller than one millisecond to satisfy the requirement of 1 ms latency in URLLC.

In RRC IE TrafficPatternInfo, add new values in the fields trafficPeriodicity and timingOffset to allow shorter periodicity and more granular timing offset.

# Conclusion

In section 2 we made the following observations:

Observation 1	Periodic URLLC traffic can be configured to be transmitted by SPS.

Observation 2	A mis-alignment of SPS configuration can lead to a constant delay for periodic URLLC traffic.

Based on the discussion in section 2 we propose the following:

Proposal 1	In RRC IE TrafficPatternInfo, add new values in the fields trafficPeriodicity and timingOffset to allow shorter periodicity and more granular timing offset.

# References

R1-1719210, LS on Ultra Reliable Low Latency Communication for LTE

3GPP TR 22.862, Feasibility Study on New Services and Markets Technology Enablers for Critical Communications, Stage 1.