3GPP TSG-RAN WG2 #101	Tdoc R2-1803059

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.4.1.6.7

Source:	Ericsson

Title:	TP to 38.331 on System Information

Document for:	Discussion, Decision

# Introduction

There are many open issues in the sub-clauses for System Information that are included in TS 38.331. Some of the open issues are awaiting further progress before they can be concluded, and some are related to topics that are being addressed by different contributions. There are however some open issues that are related to how the procedures should be specified.

This contribution discusses, and contains a Text Proposal for, specification related open issues, as well as some missing already agreed parts, in the System Information parts of TS 38.331. The latest updates to 38.331 are included in the CR in [1]. The Text Proposal in this document is based on that CR, where the track changes in sub-clauses 5.2.2.3.1 and 5.2.2.4.1 have been accepted.

# Discussion

The MIB content has been concluded in TS 38.331. Sub-clause 5.2.2.4.1, describing the UE actions upon receiving the MIB can thus be specified.

Based on pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position in the MIB, the UE determines where to find the SIB1.

The UE action when cellBarred is set to barred includes to check intraFreqReselection (also in the MIB) to determine whether the UE is allowed to perform cell-reselection to another cell on the same frequency. The specific procedure is further described in TS 38.304.

Specify the UE actions upon reception of the MIB in sub-clause 5.2.2.4.1 in TS 38.331.

Sub-clause 5.2.2.4.2 is specified as if the scheduled SI messages are either all provided on-demand or broadcasted. This will often not be the case as some of the SI messages may be broadcasted by the network whereas some are only provided on-demand. This has been addressed in the Editor’s Note “To be updated how to capture the UE behaviour when some required SIBs are from broadcast and other required SIBs through SI request.”

It is proposed to update the procedure specification in 5.2.2.4.2 to describe that SIB1 may contain scheduling information for both SI messages that are broadcasted and SI messages that are provided on-demand, that the UE needs to acquire.

Update the procedure specification in sub-clause 5.2.2.4.2 in TS 38.331 to cover that the UE may need to acquire both SI messages that are broadcasted and SI messages that are provided on-demand.

The logical channel configuration for BCCH, which the UE should apply in order to acquire the MIB and SIB1, is missing in TS 38.331. It is thus proposed to add it to sub-clause 9.1.1. The same configuration as for LTE is valid for NR, i.e. that Transparent Mode (TM) is used for both the RLC and MAC configuration whereas the PDCP configuration is not applicable.

The BCCH configuration should be added in sub-clause 9.1.1 in TS 38.331.

It should be specified in TS 38.331 (sub-clause 5.2.2.3.1) that the UE shall apply the BCCH configuration

The Text Proposal in section 3 should be included in TS 38.331.

# Text Proposal to TS 38.331

START OF FIRST CHANGES

##### 5.2.2.3.1	Acquisition of MIB and SIB1

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.X;

1>	if the cell is a PSCell:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	perform the actions specified in section 5.2.2.4.1;

1>	else:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2> if the UE is unable to acquire the MIB;

3> follow the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in section 5.2.2.4.1;

2>	acquire the SystemInformationBlockType1 as specified in [X];

2>	if the UE is unable to acquire the SystemInformationBlockType1:

3> follow the actions as specified in clause 5.2.2.5;

2>	else:

3>perform the actions specified in section 5.2.2.4.2.

Editor’s Note: Reference to RAN1 [X] specification may be used for the scheduling of SIB1.FFS_Standalone

END OF FIRST CHANGES

START OF SECOND CHANGES

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or if the UE is in RRC_INACTIVE or if the UE is in RRC_CONNECTED while T311 is running: [FFS]

2> if the cellBarred in the acquired MIB is set to barred:

3>	consider the cell as barred in accordance with TS 38.304 [FFS];

3>	if intraFreqReselection is set to notAllowed:

4> consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [X].

3>	else:

4> consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [X].

2>	else:

3>	apply the  pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position to acquire SIB1.

##### 5.2.2.4.2	Actions upon reception of the SystemInformationBlockType1

Upon receiving the SystemInformationBlockType1 the UE shall:

1>	store the acquired SIB1;

1>	if the UE has a stored valid version of the required SIB(s) associated with the systemInfoAreaIdentifier and systemInfoValueTag/systemInfoConfigurationIndex in the acquired SIB1:

2>	use that stored version of the SIB;

1>	if the schedulingInfoList in the SIB1 message indicates that one or more SI message(s), with a required SIB, is/are only provided on request:

2>	trigger a request to acquire the SI message(s) as defined in sub-clause 5.2.2.3.3;

1>	 if the schedulingInfoList in the SIB1 message indicates that one or more SI message(s), with a required SIB, is/are periodically broadcasted:

2>	acquire the SI message(s) as defined in sub-clause 5.2.2.3.2.

Editor’s Note: [FFS_Standalone Whether there is an additional indication that an on-demand SI is actually being broadcast at this instant in time]

Editor’s Note: To be updated when content of the SystemInformationBlockType1 has been agreed. FFS_Standalone.

END OF SECOND CHANGES

START OF THIRD CHANGES

#### 9.1.1.X	BCCH configuration

Parameters

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| PDCP configuration | N/A |  |  |
| RLC configuration | TM |  |  |
| MAC configuration | TM |  |  |

NOTE:	RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5.

END OF THIRD CHANGES

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Specify the UE actions upon reception of the MIB in sub-clause 5.2.2.4.1 in TS 38.331.

Proposal 2	Update the procedure specification in sub-clause 5.2.2.4.2 in TS 38.331 to cover that the UE may need to acquire both SI messages that are broadcasted and SI messages that are provided on-demand.

Proposal 3	The BCCH configuration should be added in sub-clause 9.1.1 in TS 38.331.

Proposal 4	It should be specified in TS 38.331 (sub-clause 5.2.2.3.1) that the UE shall apply the BCCH configuration

Proposal 5	The Text Proposal in section 3 should be included in TS 38.331.

# References

R2-18xxxx Draft 38331 CR after RAN2 1801 AH