3GPP TSG-RAN WG2 #101	Tdoc R2-1803161

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.3.2.3

Source:	Ericsson

Title:	PDCP duplication impact to RLC

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss the remaining issues of impact of PDCP duplication to RLC. In RAN2#AH1801 the following agreements were made:

Agreements:

- Upon packet duplication activation, only PDCP SDUs/PDUs not submitted to lower layers are duplicated.

- Baseline is that packed duplication is support for data PDUs

- For packet duplication, when to submit PDCP PDUs to lower layers is up to UE implementation.  FFS on UE behaviour when duplication is deactivated and what PDCP data volume is used.

- After packet duplication is activated, for DC duplication, PDCP data volume is indicated to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the secondary RLC entity

- After packet duplication is activated, for CA duplication, PDCP data volume is included in both the LCG associated with the primary RLC entity and the LCG associated with the secondary RLC entity.

- Packet duplication does not impact RLC data volume

- The UE shall discard packets that have been acknowledged by RLC in the other RLC leg.   PDCP should indicate to the other associated RLC entity to discard the corresponding PDCP PDU.  RLC procedures and PDCP discard procedures are not impacted by this agreement.

- The deactivated RLC entity is not re-established

- For CA and DC upon deactivation of PDCP data duplication, the UE transmitting PDCP entity should indicate to lower layers to discard all PDCP PDUs provided for duplicate transmission to the secondary RLC entity

- When configuring duplication, RRC can also set the initial state (active or inactive) for DRBs.

- If SRB is configured to use duplication, the state is always active

- FFS Duplication is supported for SRBs for CA

# Discussion

When duplication in CA is deactivated, it had been agreed that the carrier transmission restrictions are lifted. Given that with the agreement from AH#1801, RLC entities are not re-established, some remaining data may still be transmitted in the secondary RLC entity after deactivation. This would be data currently in a retransmission process, i.e. in flight. New RLC data not yet transmitted is discarded as agreed at duplicate deactivation. We believe that for this few remaining re-transmission data it does not matter which carrier is used for transmission, i.e. that the lifting of the carrier transmission restrictions should also apply to this remaining data (no special handling needed).

After duplication deactivation, remaining data on RLC may be transmitted on any carrier (i.e. carrier transmission restrictions not applied anymore).

Another open issue in relation of duplication and RLC, is how the RLC logical channel triggering maximum number of retransmissions should be considered. RLC indicates the maximum number of retransmissions reached to higher layers. It is noteworthy that this case previously lead in both RLF and SCG RLF to reset of the corresponding MAC entity. This is not the case in duplication, i.e. transmission of data via the RLC entity not reaching the maximum retransmissions continues. This leads to the new situation that a RLC entity that is suspended due to maximum number of retransmission reached continues to be considered by MAC. It should be clarified that data of the suspended RLC logical channel is not considered on MAC, i.e. simply ignored as if there not such a logical channel.

When an RLC logical channel is suspended due to maximum number of retransmissions reached, remaining data on that logical channel is ignored by MAC, i.e. MAC ignores suspended logical channels in LCP and BSR calculation.

# Conclusions

Based on the discussion in section 2 we propose the following:

Proposal 1	After duplication deactivation, remaining data on RLC may be transmitted on any carrier (i.e. carrier transmission restrictions not applied anymore).

Proposal 2	When an RLC logical channel is suspended due to maximum number of retransmissions reached, remaining data on that logical channel is ignored by MAC, i.e. MAC ignores suspended logical channels in LCP and BSR calculation.