3GPP TSG-RAN2 NR Ad Hoc	R2-1803259

# Athens, Greece, 26th Feb - 2nd Mar, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | abcd | rev | - | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on speed dependent mobility in TS 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current LTE specification, speed dependent mobility measurement is supported for NR frequency measurement. It would be not reasonable obviously that MN can configure NR frequency measurement with supporting speed dependent mobility while SN cannot in EN-DC. Therefore, speed dependent mobility should also be supported in NR. For NR, the cell layout could be homogeneous or heterogeneous, as agreed in TR 38.804. It needs to support mobility in high frequency deployment scenarios which results in smaller cell coverage. Besides, due to the large path loss and fragile channel behaviour, handover in NR may be more frequently than in LTE. Supporting speed dependent mobility in NR also will improve NR mobility performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 5.5 and 6.3.2, capture speed state parameters in NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR cannot support speed dependent mobilty . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 6.3.2 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

#### 5.5.2.1	General

The network applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig, it includes a measObject for each serving frequency;

Editor’s Note: FFS How the procedure is used for CGI reporting.

The UE shall:

1>	if the received measConfig includes the measObjectToRemoveList:

2>	perform the measurement object removal procedure as specified in 5.5.2.4;

1>	if the received measConfig includes the measObjectToAddModList:

2>	perform the measurement object addition/ modification procedure as specified in 5.5.2.5;

1>	if the received measConfig includes the reportConfigToRemoveList:

2>	perform the reporting configuration removal procedure as specified in 5.5.2.6;

1>	if the received measConfig includes the reportConfigToAddModList:

2>	perform the reporting configuration addition/ modification procedure as specified in 5.5.2.7;

1>	if the received measConfig includes the measIdToRemoveList:

2>	perform the measurement identity removal procedure as specified in 5.5.2.2;

1>	if the received measConfig includes the measIdToAddModList:

2>	perform the measurement identity addition/ modification procedure as specified in 5.5.2.3;

1>	if the received measConfig includes the measGapConfig:

2>	perform the measurement gap configuration procedure as specified in 5.5.2.9;

1>	if the received measConfig includes the s-MeasureConfig:

2>	if s-MeasureConfig is set to ssb-rsrp, set parameter ssb-rsrp of s-MeasureConfig within VarMeasConfig to the lowest value of the RSRP ranges indicated by the received value of s-MeasureConfig;

2>	else, set parameter csi-rsrp of s-MeasureConfig within VarMeasConfig to the lowest value of the RSRP ranges indicated by the received value of s-MeasureConfig;

1>	if the received measConfig includes the speedStatePars:

2>	set the parameter speedStatePars within VarMeasConfig to the received value of speedStatePars;

NEXT CHANGE

### 5.5.x	Measurement related actions

5.5.x.a	Mobility states of a UE

Besides Normal-mobility state a High-mobility and a Medium-mobility state are applicable if the parameters (TCRmax, NCR_H, NCR_M and TCRmaxHyst) are with VarMeasConfig.

State detection criteria:

Medium-mobility state criteria:

-	If number of cell changes (handover) during time period TCRmax exceeds NCR_M and not exceeds NCR_H

High-mobility state criteria:

-	If number of cell changes during time period TCRmax exceeds NCR_H

The UE shall not count consecutive changes between same two cells into mobility state detection criteria if same cell is changed just after one other change.

State transitions:

The UE shall:

-	if the criteria for High-mobility state is detected:

-	enter High-mobility state.

-	else if the criteria for Medium-mobility state is detected:

-	enter Medium-mobility state.

-	else if criteria for either Medium- or High-mobility state is not detected during time period TCRmaxHyst:

-	enter Normal-mobility state.

### If the UE is in High- or Medium-mobility state, the UE shall apply the speed dependent scaling rules as defined in subclause 5.5.x.b.

#### 5.5.x.b	Speed dependant scaling of measurement related parameters

The UE shall adjust the value of the following parameter configured by the NR depending on the UE speed: timeToTrigger. The UE shall apply 3 different levels, which are selected as follows:

The UE shall:

1>	perform mobility state detection using the mobility state detection as specified in subclause 5.5.x.a with the following modifications:

2>	applying the parameter applicable for RRC_CONNECTED as included in speedStatePars within VarMeasConfig;

1>	if high mobility state is detected:

2>	use the timeToTrigger value multiplied by sf-High within VarMeasConfig;

1>	else if medium mobility state is detected:

2>	use the timeToTrigger value multiplied by sf-Medium within VarMeasConfig;

1>	else:

#### 2>	no scaling is applied;

NEXT CHANGE

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEAS-CONFIG-START

MeasConfig ::=							SEQUENCE {

-- Measurement objects

measObjectToRemoveList					MeasObjectToRemoveList											OPTIONAL,

measObjectToAddModList					MeasObjectToAddModList											OPTIONAL,

-- Reporting configurations

reportConfigToRemoveList				ReportConfigToRemoveList										OPTIONAL,

reportConfigToAddModList				ReportConfigToAddModList										OPTIONAL,

-- Measurement identities

measIdToRemoveList						MeasIdToRemoveList												OPTIONAL,

measIdToAddModList						MeasIdToAddModList												OPTIONAL,

-- Other parameters

-- s-Measure config

s-MeasureConfig							CHOICE {

ssb-rsrp								RSRP-Range,

csi-rsrp								RSRP-Range

} 																										OPTIONAL,

speedStatePars			CHOICE {

release								NULL,

setup								SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

timeToTrigger-SF					SpeedStateScaleFactors

}

}														OPTIONAL,	-- Need N

quantityConfig						QuantityConfig														OPTIONAL,

-- Placehold for measGapConfig

measGapConfig							MeasGapConfig													OPTIONAL

}

MeasObjectToRemoveList ::=				SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=					SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=			SEQUENCE (SIZE (1..maxNrofReportConfigId)) OF ReportConfigId

-- TAG-MEAS-CONFIG-STOP

-- ASN1STOP

NEXT CHANGE

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency and inter- RAT mobility.

MeasResults information element

-- ASN1START

-- TAG-MEAS-RESULTS-START

MeasResults ::=								SEQUENCE {

measId										MeasId,

measResultServingFreqList					MeasResultServFreqList,

measResultNeighCells						CHOICE {

measResultListNR							MeasResultListNR,

measResultListEUTRA							MeasResultListEUTRA,

...

}																														OPTIONAL

}

MeasResultServFreqList ::=					SEQUENCE (SIZE (1..maxNrofSCells)) OF MeasResultServFreq

MeasResultServFreq ::=						SEQUENCE {

servFreqId									ServCellIndex																OPTIONAL,

measResultServingCell						MeasResultNR,

measResultBestNeighCell						MeasResultNR

}

MeasResultListNR ::=						SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=							SEQUENCE {

physCellId									PhysCellId																	OPTIONAL,

--FFS: Details of cgi info

cgi-Info									ENUMERATED {ffsTypeAndValue},

measResult									SEQUENCE {

cellResults									SEQUENCE{

resultsSSBCell								ResultsSSBCell														OPTIONAL,

resultsCSI-RSCell 							ResultsCSI-RSCell													OPTIONAL

},

rsIndexResults								SEQUENCE{

resultsSSB-Indexes							ResultsPerSSB-IndexList												OPTIONAL,

resultsCSI-RSIndexes						ResultsPerCSI-RSIndexList 											OPTIONAL

}																													OPTIONAL

}

}

ResultsSSBCell ::= 							SEQUENCE {

ssb-Cellrsrp								RSRP-Range																	OPTIONAL,

ssb-Cellrsrq								RSRQ-Range																	OPTIONAL,

ssb-Cellsinr								SINR-Range																	OPTIONAL

}

ResultsCSI-RSCell ::= 						SEQUENCE {

csi-rs-Cellrsrp								RSRP-Range																	OPTIONAL,

csi-rs-Cellrsrq								RSRQ-Range																	OPTIONAL,

csi-rs-Cellsinr								SINR-Range																	OPTIONAL

}

ResultsPerSSB-IndexList ::= 					SEQUENCE (SIZE (1..maxNrofSSBs)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::= 						SEQUENCE {

ssb-Index									SSB-Index,

ss-rsrp										RSRP-Range																	OPTIONAL,

ss-rsrq										RSRQ-Range																	OPTIONAL,

ss-sinr										SINR-Range																	OPTIONAL

}

ResultsPerCSI-RSIndexList ::= 				SEQUENCE (SIZE (1..maxNrofCSI-RS)) OF ResultsPerCSI-RSIndex

ResultsPerCSI-RSIndex ::= 					SEQUENCE {

csi-rsIndex									CSI-RSIndex,

csi-rsrp									RSRP-Range																	OPTIONAL,

csi-rsrq									RSRQ-Range																	OPTIONAL,

csi-sinr									SINR-Range																	OPTIONAL

}

-- TAG-MEAS-RESULTS-STOP

-- ASN1STOP

#### –	MobilityStateParameters

The IE MobilityStateParameters contains parameters to determine UE mobility state.

MobilityStateParameters information element

-- ASN1START

MobilityStateParameters ::=			SEQUENCE {

t-Evaluation						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

t-HystNormal						ENUMERATED {

s30, s60, s120, s180, s240, spare3, spare2, spare1},

n-CellChangeMedium					INTEGER (1..16),

n-CellChangeHigh					INTEGER (1..16)

}

-- ASN1STOP

| MobilityStateParameters field descriptions |  |
| --- | --- |
| n-CellChangeHigh The number of cell changes to enter high mobility state. Corresponds to NCR_H in subclause 5.5.x.a. |  |
| n-CellChangeMedium The number of cell changes to enter medium mobility state. Corresponds to NCR_M in subclause 5.5.x.a. |  |
| t-Evaluation The duration for evaluating criteria to enter mobility states. Corresponds to TCRmax in subclause 5.5.x.a. Value in seconds, s30 corresponds to 30 s and so on. |  |
| t-HystNormal The additional duration for evaluating criteria to enter normal mobility state. Corresponds to TCRmaxHyst in subclause 5.5.x.a. Value in seconds, s30 corresponds to 30 s and so on. |  |
| NEXT CHANGE |  |

#### –	SubcarrierSpacing

The SubcarrierSpacing IE determines the subcarrier spacing.

SubcarrierSpacing information element

-- ASN1START

-- TAG-SUBCARRIER-SPACING-START

-- Check value range! Currently used for subcarrierSpacingCommon (SIB1, Msg2, Msg4

-- FFS: Replace this 2-bit field by a 1-bit field: ENUMERATED {sc1, sc2}. Then define that sc1 = 15 kHz and sc2 = 30 kHz

-- when carrier frequency < 6 GHz and sc1 = 60 kHz and sc2 = 120 kHz when carrier frequency is > 6GHz?

SubcarrierSpacing ::= 					ENUMERATED {kHz15, kHz30, kHz60, kHz120}

-- 15 or 30 kHz (<6GHz), 120 and 240 kHz (>6GHz).

SubcarrierSpacingSSB ::= 				ENUMERATED {kHz15, kHz30, kHz120, kHz240}

-- FFS_CHECK: Can probably be removed since PRACH Msg1 uses the SubcarrierSpacing values above.

SubcarrierSpacingRACH ::= 				ENUMERATED {ffsTypeAndValue}

-- TAG-SUBCARRIER-SPACING-STOP

-- ASN1STOP

#### –	SpeedStateScaleFactors

The IE SpeedStateScaleFactors concerns factors, to be applied when the UE is in medium or high speed state, used for scaling a mobility control related parameter.

SpeedStateScaleFactors information element

-- ASN1START

SpeedStateScaleFactors ::=			SEQUENCE {

sf-Medium							ENUMERATED {oDot25, oDot5, oDot75, lDot0},

sf-High								ENUMERATED {oDot25, oDot5, oDot75, lDot0}

}

-- ASN1STOP

SpeedStateScaleFactors field descriptions

sf-High The concerned mobility control related parameter is multiplied with this factor if the UE is in High Mobility state as defined in subclause 5.5.x.a. Value oDot25 corresponds to 0.25, oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on.

sf-Medium The concerned mobility control related parameter is multiplied with this factor if the UE is in Medium Mobility state as defined in subclause 5.5.x.a. Value oDot25 corresponds to 0.25, oDot5 corresponds to 0.5, oDot75 corresponds to 0.75 and so on.

NEXT CHANGE

#### –	VarMeasConfig

The UE variable VarMeasConfig includes the accumulated configuration of the measurements to be performed by the UE, covering intra-frequency, inter-frequency and inter-RAT mobility related measurements.

VarMeasConfig UE variable

-- ASN1START

-- TAG-VAR-MEAS-CONFIG-START

VarMeasConfig ::=					SEQUENCE {

-- Measurement identities

measIdList							MeasIdToAddModList					OPTIONAL,

-- Measurement objects

measObjectList						MeasObjectToAddModList				OPTIONAL,

-- Reporting configurations

reportConfigList					ReportConfigToAddModList			OPTIONAL,

-- Other parameters

quantityConfig						QuantityConfig						OPTIONAL,

s-MeasureConfig							CHOICE {

ssb-rsrp								RSRP-Range,

csi-rsrp								RSRP-Range

} 																		OPTIONAL

speedStatePars						CHOICE {

release								NULL,

setup								SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

timeToTrigger-SF					SpeedStateScaleFactors

}

}																		OPTIONAL,

}

-- TAG-VAR-MEAS-CONFIG-STOP

-- ASN1STOP

CHANGE END