3GPP TSG-RAN WG2 Meeting #101	R2-1803733

Athens, Greece, 26th February - 2nd March 2018

Source: 	CMCC

Title:	Report from Break-Out Session

## 9.5	Further video enhancements for LTE

(LTE_ViLTE_enh2-Core; leading WG: RAN2; REL-15; started: Mar. 17; target: Jun. 18: WID: RP-172726)

Time budget: 0 TU

Documents in this agenda item will be handled in a break out session

This WI is complete from RAN2 point of view but RAN2 CRs have not been implemented to the specification as described in RP-172755. The CRs will be maintained as running CRs and then agreed again in RAN2#102. This AI is for corrections to the running CRs.

R2-1802876	Discussion on corrections to eVideo	Huawei, HiSilicon	discussion	Rel-15	36.300	LTE_ViLTE_enh2-Core

=>	Change to “UE can report to the network its capability of supporting assistance information bit for local cache. If supported, the UE assisted local cache function can be activated by the eNB. After that, the UE may indicate the assistance information bit in the uplink PDCP PDU. Whether the UE indicates this assistance information bit is based on for instance the service from the application layer the UE requests that support local cache handling.”

=>	With this change, the change in annex is agreed.

R2-1803699	Assistance information for local cache	Ericsson	discussion	Rel-15	LTE_ViLTE_enh2-Core

=>	Capture the change with separate figures for uplink and downlink in this paper.

R2-1802990	Discussion on different handling for critical data	LG Electronics France	discussion	Rel-15	LTE_ViLTE_enh2-Core	R2-1713685

R2-1802991	Differant handling for critical data	LG Electronics France	CR	Rel-15	36.323	14.5.0	0214	1	B	LTE_ViLTE_enh2-Core	R2-1713686

R2-1802999	Differant handling for critical data	LG Electronics France	CR	Rel-15	36.331	15.0.1	3196	1	B	LTE_ViLTE_enh2-Core	R2-1713689

## 9.6	QoE Measurement Collection for streaming services in E-UTRAN

(LTE_QMC_Streaming; leading WG: RAN2; REL-15; started: Mar. 17; target: Jun. 18: WID: RP-170956)

Time budget: 0 TU

Documents in this agenda item will be handled in a break out session

This WI is complete from RAN2 point of view but RAN2 CRs have not been implemented to the specification as described in RP-172755. The CRs will be maintained as running CRs and then agreed again in RAN2#102. This AI is for corrections to the running CRs.

R2-1801708	Service types for QMC reporting (C1-180636; contact: Ericsson)	CT1	LS in	Rel-15	EQoE_MTSI, UMTS_QMC_Streaming, LTE_QMC_Streaming, QOED	To:SA4, RAN2, RAN6	Cc:SA5

=>	Noted

=>	Corresponding discussion will be taken in 9.20.

R2-1801754	LS on Attributes for QoE measurement collection (S5-176409; contact: Ericsson)	SA5	LS in	Rel-15	LTE_QMC_Streaming	To:SA4, RAN2, RAN3

=>	Noted

R2-1801769	Reply LS on Attributes for QoE measurement collection (S4-180240; contact: Ericsson)	SA4	LS in	Rel-15	UMTS_QMC_Streaming, LTE_QMC_Streaming, EQoE_MTSI, QOED	To:SA5	Cc:RAN2, RAN3

=>	Noted

R2-1801768	Reply LS on adding new service type in QMC reporting (S4-180229; contact: Ericsson)	SA4	LS in	Rel-15	UMTS_QMC_Streaming, LTE_QMC_Streaming, EQoE_MTSI, QOED	To:RAN6	Cc:CT1, RAN2

=>	Noted

R2-1802888	Running CR on Introduction of QoE Measurement Collection for LTE	Huawei, HiSilicon	CR	Rel-15	36.300	15.0.0	1098	-	B	LTE_QMC_Streaming-Core

R2-1802889	Running CR on Introduction of QoE Measurement Collection for LTE	Huawei, HiSilicon	CR	Rel-15	36.306	14.5.0	1558	-	B	LTE_QMC_Streaming-Core

R2-1802890	Running CR on Introduction of QoE Measurement Collection for LTE	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3249	-	B	LTE_QMC_Streaming-Core

R2-1802891	Discussion on QoE configuration id	Huawei, HiSilicon	discussion	Rel-15	LTE_QMC_Streaming-Core

## 9.19	Bluetooth/WLAN measurement collection in MDT

(LTE_MDT_BT_WLAN-Core; leading WG: RAN2; REL-15; started: Dec. 17; target: June. 18: WID: RP-172820)

Time budget: 0.5 TU

Documents in this agenda item will be handled in a break out session

R2-1803223	Work plan for New WID on Bluetooth/WLAN measurement collection in MDT	CMCC	discussion	Rel-15	LTE_MDT_BT_WLAN-Core

=>	Noted

R2-1802539	MDT exception for reducing power consumption	LG Electronics Mobile Research	discussion	LTE_MDT_BT_WLAN-Core

=>	Noted

R2-1802877	Use cases for WLAN and Bluetooth measurement collection in MDT	Huawei, HiSilicon	discussion	Rel-15	LTE_MDT_BT_WLAN-Core

Agreement:

Immediate MDT, Logged MDT and RLF report are supported for WLAN/BT measurement collection if the measurements available.

R2-1803224	Consideration on Bluetooth/WLAN measurement collection in MDT	CMCC	discussion	Rel-15	LTE_MDT_BT_WLAN-Core

=> Revised in R2-1803770

R2-1803770	Consideration on Bluetooth/WLAN measurement collection in MDT	CMCC	discussion	Rel-15	LTE_MDT_BT_WLAN-Core

Agreements:

1	UE reports WLAN/BT measurements to eNB.

2	For BT measurements, reuse the measurement information (i.e., btAddr, rssi) already defined in LPP specification 36.355 as much as possible.

For WLAN measurements, reuse the measurements in specification 36.331 as much as possible.

3	For Immediate MDT, WLAN and BT measurements is carried as measurement results.

4	The reporting form of BT/WLAN measurements in logged MDT is carried as measurement results.

5	At least the MAC addresses of Bluetooth beacons and BSSID/SSID/HESSID of WLAN APs should be reported if UE reports Bluetooth/WLAN measurements.

6	If requested by the network, the report can further include RSSI for Bluetooth beacons and both RSSI and RTT for WLAN Aps if these measurement results available (the availability has the same meaning as available GNSS information in Rel-10 MDT) in UE.

6.1	Enforce UE to turn on WLAN/BT is not supported.

7	2 bits should be defined separately in UE capability to indicate whether the UE support Bluetooth and WLAN measurements or not.

8    The existing MDT measurement configuration procedures can be used.

R2-1802879	Analysis on measurement procedures for WLAN and Bluetooth measurement collection in MDT	Huawei, HiSilicon	discussion	Rel-15	LTE_MDT_BT_WLAN-Core

=>	To minimize the impacts due to large WLAN and Bluetooth reports:

-	Decrease maximum number of WLAN APs and Bluetooth beacons measured.

-	Restrict the configuration parameters for WLAN measurement.

-	Measurement results are carried by SRB 2.

R2-1803426	Considerations for MDT with WLAN/BT measurements 	Kyocera	discussion

Agreement:

1	The UE includes in the availability indicator an indication of whether any WLAN measurement is logged.

R2-1802878	Overview of WLAN and Bluetooth measurement collection in MDT	Huawei, HiSilicon	discussion	Rel-15	LTE_MDT_BT_WLAN-Core

R2-1803237	Introduction of Bluetooth/ WLAN measurement collection in MDT	CMCC	CR	Rel-15	36.306	14.5.0	1560	-	B	LTE_MDT_BT_WLAN-Core

R2-1803238	Introduction of new measurement collection in MDT-location information	CMCC	CR	Rel-15	36.331	15.0.1	3269	-	B	LTE_MDT_BT_WLAN-Core

R2-1803239	Introduction of new measurement collection in MDT-measurement results	CMCC	CR	Rel-15	36.331	15.0.1	3270	-	B	LTE_MDT_BT_WLAN-Core

R2-1803240	Introduction of new measurement collection in MDT	CMCC	CR	Rel-15	37.320	14.0.0	0070	-	B	LTE_MDT_BT_WLAN-Core

Email discussion on Introduction of Bluetooth/ WLAN measurement collection in MDT in 36.306 (CMCC)

Deadline:

Intended outcome: endorsed running CR

Email discussion on Introduction of Bluetooth/ WLAN measurement collection in MDT in 36.331 (Huawei)

Deadline:

Intended outcome: endorsed running CR

Email discussion on Introduction of Bluetooth/ WLAN measurement collection in MDT in 37.320 (CMCC)

Deadline:

Intended outcome: endorsed running CR

## 9.9	Enhancing CA Utilization

(LTE_euCA-Core; leading WG: RAN2; REL-15; started: Mar. 17; target: Jun. 18: WID: RP-170805)

Time budget: 0.5 TU

Documents in this agenda item will be handled in a break out session

### 9.9.1 General

Including incoming LSs, work plan, rapporteur inputs, running CRs

R2-1802750	Updated work plan for the euCA WID	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_euCA-Core

-	Intel think there will be small work in RAN1. Nokia think it can be addressed by LS.

=>	No RAN1 TUs are foreseen to be needed for the euCA WID.

=>	No TUs are foreseen to be needed for remaining RAN3 work.

### 9.9.3 Signalling overhead reduction for configuration activation

Including output of email discussion [100#36][LTE/euCA] Solutions on signalling overhead reduction (Nokia)

R2-1802758	Report of email discussion 100#36 (common configuration)	Nokia (rapporteur)	discussion	Rel-15	LTE_euCA-Core

-	Huawei think common parameters bring more complexity.

-	Qualcomm and Ericsson think we should follow the email discussion results from majority.

Agreements:

1	Adopt the new common parameters structure as proposed in R2-1713491 / R2-1712257.

2	Dedicated Scell configuration can be used to override common parameter configuration.

3	Four common SCell parameter groups are allowed by the RRC signalling.

4	Do not define a new group-based MAC CE.

R2-1802658	Discussion on the details of idle mode measurements	Huawei, HiSilicon	discussion	LTE_euCA-Core

R2-1802668	Signalling overhead reduction for SCell Configuration	Huawei, HiSilicon	discussion	Rel-15	LTE_euCA-Core

R2-1802670	Signalling overhead reduction for SCell (de)Activation	Huawei, HiSilicon	discussion	Rel-15	LTE_euCA-Core	R2-1713335

Late

R2-1802757	Remaining work on common SCell configuration	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_euCA-Core	R2-1713491	Late

R2-1802759	RRC CR based on the outcome of email discusion [100#36]	Nokia (rapporteur)	discussion	Rel-15	LTE_euCA-Core	Late

### 9.9.2 Delay reduction for SCell set-up

Idle measurement:

R2-1802751	Remaining Stage-2 work on IDLE mode measurements	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_euCA-Core	R2-1713161

-	vivo support proposal 1 and think not all UEs should do this measurement.

-	Qualcomm think we need wait for RAN4 progress. Nokia think we can do the procedure issue.

Agreements:

1	Do not define a validity timer for IDLE measurement configuration in SIB5. It doesn’t exclude UE performs aperiodic measurement.

2	The information that can be broadcast in SIB5 should be limited to at most 3 inter-frequencies and at most N cells per carrier frequency. Value of N is FFS.

3	UE can be provided with a “CA measurement configuration” within RRCConnectionRelease that is different from SIB5 configuration, and which overrides any SIB5 configurations. FFS the dedicated configuration is still valid after cell reselection.

4	Validity timer is only defined for configuration given in RRCConnectionRelease.

5	Only CRS-based measurements (RSRP/RSRQ/RS-SINR) are supported for IDLE mode measurements.

6	Network can indicate which measurement quantities out of RSRP, RSRQ and RS-SINR UE should report for IDLE mode measurements.

R2-1802007	Consideration on IDLE measurement across cells	vivo	discussion

P4-6

-	Ericsson and Intel think timer is enough and don’t need to introduce valid area.

-	OPPO wonder how to configure the area.

Agreements:

1	UE may continue performing IDLE state euCA measurement after cell reselection.

If the ongoing IDLE state measurement is configured only via SIB5 in source cell:

2	After cell reselection, if the SIB5 of the target cell includes the IDLE state measurement configuration, UE continues performing IDLE state measurement and updated the configuration according to the SIB5 of the target cell.

3	After cell reselection, if the SIB5 of the target cell includes no configuration for the IDLE state measurement, UE stops performing IDLE state measurement.

[101#xx][LTE/euCA] Introducing valid area mechanism (vivo)

-	Identify the scenarios of valid area

-	discuss whether or not to introduce valid area mechanism

Intended outcome: Report from email discussion

Deadline:

R2-1802752	Remaining Stage-3 work on IDLE mode measurements	Nokia, Nokia Shanghai Bell

=>	Noted

R2-1803660	Reporting condition of IDLE measurements	LG electronics inc	discussion

discussion	Rel-15	LTE_euCA-Core	R2-1713161

R2-1801794	Discussion on Measurement Configuration for Fast SCell Configuration	OPPO	discussion	LTE_euCA

R2-1803607	CA measurements in Idle	Ericsson	discussion	Rel-15	LTE_euCA-Core

Agreement:

The following behaviour is to adopt for reporting of the IDLE mode measurements:

The UE waits for an explicit eNB-request for the IDLE mode measurements. The measurements should be reported after security establishment.

Idle measurement reporting:

R2-1802073	Fast SCell Configuration through Quick SCell Measurement Reporting 	Qualcomm 	discussion	Rel-15	LTE_euCA-Core

Agreements:

1	Timer T331 starts upon RRC Connection release message received by UE.

2	Minimum SCell Signal Quality threshold for reporting may be configured by eNB.

R2-1802753	Security aspects of IDLE mode measurements	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_euCA-Core	R2-1713162

R2-1802003	Remaining Issues of IDLE State Measurement	vivo	discussion

R2-1802669	IDLE Mode Measurement Reporting message	Huawei, HiSilicon	discussion	Rel-15	LTE_euCA-Core

Temporary CQI reporting:

R2-1802657	Remaining issues of temporary CQI reporting	Huawei, HiSilicon	discussion	LTE_euCA-Core

R2-1802756	Remaining work on the temporary CQI reporting during Scell activation	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_euCA-Core

R2-1803609	Fallback to long CQI reporting period	Ericsson	discussion	Rel-15	LTE_euCA-Core

R2-1802004	Fallback to long CQI report	vivo	discussion

R2-1802911	Handling of CSI for SCell activation in euCA	Intel Corporation	discussion	Rel-15	LTE_euCA-Core

Timing of direct activation:

R2-1802754	Remaining Stage-3 work on direct activation of SCells	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_euCA-Core	R2-1713164

Agreements

1	When SCell is configured as activated, the PUCCH format changes to take this into account at n+20 (i.e. after the RRC processing delay). This should be confirmed from RAN4.

CBF:=>	Draft LS in R2-xxxxx to RAN1/RAN4 to inform the agreements. Ask RAN4 if CSI reporting delay requirements (today 24 (34) ms after activation) can be reduced if UE has been monitoring the Scell carrier. (Nokia, offline#222)

R2-1803608	Direct activation at configuration	Ericsson	discussion	Rel-15	LTE_euCA-Core

=>	Noted

R2-1802301	Timing issue in configuring SCell in activated state	LG Electronics Mobile Research	discussion	LTE_euCA-Core

R2-1802072	Draft LS about direct SCell activation delay during HO and Non-HO cases   	Qualcomm 	LS out	Rel-15	LTE_euCA-Core	To:RAN 4

R2-1802656	Discussion on timing issue of direct activation	Huawei, HiSilicon	discussion	LTE_euCA-Core

New State:

R2-1802755	Remaining work on the New Scell State	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_euCA-Core

Agreements:

1	Use “Dormant SCell state” as the name of the new Scell state and use ”Hibernate” as the action for moving from Activated SCell state to Dormant Scell state if necessary.

2	Dormant Scell follows PCell DRX for CQI/RRM measurement report triggering.

3	Additional new MAC CE is needed for state transition in/out of the dormant state. Legacy MAC CE is still used as in Rel-10 CA.

4	Define a MAC CE for state transition from activated SCell state to the dormant SCell state.

5    It is supported that UE transits from dormant state to deactivate state.

R2-1802074	SCell New State Transitions and Signalling Procedures	Qualcomm 	discussion	Rel-15	LTE_euCA-Core

R2-1802655	Discussion on new SCell state	Huawei, HiSilicon	discussion	Rel-15	LTE_euCA-Core

R2-1802002	MAC CE for the new SCell state	vivo	discussion

R2-1802667	Clarification on the MAC CE for new SCell state transition	Huawei, HiSilicon	discussion	Rel-15	LTE_euCA-Core

Others:

R2-1803605	Measurement improvements for euCA	Ericsson	discussion	Rel-15	LTE_euCA-Core

R2-1803604	[DRAFT] LS to RAN4 on euCA measurement improvements	Ericsson	LS out	Rel-15	LTE_euCA-Core	To:RAN4

R2-1803606	RRC Suspended and CA establishment	Ericsson	discussion	Rel-15	LTE_euCA-Core

### 9.9.4 Others

R2-1802760	Applying IDLE mode measurements for EN-DC	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_euCA-Core	R2-1711004

## 9.17	Further enhancements to CoMP for LTE

(feCOMP_LTE-Core; leading WG: RAN1; REL-15; started: Mar. 17; target: Mar. 18: WID: RP-171031)

Time budget: 0.5 TU

Documents in this agenda item will be handled in a break out session

R2-1802898	Introduction of further enhancements to CoMP	Intel Corporation	CR	Rel-15	36.331	15.0.1	3251	-	B	feCOMP_LTE-Core

=>	Revised in R2-xxxxx.

CBF offline#255 (Intel)

## 9.11	High capacity stationary wireless and 1024 QAM

(LTE_1024QAM_DL-Core; leading WG: RAN1; REL-15; started: Mar. 17; target: Mar. 18: WID: RP-171738)

Time budget: 0.5 TU

Documents in this agenda item will be handled in a break out session

### 9.11.1 General

Including incoming LSs, work plan, rapporteur inputs, running CRs

R2-1801714	LS on RRC parameters for high capacity stationary wireless link and introduction of 1024 QAM for LTE (R1-1721731; contact: Huawei)	RAN1	LS in	Rel-15	LTE_1024QAM_DL	To:RAN2

-	Huawei clarifies this capability is per band and BC.

-	Qualcomm think it is possible when carrier aggregation UE can activate 1024 QAM only in one carrier.

=>	Noted

### 9.11.2 UE capability and potential new categories

R2-1802247	Support of 1024QAM in TS 36.306	Huawei, HiSilicon	CR	Rel-15	36.306	14.5.0	1547	-	B	LTE_1024QAM_DL-Core

=>	Changed to “This field defines whether the UE supports 1024QAM in DL on this band or on this band within the band combination as described in TS 36.331 [5].”

=>	Endorsed as the running CR in R2-1803994.

### 9.11.3 Corresponding higher-layer procedures and signalling

R2-1802246	Support of 1024QAM in TS 36.331	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3227	-	B	LTE_1024QAM_DL-Core

=>	Revised to R2-1803970

R2-1803970	Support of 1024QAM in TS 36.331	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3227	-	B	LTE_1024QAM_DL-Core

=>	Endorsed as the running CR.

## 9.16	UL data compression in LTE

(LTE_UDC-Core; leading WG: RAN2; Rel-15; started Sep 17; target: Mar 18; WID RP-172365)

Time budget: 1.0 TU

Documents in this agenda item will be handled in a break out session

Running CR:

R2-1801913	Introduction of DEFLATE based UDC Solution	CATT	CR	Rel-15	36.323	14.5.0	0217	-	B	LTE_UDC-Core

=>	Endorsed as running CR

R2-1801914	Introduction of DEFLATE based UDC Solution	CATT	CR	Rel-15	36.331	15.0.1	3211	-	B	LTE_UDC-Core

=>	Endorsed as running CR

R2-1801915	Introduction of DEFLATE based UDC Solution	CATT	CR	Rel-15	36.300	15.0.0	1090	-	B	LTE_UDC-Core

=>	Endorsed as running CR

Operator defined dictionary:

R2-1801917	Further Discussion on Operator Defined Dictionary	CATT, MediaTek Inc.	discussion	Rel-15	LTE_UDC-Core

-	Ericsson wonder which PLMN ID should be reported. CATT think it should left UE implementation.

P2:

-	Huawei think there is no RAN2 impact

Agreements:

1	Define an associated PLMN ID in UDC related UE capability reporting if it reports supporting operator defined dictionary.

2	From RAN2 perspective, UE is not required to report supporting operator defined dictionary when it enters a roaming network.

3	4 bits version ID for operator defined dictionary can be defined in UDC related UE capability reporting.

R2-1803469	Pre-Defined Dictionary for UDC	Ericsson	discussion	Rel-15

P3:

-	LG and CATT think it is not needed.

=>	Noted

R2-1802854	Discussion on operator defined dictionary	Huawei, HiSilicon	discussion	Rel-15	LTE_UDC-Core

=>	RAN confirms both eNB and UE should have the same configuration for ODD.

R2-1801918	Introduction of Operator Defined Dictionary Capability	CATT, MediaTek Inc.	draftCR	Rel-15	36.331	15.0.1	B	LTE_UDC-Core

R2-1801919	Introduction of Operator Defined Dictionary Capability	CATT, MediaTek Inc.	draftCR	Rel-15	36.306	14.5.0	B	LTE_UDC-Core

R2-1803467	Pre-Defined Dictionary Configuration for UDC	Ericsson	CR	Rel-15	36.331	15.0.1	3280	-	B	LTE_UDC-Core

R2-1803468	Pre-defined Dictionary for UDC	Ericsson	CR	Rel-15	36.323	14.5.0	0226	-	B	LTE_UDC-Core

R2-1803471	Standard and Operator defined dictionary capability support for UDC	Ericsson	CR	Rel-15	36.306	14.5.0	1562	-	B	LTE_UDC-Core

UDC procedure:

R2-1802855	Discussion on setup and release of UDC	Huawei, HiSilicon	discussion	Rel-15	LTE_UDC-Core	R2-1712721

-	LG think case 2 and 4 don’t exist.

-	LG, CATT and MTK think handover procedure can be used for these cases. CMCC slightly prefer to use handover procedure.

=>	RAN2 confirm that handover procedure can be used for case 1-4 in R2-1802855.

R2-1802130	Discussion on UDC Reconfiguration	MediaTek Inc.	discussion	LTE_UDC-Core

Agreement:

1	Specify in RAN2 spec that UDC configuration is not used for split and LWA DRBs.

R2-1803297	Handling of UDC configuration for resume procedure	Intel Corporation	discussion	Rel-15	LTE_UDC-Core

Agreement:

1	Upon initiation of RRC connection resume, the UE shall release the UDC configuration if configured.

R2-1803470	Setup and Release Behaviour for UDC during setup/release	Ericsson	discussion	Rel-15

R2-1802856	CR on UDC configuration	Huawei, HiSilicon	CR	Rel-15	36.323	14.5.0	0220	-	B	LTE_UDC-Core

R2-1802857	CR on UDC configuration	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3246	-	B	LTE_UDC-Core

FU bit clarification:

R2-1802128	Discussion on UDC FU Bit	MediaTek Inc., Ericsson	discussion	LTE_UDC-Core

Agreement:

Replace “processed by UDC” by “compressed by UDC” in RAN2 spec to clarify UDC function behavior.

R2-1801920	Clarification on FU bit	CATT, Ericsson	draftCR	Rel-15	36.323	14.5.0	F	LTE_UDC-Core

R2-1802129	Clarification of UDC Behavior on FU bit	MediaTek Inc.	draftCR	Rel-15	36.323	14.5.0	F	LTE_UDC-Core

Capability:

R2-1802858	Discussion on UE capabilities of UDC	Huawei, HiSilicon	discussion	Rel-15	LTE_UDC-Core

Agreements:

There are three types of UE capability regarding UDC:

-	(1)	basic UDC functionality

-	(2)	standard dictionary

-	(3)	operator defined dictionary

Dictionary related capabilities (2) and (3) are dependent on the basic UDC functionality (1). In addition, A UE that supports (1) shall support 8192 bytes for compression buffer per DRB.

R2-1802859	CR on UE capabilities of UDC	Huawei, HiSilicon	CR	Rel-15	36.306	14.5.0	1556	-	B	LTE_UDC-Core

R2-1801916	Introduction of DEFLATE based UDC Solution	CATT	CR	Rel-15	36.306	14.5.0	1543	-	B	LTE_UDC-Core

Others:

R2-1803296	UE initiated history buffer reset in UDC	Intel Corporation	discussion	Rel-15	LTE_UDC-Core

Agreement:

The UE can set FR bit to trigger a history buffer reset without getting a request from eNB before.

R2-1802761	UDC processing requirement	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_UDC-Core

-	CATT think it is not necessary to assess processing time as we didn’t do for RoHC.

=>	Noted

R2-1802860	CR on UDC header	Huawei, HiSilicon	CR	Rel-15	36.323	14.5.0	0221	-	B	LTE_UDC-Core

R2-1802861	CR on UDC buffer if dictionary is not configured	Huawei, HiSilicon	CR	Rel-15	36.323	14.5.0	0222	-	B	LTE_UDC-Core

R2-1802862	CR on standard dictionary	Huawei, HiSilicon	CR	Rel-15	36.323	14.5.0	0223	-	B	LTE_UDC-Core

R2-1802863	CR on standard dictionary	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3247	-	B	LTE_UDC-Core

R2-1803729	Possible enhancement for reset feedback in UDC	Ericsson	discussion

=> Withdrawn

[101#xx][LTE/UDC] Introduction of DEFLATE based UDC Solution in TS36.323 (CATT)

Intended outcome: endorsed CR

Deadline: One week

[101#xx][LTE/UDC] Introduction of DEFLATE based UDC Solution in TS36.331 (CATT)

Intended outcome: endorsed CR

Deadline: One week

[101#xx][LTE/UDC] Introduction of DEFLATE based UDC Solution in TS36.306 (CATT)

Intended outcome: endorsed CR

Deadline: One week

[101#xx][LTE/UDC] Introduction of DEFLATE based UDC Solution in TS36.300 (CATT)

Intended outcome: endorsed CR

Deadline: One week

=>	From RAN perspective the WI is completed.

## 9.20	Other LTE Rel-15 WIs

This agenda item may be used for documents relating to Rel-15 WIs with no allocated RAN2 time but which might have minor RAN2 impact (e.g. CT/SA WIs for which we have received an LS requesting RAN2 action)

R2-1802728	Handling of incoming LS on new service type in QMC reporting	Ericsson	discussion	Rel-15	TEI15

=>	Noted

R2-1802875	Discussion on QoE requirements from SA4	Huawei, HiSilicon	discussion	Rel-15	TEI15

=>	Noted

CBF:=>	Draft LS in R2-xxxxxx to SA4, SA5  cc RAN3 to ask the meaning of new service and ask the possibility to address this in container. Add description that this cannot be handled in LTE QMC (Ericsson ,offline#234)

R2-1803183	Handling of incoming LS on Attributes for QoE measurement collection	Ericsson	discussion	Rel-15	Revised

=>	Revised to R2-1803727

R2-1803727	Handling of incoming LS on Attributes for QoE measurement collection	Ericsson	discussion	Rel-15	Revised

CBF:=>	Draft a reply LS in R2-xxxx to SA5 cc SA4, RAN3 saying that RAN2 has analysed the proposed new attributes and concluded that if the areaScope is not sent to the UE the size of the RRC message is decreased.  Ask the possibility of adding new service type in container (Ericsson offline#234).

R2-1803184	[DRAFT] Reply LS on Attributes for QoE measurement collection	Ericsson	LS out	Rel-15	QOED	To:SA5	Cc:RAN3, SA4	Revised

R2-1803185	[DRAFT] Reply LS on adding new service type in QMC reporting	Ericsson	LS out	Rel-15	QOED	To:SA4	Cc:RAN3, SA5

R2-1802729	Introduction of QoE Measurement Collection for MTSI services	Ericsson	CR	Rel-15	36.300	15.0.0	1096	-	B	TEI15

R2-1802730	Introduction of QoE Measurement Collection for MTSI services	Ericsson	CR	Rel-15	36.306	14.5.0	1554	-	B	TEI15

R2-1802731	Introduction of QoE Measurement Collection for MTSI services	Ericsson	CR	Rel-15	36.331	15.0.1	3243	-	B	TEI15

Late

R2-1803727	Handling of incoming LS on Attributes for QoE measurement collection	Ericsson	discussion	Rel-15	R2-1803183	Late

R2-1803728	[DRAFT] Reply LS on Attributes for QoE measurement collection	Ericsson	LS out	Rel-15	QOED	R2-1803184	To:SA5	Cc:RAN3, SA4	Late

## 9.21	LTE TEI15 enhancements

Small Technical Enhancements affecting LTE Rel-15 that do not belong to any Rel-15 WI.

Note: A TEI enhancement proposal should be treated for only one meeting cycle and involve only one WG. Otherwise, a WI should be proposed at RAN plenary!

Time budget: 1 TU

### 9.21.1	CP latency for LTE

Contributions related to the task given to RAN2 from RAN#78 as described in LS RP-172840.

R2-1802271	Discussion on CP Latency for IMT-2020 submission	OPPO	discussion

=>	Not treated

R2-1802413	Discussion on CP Latency Reduction Options	MediaTek Inc.	discussion	Rel-15	TEI15

=>	Noted

R2-1803515	Control Plane Latency Reduction	Ericsson	discussion	Rel-15	TEI15

=>	Noted

-	CATT think for TDD case component 5 must be reduced.

=>	RAN2 will focus on component 5, 7, 9 and 10 to reduce the CP latency.

=>	The following table is used as baseline.

| Component | Description | Op1:Latency [ms] | Op2:Latency [ms] | Op3:Latency [ms] | Op4:Latency [ms] |
| --- | --- | --- | --- | --- | --- |
| 1 | Average delay due to RACH scheduling period (1TTI) | 0.5 | 0.5 | 0.5 | 0.5 |
| 2 | Transmission of RACH Preamble | 1 | 1 | 1 | 1 |
| 3 | Preamble detection and processing in eNB | 2 | 2 | 2 | 2 |
| 4 | Transmission of RA response | 1 | 1 | 1 | 1 |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Resume Request) | 3 | 4 | 4[5] | 5 |
| 6 | Transmission of RRC Connection Resume Request | 1 | 1 | 1 | 1 |
| 7 | Processing delay in eNB (L2 and RRC) | 3 | 2 | 4[3] | 2 |
| 8 | Transmission of RRC Connection Resume | 1 | 1 | 1 | 1 |
| 9 | Processing delay in the UE (L2 and RRC; including UL grant reception) | 5 | 7 | 4 | 5 |
| 10 | Transmission of RRC Connection Resume Complete and UP data | [0] | [0] | [0] | [0] |
|  | Total delay [ms] | 17.5 | 19.5 | 18.5 | 18.5 |

CB tomorrow morning :=>	Draft LS to RAN1 inform our situation. Ask what RAN1 can do for component 5.  (Ericsson offline#288)

R2-1803979	LS	Ericsson	discussion	Rel-15	TEI15

=>	With the change “how much specification effort is required and how much gain could be obtained.”

=>	LS is approved in R2-180xxxx.

R2-1801923	Control Plane Latency Reduction in LTE for TDD	CATT	discussion	Rel-15	TEI15

R2-1802626	CP Latency Reduction using Early Data Transmission	MediaTek Beijing Inc.	discussion

R2-1802653	Consideration on LTE Control Plane Latency Reduction	Huawei, HiSilicon	discussion	Rel-15	TEI15

R2-1802894	CP latency reduction for LTE	Intel Corporation	discussion	Rel-15	TEI15

R2-1803513	Control Plane latency reduction	Ericsson	CR	Rel-15	36.306	14.5.0	1563	-	B	TEI15

R2-1803514	Control Plane latency reduction	Ericsson	CR	Rel-15	36.331	15.0.1	3283	-	B	TEI15

### 9.21.2	Other

Including output of email discussion [100#35][LTE/TEI15] New L2 measurements (Huawei)

HSDN:

R2-1802897	Cell reselection for the UE on high-speed-dedicated network	Intel Corporation	discussion	Rel-15	TEI15

Agreement:

1	Introduce 1 new bit indication in SIB1 to indicate the high speed dedicated network cell.

2	Maximum value for the weight for high speed dedicated network cell is 16. New value of MSE cell count for HSDN case can be further considered.

3	When the UE is in high mobility state, the UE prioritize to re-select the high speed dedicated network cell regardless of frequency priority.

4	When the UE supporting the HSDN feature is not in high mobility state, the UE deprioritize (i.e., lowest priority) to re-select the high speed dedicated network cell.

R2-1803213	Support of mobility-state-based cell reselection in the high-speed-railway dedicated LTE network (HSDN)	CMCC	discussion	Rel-15	TEI15

R2-1801924	Open Issues of Cell Reselection in HSDN	CATT	discussion	Rel-15	TEI15

-	Ericsson think P3 is kind of optimisation

Agreements:

1	HSDN neighbouring cell list can be provided in SIB. FFS the content of the list.

[101#xx][LTE/TEI15] Support of mobility-state-based cell reselection in the high-speed-railway dedicated LTE network in 36.304 (OPPO)

Intended outcome: endorsed CR

Deadline: until next meeting

[101#xx][LTE/TEI15] Support of mobility-state-based cell reselection in the high-speed-railway dedicated LTE network in 36.331 (CMCC)

Intended outcome: endorsed CR

Deadline: until next meeting

R2-1801773	Discussion on System Information and Cell Reselection Issues for High Speed Dedicated Network	OPPO	discussion	Rel-15	TEI15

R2-1801774	CR to TS 36.331 on System Information Issues for High Speed Dedicated Network	OPPO	draftCR	Rel-15	36.331	15.0.1	TEI15

R2-1801775	CR to TS 36.304 on Cell Reselection Issues for High Speed Dedicated Network	OPPO	draftCR	Rel-14	36.304	14.5.0	TEI15

R2-1802257	Discussion on the issues for UE on high-speed dedicated network	Huawei, HiSilicon	discussion	Rel-15	TEI15

R2-1802896	CR for support of cell reselection for high speed dedicated network	Intel Corporation	CR	Rel-15	36.331	15.0.1	3250	-	B	TEI15

R2-1803235	Support of mobility-state-based cell reselection in the high-speed-railway dedicated LTE network (HSDN)	CMCC	CR	Rel-15	36.304	14.5.0	0406	-	F	TEI15

R2-1803236	Support of accurate UE mobility state estimation and mobility-state-based cell reselection for HSDN	CMCC	CR	Rel-15	36.331	15.0.1	3268	-	F	TEI15

R2-1803308	CR for support of cell reselection for high speed dedicated network	Intel Corporation	CR	Rel-15	36.304	14.5.0	0408	-	B	TEI15

L2 measurement:

R2-1802864	Email discussion summary on new L2 measurements	Huawei	discussion	Rel-15	TEI15

Agreement:

1	for use case 1, introduce the following two new measurements:

- Distribution of total PRB usage for DL

- Distribution of total PRB usage for UL

CBF: Check the following proposal can be agreed or not and try to agree CR of TS36.314 (Huawei, offline 298)

- Proposal: for use case 2, Introduce following two new measurements:

- Distribution of Scheduled IP Throughput in DL

- Distribution of Scheduled IP Throughput in UL

=>	For the new measurements, it is proposed to have new section(s) to describe them.

CBF:=>	(Huawei, offline 299) Draft LS to SA5:

- for use case 1, regarding the following two new measurements, consult SA5 on impacts due to lot of samples.

- Distribution of PRB usage per traffic class for DL

- Distribution of PRB usage per traffic class for UL

for use case 2, regarding the following two new measurements, consult SA5 on impacts due to lot of samples.

- Distribution of Scheduled IP Throughput in DL (per QCI)

- Distribution of Scheduled IP Throughput in UL (per QCI)

for use case 3, consult SA5 on the following two comments:

- whether the existing performance measurements and their combinations provide various means to reflect effectively RRC-connected UEs, e.g. RRC measurements defined in TS 32.425

- the intention, i.e. in case if UE has multiple E-RABs with different QCI, then the same UE could be counted twice.

R2-1802865	CR on distribution of total PRB usage	Huawei, HiSilicon	CR	Rel-15	36.314	14.0.0	0049	-	F	TEI15

R2-1802866	CR on distribution of scheduled IP throughput	Huawei, HiSilicon	CR	Rel-15	36.314	14.0.0	0050	-	F	TEI15

=>	Revised in R2-1803953

R2-1803953	CR on distribution of scheduled IP throughput	Huawei, HiSilicon	CR	Rel-15	36.314	14.0.0	0050	-	F	TEI15

R2-1802867	]DRAFT] Response LS on adding PRB usage distribution and IP throughput distribution measurements	Huawei	LS out	Rel-15	TEI15	To:SA5

R2-1802868	[DRAFT] Response LS on adding measurements on average number of total active UEs	Huawei	LS out	Rel-15	TEI15	To:SA5

Others:

R2-1803603	Addition of Missing Figures for R-PDCCH, EPDCCH and MPDCCH	Ericsson	CR	Rel-15	36.300	15.0.0	1111	-	F	TEI15

-	Qualcomm think it is not needed.

=>	Postponed

R2-1802870	Discussion on additional PDCP discard timer	Huawei, HiSilicon	discussion	Rel-15	TEI15	R2-1712725

R2-1802871	Introduction of additional PDCP discard timer	Huawei, HiSilicon	CR	Rel-15	36.300	15.0.0	1097	-	B	TEI15

R2-1802872	Introduction of additional PDCP discard timer	Huawei, HiSilicon	CR	Rel-15	36.306	14.5.0	1557	-	B	TEI15

R2-1802873	Introduction of additional PDCP discard timer	Huawei, HiSilicon	CR	Rel-15	36.323	14.5.0	0224	-	B	TEI15

R2-1802874	Introduction of additional PDCP discard timer	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3248	-	B	TEI15

R2-1802244	Enhancement of SRS antenna switching in TS 36.331	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3226	-	B	TEI15

R2-1802245	Enhancement of SRS antenna switching in TS 36.306	Huawei, HiSilicon	CR	Rel-15	36.306	14.5.0	1546	-	B	TEI15

R2-1802869	Discussion on dual registration and issues in LTE	Huawei, HiSilicon	discussion	Rel-15	TEI15

R2-1803439	Enabling MBMS Bearer Event Notification	Ericsson Limited	CR	Rel-15	36.300	15.0.0	1103	-	F	MBMS_Mcservices

R2-1802421	Clarification on RSTD Reporting for Assistance Data Reference Cell	Samsung Electronics France SA	discussion	Rel-15	TEI15

R2-1802423	CR on Solution 1 for RSTD Reporting for Assistance Data Reference Cell	Samsung Electronics France SA	CR	Rel-15	36.355	14.4.0	0190	-	C	TEI15

R2-1802426	CR on Solution 2 for RSTD Reporting for Assistance Data Reference Cell	Samsung Electronics France SA	CR	Rel-15	36.355	14.4.0	0191	-	C	TEI15

R2-1803028	Japanese regulation regarding HPUE UL CA in Band 41	KDDI Corporation	discussion	Rel-15

## 9.12	Enhancements to LTE operation in unlicensed spectrum

(LTE_unlic-Core; leading WG: RAN1; REL-15; started: Mar. 17; target: Jun. 18: WID: RP-172841)

Time budget: 1 TU

Documents in this agenda item will be handled in a break out session

### 9.12.1 General

Including incoming LSs, work plan, rapporteur inputs, running CRs

R2-1803172	Summary of RAN1 agreements and RAN2 implications	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_unlic-Core

### 9.12.2 Autonomous uplink access on Frame structure type 3

R2-1803972	proposal to progress feLAA	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_unlic-Core

-	Huawei think P1 and P2 are not needed. LG, Qualcomm and Nokia support P1.

-	Moto think it cannot work sufficiently with H1.

-	Moto think reusing SPS RNTI is very complicated and bringing much impact on RAN1 specs.

-	Intel and LG support introduce a new RNTI.

P1: Agree on multi-bit activation/deactivation command

P2: introduce new RNTI for activation/deactivation

H1: Reuse current zero-bit confirmation MAC CE for AUL activation/deactivation confirmation.

H2: Introduce a new RNTI for AUL activation/deactivation.

E1: Reuse SPS RNTI

Agreements:

1	Introduce multi-bit activation/deactivation MAC CE confirmation.

2	Introduce new RNTI for activation/deactivation.

3	For the AUL confirmation MAC CE, same triggering procedures specified for the Rel.14 SPS confirmation are adopted, i.e. AUL confirmation triggered at reception of PDCCH indicating AUL release/activation.

SR:

Agreements:

1	No enhancement on SR triggering will be introduced in FeLAA. No specification changes are needed.

2	No enhancement on SR cancellation will be introduced in FeLAA. No specification changes are needed.

R2-1802251	Issues on AUL activation and deactivation	Huawei, HiSilicon	discussion	Rel-15	LTE_unlic-Core

=>	Noted

R2-1803509	MAC Modelling for AUL	Ericsson	discussion	Rel-15	LTE_unlic-Core

Agreements

1	When AUL is configured, the UE is allowed to retransmit a MAC PDU in a later AUL subframe (e.g. in the next AUL subframe) if the associated LBT procedure failed, without the need to wait for a SUL grant, HARQ feedback or expiry of the retransmission timer X. This agreement can be consulted RAN1 for the feasibility.

2	RAN2 understanding is that “Physical layer informs the MAC layer on the outcome of the LBT procedure (e.g., by ACK/NACK), so that MAC can trigger a retransmission in a later subframe, without the need to wait for a SUL grant or the expiry of the retransmission timer X.” How to capture the modelling can be further studied.

R2-1802252	Issues on BS and PH calculation for AUL	Huawei, HiSilicon	discussion	Rel-15	LTE_unlic-Core

-	Moto think the current mechanism can work and don’t think P2 is necessary.

-	Huawei think the information of the time on BSR/PHR is important from eNB perspective.

-	Qualcomm think the problem already exist now.

-	Ericsson confirm the problem but not support the solution.

-	Qualcomm don’t see much benefit for P2.

Agreements:

1	In FeLAA, no enhancement on BSR/PHR trigger and cancellation is introduced.

2	It is up to UE implementation to solve issue of out of date BSR/PHR if necessary.

FFS the impact on spec.

R2-1802253	Handling on Timer X with AUL	Huawei, HiSilicon	discussion	Rel-15	LTE_unlic-Core

Agreements:

1	Timer X is maintained per HARQ process for AUL.

2	UE shall not use the AUL resource to perform both new transmission and retransmission for the same HARQ process before the Timer X stops/expires.

3	Start/restart the Timer X after the initial AUL transmission/AUL retransmission of a given HARQ process.

R2-1802254	Issues on coexistence between AUL and SUL	Huawei, HiSilicon	discussion	Rel-15	LTE_unlic-Core

R2-1802255	Disucssion on the priority class for AUL	Huawei, HiSilicon	discussion	Rel-15	LTE_unlic-Core

R2-1802256	Other MAC aspects of AUL	Huawei, HiSilicon	discussion	Rel-15	LTE_unlic-Core

R2-1802851	Necessity of multi bit confirmation MAC CE	LG Electronics Inc.	discussion	Rel-15	36.321	LTE_unlic-Core	R2-1713646

R2-1802892	HARQ aspects of AUL	Intel Corporation	discussion	Rel-15	LTE_unlic-Core

R2-1802893	Other aspects of AUL	Intel Corporation	discussion	Rel-15	LTE_unlic-Core

R2-1802919	HARQ process collision between AUL and SUL	LG Electronics Inc.	discussion	Rel-15	36.321	LTE_unlic-Core

R2-1803173	Open points of AUL	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_unlic-Core

R2-1803181	Remaining HARQ Aspects for Autonomous Uplink	Motorola Mobility Germany GmbH	discussion	Rel-15	LTE_unlic-Core	R2-1712979

-	Huawei and Qualcomm think it is up to UE implementation.

-	Ericsson observes the problem.

=>	Noted and the topic mentioned in proposal1 can be discussed in next meeting.

R2-1803187	Remaining MAC Aspects for Autonomous Uplink	Motorola Mobility Germany GmbH	discussion	Rel-15	LTE_unlic-Core

R2-1803502	AUL Configuration for feLAA	Ericsson	discussion	Rel-15	LTE_unlic-Core

R2-1803503	Channel Access Priority Classes for feLAA	Ericsson	discussion	Rel-15	LTE_unlic-Core

R2-1803504	Coexistence Between AUL and Dynamically Scheduled UL Grants	Ericsson	discussion	Rel-15	LTE_unlic-Core

R2-1803505	HARQ Retransmissions for Autonomous UL Access	Ericsson	discussion	Rel-15	LTE_unlic-Core

R2-1803506	Impact on BSR for feLAA	Ericsson	discussion	Rel-15	LTE_unlic-Core

R2-1803507	Impact on Uplink Timing Alignment for feLAA	Ericsson	discussion	Rel-15	LTE_unlic-Core

=>	Noted

Agreement:

Like for the pTAG, the expiry of an timeAlignmentTimer in an sTAG clears AUL resources for any associated SCell, hence AUL transmissions on those SCells are stopped

R2-1803511	On AUL Confirmation	Ericsson	discussion	Rel-15	LTE_unlic-Core

R2-1803559	Remaining Issues for AUL	Qualcomm Incorporated	discussion

### 9.12.3 Other operation on Frame structure type 3

R2-1803512	RAN2 Impact on Multiple Starting and Ending Positions in a Subframe	Ericsson	discussion	Rel-15	LTE_unlic-Core

### 9.12.4 Others

[101#xx][LTE/feLAA] Introduce feLLA in TS 36.331 (Nokia)

Capture the agreements so far from RAN1 and Ran2

Intended outcome: endorsed running CR

Deadline:

[101#xx][LTE/feLAA] Introduce feLLA in TS 36.321 (Ericsson)

Intended outcome: endorsed running CR

Deadline:

## 9.15	Highly Reliable Low Latency Communication for LTE

LTE_HRLLC-Core; leading WG: RAN1; REL-15; started: Mar. 17; target: Jun. 18: WID: RP-172845

Time budget: 1.0 TU

Documents in this agenda item will be handled in a break out session

Duplication:

R2-1802927	Issues on PDCP duplication	LG Electronics France	discussion	LTE_HRLLC-Core

=>	Noted

R2-1802714	PDCP duplication in LTE	Ericsson	discussion	Rel-15	LTE_HRLLC

=>	Noted

L1:

-	OPPO and Ericsson prefer current mechanism.

-	Huawei support P1 and P2. OPPO support P1 only for SRB case.

-	LG, Huawei and OPPO don’t agree proposal 6.

-	LG think 1 byte is enough for the MAC CE. Huawei share the same view.

L 1. The PDCP entity transmits duplicated PDCP PDU only when requested by lower layers, as in LTE.

L 2. CA and DC PDCP duplication for AM DRBs is not supported.

L 3. CA PDCP duplication for SRBs is not supported.

Proposal 1: Support RLC AM for packet duplication via DC.

Proposal 2: Support RLC AM for packet duplication via CA.

Proposal 3: The same PDCP mechanism for “Data volume calculation” can be used for both CA and DC duplication.

Proposal 4: When PDCP duplication is activated, all PDCP data is reported to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the second RLC entity.

Proposal 5: For CA case, When PDCP duplication is configured but not activated, all PDCP data is reported only to the MAC entity associated with the primary RLC entity.

Proposal 6: If configured, PDCP delivers data out-of-order.

Proposal 7: Introduce a three-octet bitmap for MAC CE activation/deactivation of PDCP duplication.

Proposal 8: Introduce a UE capability for packet duplication.

Agreements:

1	The PDCP entity transmits duplicated PDCP PDU only when requested by lower layers, as in LTE

2	Support RLC AM for packet duplication via DC for DRB.

3	Support RLC AM for packet duplication via CA for DRB.

4	The same PDCP mechanism for “Data volume calculation” can be used for both CA and DC duplication.

5	For DC case, when PDCP duplication is activated, all PDCP data is reported to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the second RLC entity.

6	For CA case, when PDCP duplication is activated, all PDCP data is reported to the MAC entity which is the same as agreed in NR session.

7	For CA case, when PDCP duplication is configured but not activated, all PDCP data is reported only to the MAC entity associated with the primary RLC entity.

8	Introduce one byte bitmap for MAC CE activation/deactivation of PDCP duplication at least for DRB.

9	Introduce UE capabilities for packet duplication.

R2-1801846	Discussion on LTE PDCP operation	OPPO	discussion	Rel-15	LTE_HRLLC-Core

Agreements:

1	The UE shall discard packets that have been acknowledged by RLC in the other RLC leg. PDCP should indicate to the other associated RLC entity to discard the corresponding PDCP PDU. RLC procedures and PDCP discard procedures are not impacted by this agreement.

2	For CA and DC upon deactivation of PDCP data duplication, the UE transmitting PDCP entity should indicate to lower layers to discard all PDCP PDUs provided for duplicate transmission to the secondary RLC entity.

R2-1802715	PDCP duplication discard and recovery	Ericsson	discussion	Rel-15	LTE_HRLLC

-	LG think reliability cannot be guaranteed for P3.

-	OPPO think it should be applied for UM mode and don’t support introducing a new timer.

Agreements:

1	 At PDCP re-establishment, when retransmitting PDCP SDUs for AM DRBs, duplicated PDUs are submitted to both associated RLC entities when PDCP duplication is activated.

2	 At PDCP data recovery, when retransmitting PDCP SDUs for AM DRBs, duplicated PDUs are submitted to both associated RLC entities when PDCP duplication is activated.

3	For packet duplication via CA, meeting the maximum number of RLC retransmissions for the secondary leg does not trigger RLF.

R2-1802675	Discussion on the MAC impact from packet duplication	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

-	LG agree with P1.

Agreements:

1	SRB duplication activation/deactivation via MAC CE is not supported in LTE. Once the duplication for SRB is configured, it is always activated.

2	RLC UM split bearer, i.e. two UM mode RLCs configured for one PDCP-config needs to be supported in LTE for the packet duplication.

3	For DC, when DRB duplication is deactivated via MAC CE, the UE falls back to the split bearer operation. For CA, when DRB duplication is deactivated via MAC CE, the UE falls back to the single RLC operation.

4	RRC can be used to de-configure the packet duplication directly.

R2-1802719	Prioritization between MAC CEs and High Reliability LCHs	Ericsson	discussion	Rel-15	LTE_HRLLC

Agreement:

1	Maintain the current LTE prioritization rules for handling MAC CE and LCHs configured with enhanced reliability requirements.

R2-1802677	Discussion on PDCP impact for PDCP duplication	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1802676	BSR procedure for data duplication	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1802671	Impact on CA Cell State from the PDCP duplication	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1801842	Discussion on UL duplication control	OPPO	discussion	Rel-15	LTE_HRLLC-Core

R2-1801843	Discussion on use case of UL duplication	OPPO	discussion	Rel-15	LTE_HRLLC-Core

R2-1801844	Discussion on RLF handling for CA duplication	OPPO	discussion	Rel-15	LTE_HRLLC-Core

R2-1801964	Discussion on PDCP packet duplication in LTE	ZTE	discussion	Rel-15

R2-1802672	Discussion on the RRC Impact from Packet Duplication	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1802674	MAC Structure and Operation for HRLLC	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1802882	Discussion on support RLC AM for DRB for PDCP duplication	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1802883	Discussion on the RLC impact from PDCP duplication	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1802926	PDCP configuration for SRB	LG Electronics France	discussion	Rel-15	LTE_HRLLC-Core

SPS:

R2-1802300	SPS for HRLLC	LG Electronics Mobile Research	discussion	LTE_HRLLC-Core

-	OPPO and Huawei prefer to use DCI as legacy behaviour.

=>	Noted

R2-1801845	Discussion on HARQ configuration	OPPO	discussion	Rel-15	LTE_HRLLC-Core

-	Ericsson think this timer is not needed in LTE. OPPO think the scenario is similar with NR.

-	LG support the proposal.

=>	Noted

R2-1802717	Reliability for PHICH-less UL SPS	Ericsson	discussion	Rel-15	LTE_HRLLC

R2-1803107	[DRAFT] LS on mutiple cell SPS (de)activation	OPPO	LS out	LTE_HRLLC-Core	To:RAN1

TB repetition:

R2-1802720	Transport block repetition for reliability	Ericsson	discussion	Rel-15	LTE_HRLLC

Agreements

1	A variable number of consecutive repetitions in time for SPS UL should be supported and is to be controlled by the eNB through RRC signalling.

R2-1802885	TB repetition for HRLLC	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1801965	Discussion on transport block repetition	ZTE	discussion	Rel-15

R2-1802718	Support periodic URLLC traffic with SPS	Ericsson	discussion	Rel-15	LTE_HRLLC

R2-1802299	Autonomous TTI bundling for HRLLC	LG Electronics Mobile Research	discussion	LTE_HRLLC-Core

Time Synchronization:

R2-1802880	Discussion on use cases and requirements on synchronization accuracy	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1802881	Discussion on solutions of providing time reference information	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

-	ZTE wonder it is for TDD or FDD. Huawei indicate it is for TDD.

-	LG think we should first check the feasibility in RAN4. Huawei think RAN4 is not involved in this WI for this topic.

[101#xx][LTE/URLLC] The granularity of time reference discussion (Huawei)

- - Discussion on use cases and requirements on synchronization accuracy and identify the gap between the requirement and the current synch accuracy

- Discussion on granularity of time

Intended outcome: Report of email discussion

Deadline:

R2-1802721	UE Time Synchronization	Ericsson	discussion	Rel-15	LTE_HRLLC

Others:

R2-1802884	Potential enhancements for HRLLC based on sTTI	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core

R2-1802722	sPUCCH Utilization Strategy	Ericsson	discussion	Rel-15	LTE_HRLLC

Running CRs:

R2-1802678	Configuration of packet duplication over CA and DC for TS 36.331	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3242	-	B	LTE_HRLLC-Core

R2-1802679	Introduction of HRLLC in LTE for TS 36.300	Huawei, HiSilicon	CR	Rel-15	36.300	15.0.0	1095	-	B	LTE_HRLLC-Core

R2-1802723	Running CR for introduction of Ultra Reliable Low Latency Communicaton for LTE	Ericsson	draftCR	Rel-15	36.300	15.0.0	B	LTE_HRLLC

R2-1802724	Running CR for introduction of Ultra Reliable Low Latency Communicaton for LTE	Ericsson	draftCR	Rel-15	36.321	15.0.0	B	LTE_HRLLC

R2-1802725	Running CR for introduction of Ultra Reliable Low Latency Communicaton for LTE	Ericsson	draftCR	Rel-15	36.322	15.0.0	B	LTE_HRLLC

R2-1802726	Running CR for introduction of Ultra Reliable Low Latency Communicaton for LTE	Ericsson	draftCR	Rel-15	36.323	14.5.0	B	LTE_HRLLC

R2-1802727	Running CR for introduction of Ultra Reliable Low Latency Communicaton for LTE	Ericsson	draftCR	Rel-15	36.331	15.0.1	B	LTE_HRLLC

R2-1802716	TP for PDCP duplication	Ericsson	discussion	Rel-15	LTE_HRLLC

R2-1802886	Introduction of HRLLC in LTE	Huawei, HiSilicon	CR	Rel-15	36.321	15.0.0	1214	-	B	LTE_HRLLC-Core

R2-1802887	Introduction of HRLLC in LTE	Huawei, HiSilicon	CR	Rel-15	36.323	14.5.0	0225	-	B	LTE_HRLLC-Core

Withdrawn

R2-1802673	Cell deactivation impact on PDCP dulication	Huawei, HiSilicon	discussion	Rel-15	LTE_HRLLC-Core	Withdrawn

CBF: Draft LS in R2-18xxxx to RAN1 to inform our progress of this meeting (Ericsson, offline# 201)

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.321 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.323 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.322 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.331 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.300 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

## 9.18	Enhanced LTE Support for Aerial Vehicles

(LTE_Aerial-Core;leading WG: RAN2; REL-15; started: Dec. 17; target: June. 18: WID: RP-172826)

Time budget: 1.0 TU

Documents in this agenda item will be handled in a break out session

Subscription based identification:

R2-1803715	Subscription based UAV Identification	NTT DOCOMO INC.	discussion

=> Revised in R2-1803744

R2-1803744	Subscription based UAV Identification	NTT DOCOMO INC.	discussion

-	Nokia think we don’t need to involve CT4 for this moment.

CBF:=>	Draft LS to SA2 with the contents of the work split (DoCoMo, offline#205)

Agreement:

1	RAN2 confirm the following work/responsibility split between WGs:

RAN2:

-	To clarify and confirm the usage of the subscription based identification and the overall stage2 behaviour of UAV identification signaling.

RAN3:

-	To define the signaling/IE for subscription based UAV identification.

SA2:

-	To define the subscription based UAV identification and the necessary signaling in interfaces between CN nodes (e.g., MME – HSS, etc.)

2	RAN2 to confirm that the subscription based information is signaled from the MME to the eNB

R2-1803437	[DRAFT] Reply LS on Certification/License and Identification of Aerial Vehicles	Ericsson	LS out	Rel-15	FS_LTE_Aerial	To:SA2, RAN3	Cc:SA1

R2-1802614	[DRAFT] LS on UAV UE identification	Nokia	LS out	Rel-15	LTE_Aerial-Core	To:RAN3, SA2	Cc:SA1

R2-1802662	Identification of Air-borne UE	Huawei, HiSilicon	discussion	Rel-15	LTE_Aerial-Core

Mobility enhancement:

Conditional handover

R2-1802709	General aspects of Conditional HO in Aerial	LG Electronics Inc.	discussion	Rel-15	LTE_27Aerial-Core

=>	Noted

R2-1802665	Discussion on Conditional HO	Huawei, HiSilicon	discussion	Rel-15	LTE_Aerial-Core

-	4 companies prefer to specify conditional handover for Aerial UE.

-	6 companies don’t support to specify conditional handover for Aerial UE.

=>	Noted

R2-1802304	Discussion on potential mobility enhancement for aerial UE	Lenovo, Motorola Mobility	discussion	Rel-15	LTE_Aerial-Core

R2-1802790	Other Mobility enhancements for aerial UEs	Ericsson	discussion	Rel-15	LTE_Aerial-Core

Measurement reporting

R2-1802306	Potential measurement enhancements for aerial UE	Lenovo, Motorola Mobility	discussion	Rel-15	LTE_Aerial-Core

=>	Introduce new measurement event/modify existing measurement events for interference detection

R2-1802895	New measurement event for interference detections from multiple cells	Intel Corporation	discussion	Rel-15	LTE_Aerial-Core

=>	Noted

R2-1802789	Measurement reporting enhancement	Ericsson	discussion	Rel-15	LTE_Aerial-Core

=>	Noted

R2-1802666	Discussion on enhancement of measurement reporting mechanism	Huawei, HiSilicon	discussion	Rel-15	LTE_Aerial-Core

R2-1802706	Mobility Enhancement using MR Triggering	LG Electronics Inc.	discussion	Rel-15	LTE_Aerial-Core

R2-1802806	Measurement reporting enhancements for Aerials	InterDigital	discussion	Rel-15	LTE_Aerial

Late

R2-1803713	Discussion on Enhancements to Measurement Report Mechanisms for UAV	NTT DOCOMO INC.	discussion	Late

=> Withdrawn

R2-1803743	Discussion on Enhancements to Measurement Report Mechanisms for UAV	NTT DOCOMO INC.	discussion	Late

Location and path information

R2-1802661	Introduction of flight path for Aerial Vehicles	Huawei, HiSilicon	discussion	Rel-15	LTE_Aerial-Core

=>	Noted

R2-1803348	Mobility enhancements for UAVs – planned flight route	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_Aerial-Core	R2-1713264

R2-1802907	Measurement reporting information for Aerial UE	Intel Corporation	discussion	Rel-15	LTE_Aerial-Core

Agreement:

UE location information are included in the measurement report for Aerial UE based on the existing location information IE and reporting mechanism. Any parameters for reporting can be further studied.

R2-1802807	Indication of UE airborne status and support of UAV functions	InterDigital	discussion	Rel-15	LTE_Aerial

R2-1802705	Mobility Enhancement using Location Information	LG Electronics Inc.	discussion	Rel-15	LTE_Aerial-Core

29

R2-1802708	Enhancement to History Information for Aerial UE	LG Electronics Inc.	discussion	Rel-15	LTE_Aerial-Core

R2-1802788	Airborne status indication and related mobility enhancements	Ericsson	discussion	Rel-15	LTE_Aerial-Core

Others:

R2-1803349	Mobility enhancements for UAVs - reference altitude	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_Aerial-Core	R2-1713265

-	Qualcomm concerns the privacy issue.

Agreement:

Provide reference altitude information (including threshold) to UAV UE provided by eNB to assist UE to identify its status (i.e., airborne status).

R2-1802538	Enhanced MSE using tier for aerial UE	LG Electronics Mobile Research	discussion	LTE_Aerial-Core

R2-1803049	Discussion points for UAV mobility enhancement	KDDI Corporation	discussion

R2-1803129	Location based mobility enhancements for UAVs	Sony	discussion	Rel-15	LTE_Aerial-Core

R2-1803217	Air-borne UE dedicated mobility management	CMCC	discussion	Rel-15	LTE_Aerial-Core

R2-1803384	Mobility Enhancements for Aerial UEs	Fraunhofer HHI, Fraunhofer IIS	discussion	Rel-15

R2-1803420	UE-based mobility solution with Uplink Interference coordination 	Kyocera	discussion

Airborne status detection:

R2-1802305	Discussion on airborne status management for aerial UE	Lenovo, Motorola Mobility	discussion	Rel-15	LTE_Aerial-Core

=>	Noted

R2-1802615	UE’s airborne status indication	Nokia, Nokia Shanghai Bell	discussion	Rel-15	LTE_Aerial-Core

R2-1803138	Potential handover enhancements for aerial UE	Sony	discussion	Rel-15	LTE_Aerial-Core	R2-1712992

R2-1803139	Discussion on identification of aerial UE	Sony	discussion	Rel-15	LTE_Aerial-Core

R2-1803218	MDT-based air-borne UE identification	CMCC	discussion	Rel-15	LTE_Aerial-Core

Others:

R2-1802303	Discussion on signaling exchange issues for UL interference detection	Lenovo, Motorola Mobility	discussion	Rel-15	LTE_Aerial-Core

R2-1802663	Potential enhancements for drones in idle state	Huawei, HiSilicon	discussion	Rel-15	LTE_Aerial-Core

R2-1802707	HOF or RLF Handling of Aerial UE	LG Electronics Inc.	discussion	Rel-15	LTE_Aerial-Core

R2-1802654	Introduction of height dependant TTT for Aerial Vehicles for TS 36.331.	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3240	-	B	LTE_Aerial-Core

R2-1802659	Introduction of flight path for Aerial Vehicles for TS 36.331	Huawei, HiSilicon	CR	Rel-15	36.331	15.0.1	3241	-	B	LTE_Aerial-Core

R2-1802660	Introduction of flight path for Aerial Vehicles for TS 36.306	Huawei, HiSilicon	CR	Rel-15	36.306	14.5.0	1553	-	B	LTE_Aerial-Core

R2-1802664	Introduction of drone related SIBs for Aerial Vehicles for TS 36.331.	Huawei, HiSilicon	discussion	Rel-15	36.331	LTE_Aerial-Core

## Summary of the LTE Break-Out session

Agreed CRs:

Comeback on Friday :

euCA :

- CBF:=>	Draft LS in R2-xxxxx to RAN1/RAN4 to inform the agreements. Ask RAN4 if CSI reporting delay requirements (today 24 (34) ms after activation) can be reduced if UE has been monitoring the Scell carrier. (Nokia, offline#222)

feCoMP

R2-1802898	Introduction of further enhancements to CoMP	Intel Corporation	CR	Rel-15	36.331	15.0.1	3251	-	B	feCOMP_LTE-Core

=>	Revised in R2-xxxxx.

CBF offline#255 (Intel)

Other LRE R15

CBF:=>	Draft LS in R2-xxxxxx to SA4, SA5  cc RAN3 to ask the meaning of new service and ask the possibility to address this in container. Add description that this cannot be handled in LTE QMC (Ericsson ,offline#234)

CBF:=>	Draft a reply LS in R2-xxxx to SA5 cc SA4, RAN3 saying that RAN2 has analysed the proposed new attributes and concluded that if the areaScope is not sent to the UE the size of the RRC message is decreased.  Ask the possibility of adding new service type in container (Ericsson offline#234).

TEI 15 L2 measurement:

CBF: Check the following proposal can be agreed or not and try to agree CR of TS36.314 (Huawei, offline 298)

- Proposal: for use case 2, Introduce following two new measurements:

- Distribution of Scheduled IP Throughput in DL

- Distribution of Scheduled IP Throughput in UL

=>	For the new measurements, it is proposed to have new section(s) to describe them.

CBF:=>	(Huawei, offline 299) Draft LS to SA5:

- for use case 1, regarding the following two new measurements, consult SA5 on impacts due to lot of samples.

- Distribution of PRB usage per traffic class for DL

- Distribution of PRB usage per traffic class for UL

for use case 2, regarding the following two new measurements, consult SA5 on impacts due to lot of samples.

- Distribution of Scheduled IP Throughput in DL (per QCI)

- Distribution of Scheduled IP Throughput in UL (per QCI)

for use case 3, consult SA5 on the following two comments:

- whether the existing performance measurements and their combinations provide various means to reflect effectively RRC-connected UEs, e.g. RRC measurements defined in TS 32.425

- the intention, i.e. in case if UE has multiple E-RABs with different QCI, then the same UE could be counted twice.

UAV:

CBF:=>	Draft LS to SA2 with the contents of the work split (DoCoMo, offline#205)

Email discussion:

BT/WLAN measurement collection:

Email discussion on Introduction of Bluetooth/ WLAN measurement collection in MDT in 36.306 (CMCC)

Deadline:

Intended outcome: endorsed running CR

Email discussion on Introduction of Bluetooth/ WLAN measurement collection in MDT in 36.331 (Huawei)

Deadline:

Intended outcome: endorsed running CR

Email discussion on Introduction of Bluetooth/ WLAN measurement collection in MDT in 37.320 (CMCC)

Deadline:

Intended outcome: endorsed running CR

euCA:

[101#xx][LTE/euCA] Introducing valid area mechanism (vivo)

-	Identify the scenarios of valid area

-	discuss whether or not to introduce valid area mechanism

Intended outcome: Report from email discussion

Deadline:

UDC:

[101#xx][LTE/UDC] Introduction of DEFLATE based UDC Solution in TS36.323 (CATT)

Intended outcome: endorsed CR

Deadline: One week

[101#xx][LTE/UDC] Introduction of DEFLATE based UDC Solution in TS36.331 (CATT)

Intended outcome: endorsed CR

Deadline: One week

[101#xx][LTE/UDC] Introduction of DEFLATE based UDC Solution in TS36.306 (CATT)

Intended outcome: endorsed CR

Deadline: One week

[101#xx][LTE/UDC] Introduction of DEFLATE based UDC Solution in TS36.300 (CATT)

Intended outcome: endorsed CR

Deadline: One week

TEI: HSDN

[101#xx][LTE/TEI15] Support of mobility-state-based cell reselection in the high-speed-railway dedicated LTE network in 36.304 (OPPO)

Intended outcome: endorsed CR

Deadline: until next meeting

[101#xx][LTE/TEI15] Support of mobility-state-based cell reselection in the high-speed-railway dedicated LTE network in 36.331 (CMCC)

Intended outcome: endorsed CR

Deadline: until next meeting

feLAA:

[101#xx][LTE/feLAA] Introduce feLLA in TS 36.331 (Nokia)

Capture the agreements so far from RAN1 and Ran2

Intended outcome: endorsed running CR

Deadline:

[101#xx][LTE/feLAA] Introduce feLLA in TS 36.321 (Ericsson)

Intended outcome: endorsed running CR

Deadline:

HRLLC:

[101#xx][LTE/URLLC] The granularity of time reference discussion (Huawei)

- - Discussion on use cases and requirements on synchronization accuracy and identify the gap between the requirement and the current synch accuracy

- Discussion on granularity of time

Intended outcome: Report of email discussion

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.321 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.323 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.322 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.331 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

[101#xx][LTE/URLLC] Introduction of URLLC in LTE in TS 36.300 (Ericsson)

Intended outcome: endorsed running CR for next meeting

Deadline:

Agreements:

BT/WLAN measurement collection:

Agreement:

Immediate MDT, Logged MDT and RLF report are supported for WLAN/BT measurement collection if the measurements available.

Agreements:

1	UE reports WLAN/BT measurements to eNB.

2	For BT measurements, reuse the measurement information (i.e., btAddr, rssi) already defined in LPP specification 36.355 as much as possible.

For WLAN measurements, reuse the measurements in specification 36.331 as much as possible.

3	For Immediate MDT, WLAN and BT measurements is carried as measurement results.

4	The reporting form of BT/WLAN measurements in logged MDT is carried as measurement results.

5	At least the MAC addresses of Bluetooth beacons and BSSID/SSID/HESSID of WLAN APs should be reported if UE reports Bluetooth/WLAN measurements.

6	If requested by the network, the report can further include RSSI for Bluetooth beacons and both RSSI and RTT for WLAN Aps if these measurement results available (the availability has the same meaning as available GNSS information in Rel-10 MDT) in UE.

6.1	Enforce UE to turn on WLAN/BT is not supported.

7	2 bits should be defined separately in UE capability to indicate whether the UE support Bluetooth and WLAN measurements or not.

8    The existing MDT measurement configuration procedures can be used.

=>	To minimize the impacts due to large WLAN and Bluetooth reports:

-	Decrease maximum number of WLAN APs and Bluetooth beacons measured.

-	Restrict the configuration parameters for WLAN measurement.

-	Measurement results are carried by SRB 2.

Agreement:

1	The UE includes in the availability indicator an indication of whether any WLAN measurement is logged.

euCA:

Agreements:

1	Adopt the new common parameters structure as proposed in R2-1713491 / R2-1712257.

2	Dedicated Scell configuration can be used to override common parameter configuration.

3	Four common SCell parameter groups are allowed by the RRC signalling.

4	Do not define a new group-based MAC CE.

Agreements:

1	Do not define a validity timer for IDLE measurement configuration in SIB5. It doesn’t exclude UE performs aperiodic measurement.

2	The information that can be broadcast in SIB5 should be limited to at most 3 inter-frequencies and at most N cells per carrier frequency. Value of N is FFS.

3	UE can be provided with a “CA measurement configuration” within RRCConnectionRelease that is different from SIB5 configuration, and which overrides any SIB5 configurations. FFS the dedicated configuration is still valid after cell reselection.

4	Validity timer is only defined for configuration given in RRCConnectionRelease.

5	Only CRS-based measurements (RSRP/RSRQ/RS-SINR) are supported for IDLE mode measurements.

6	Network can indicate which measurement quantities out of RSRP, RSRQ and RS-SINR UE should report for IDLE mode measurements.

Agreements:

1	UE may continue performing IDLE state euCA measurement after cell reselection.

If the ongoing IDLE state measurement is configured only via SIB5 in source cell:

2	After cell reselection, if the SIB5 of the target cell includes the IDLE state measurement configuration, UE continues performing IDLE state measurement and updated the configuration according to the SIB5 of the target cell.

3	After cell reselection, if the SIB5 of the target cell includes no configuration for the IDLE state measurement, UE stops performing IDLE state measurement.

Agreement:

The following behaviour is to adopt for reporting of the IDLE mode measurements:

The UE waits for an explicit eNB-request for the IDLE mode measurements. The measurements should be reported after security establishment.

Agreements:

1	Timer T331 starts upon RRC Connection release message received by UE.

2	Minimum SCell Signal Quality threshold for reporting may be configured by eNB.

Agreements

1	When SCell is configured as activated, the PUCCH format changes to take this into account at n+20 (i.e. after the RRC processing delay). This should be confirmed from RAN4.

Agreements:

1	Use “Dormant SCell state” as the name of the new Scell state and use ”Hibernate” as the action for moving from Activated SCell state to Dormant Scell state if necessary.

2	Dormant Scell follows PCell DRX for CQI/RRM measurement report triggering.

3	Additional new MAC CE is needed for state transition in/out of the dormant state. Legacy MAC CE is still used as in Rel-10 CA.

4	Define a MAC CE for state transition from activated SCell state to the dormant SCell state.

5    It is supported that UE transits from dormant state to deactivate state.

UDC:

Agreements:

1	Define an associated PLMN ID in UDC related UE capability reporting if it reports supporting operator defined dictionary.

2	From RAN2 perspective, UE is not required to report supporting operator defined dictionary when it enters a roaming network.

3	4 bits version ID for operator defined dictionary can be defined in UDC related UE capability reporting.

Agreement:

1	Specify in RAN2 spec that UDC configuration is not used for split and LWA DRBs.

Agreement:

1	Upon initiation of RRC connection resume, the UE shall release the UDC configuration if configured.

Agreement:

Replace “processed by UDC” by “compressed by UDC” in RAN2 spec to clarify UDC function behavior.

Agreements:

There are three types of UE capability regarding UDC:

-	(1)	basic UDC functionality

-	(2)	standard dictionary

-	(3)	operator defined dictionary

Dictionary related capabilities (2) and (3) are dependent on the basic UDC functionality (1). In addition, A UE that supports (1) shall support 8192 bytes for compression buffer per DRB.

Agreement:

The UE can set FR bit to trigger a history buffer reset without getting a request from eNB before.

=>	From RAN perspective the WI is completed.

TEI: L2 measurement

Agreement:

1	for use case 1, introduce the following two new measurements:

- Distribution of total PRB usage for DL

- Distribution of total PRB usage for UL

TEI 15: HSDN

Agreement:

1	Introduce 1 new bit indication in SIB1 to indicate the high speed dedicated network cell.

2	Maximum value for the weight for high speed dedicated network cell is 16. New value of MSE cell count for HSDN case can be further considered.

3	When the UE is in high mobility state, the UE prioritize to re-select the high speed dedicated network cell regardless of frequency priority.

4	When the UE supporting the HSDN feature is not in high mobility state, the UE deprioritize (i.e., lowest priority) to re-select the high speed dedicated network cell.

Agreements:

1	HSDN neighbouring cell list can be provided in SIB. FFS the content of the list.

feLAA:

Agreements:

1	Introduce multi-bit activation/deactivation MAC CE confirmation.

2	Introduce new RNTI for activation/deactivation.

3	For the AUL confirmation MAC CE, same triggering procedures specified for the Rel.14 SPS confirmation are adopted, i.e. AUL confirmation triggered at reception of PDCCH indicating AUL release/activation.

SR:

Agreements:

1	No enhancement on SR triggering will be introduced in FeLAA. No specification changes are needed.

2	No enhancement on SR cancellation will be introduced in FeLAA. No specification changes are needed.

Agreements

1	When AUL is configured, the UE is allowed to retransmit a MAC PDU in a later AUL subframe (e.g. in the next AUL subframe) if the associated LBT procedure failed, without the need to wait for a SUL grant, HARQ feedback or expiry of the retransmission timer X. This agreement can be consulted RAN1 for the feasibility.

2	RAN2 understanding is that “Physical layer informs the MAC layer on the outcome of the LBT procedure (e.g., by ACK/NACK), so that MAC can trigger a retransmission in a later subframe, without the need to wait for a SUL grant or the expiry of the retransmission timer X.” How to capture the modelling can be further studied.

Agreements:

1	In FeLAA, no enhancement on BSR/PHR trigger and cancellation is introduced.

2	It is up to UE implementation to solve issue of out of date BSR/PHR if necessary.

FFS the impact on spec.

Agreements:

1	Timer X is maintained per HARQ process for AUL.

2	UE shall not use the AUL resource to perform both new transmission and retransmission for the same HARQ process before the Timer X stops/expires.

3	Start/restart the Timer X after the initial AUL transmission/AUL retransmission of a given HARQ process.

Agreement:

Like for the pTAG, the expiry of an timeAlignmentTimer in an sTAG clears AUL resources for any associated SCell, hence AUL transmissions on those SCells are stopped

Agreements:

1	The PDCP entity transmits duplicated PDCP PDU only when requested by lower layers, as in LTE

2	Support RLC AM for packet duplication via DC for DRB.

3	Support RLC AM for packet duplication via CA for DRB.

4	The same PDCP mechanism for “Data volume calculation” can be used for both CA and DC duplication.

5	For DC case, when PDCP duplication is activated, all PDCP data is reported to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the second RLC entity.

6	For CA case, when PDCP duplication is activated, all PDCP data is reported to the MAC entity which is the same as agreed in NR session.

7	For CA case, when PDCP duplication is configured but not activated, all PDCP data is reported only to the MAC entity associated with the primary RLC entity.

8	Introduce one byte bitmap for MAC CE activation/deactivation of PDCP duplication at least for DRB.

9	Introduce UE capabilities for packet duplication.

Agreements:

1	The UE shall discard packets that have been acknowledged by RLC in the other RLC leg. PDCP should indicate to the other associated RLC entity to discard the corresponding PDCP PDU. RLC procedures and PDCP discard procedures are not impacted by this agreement.

2	For CA and DC upon deactivation of PDCP data duplication, the UE transmitting PDCP entity should indicate to lower layers to discard all PDCP PDUs provided for duplicate transmission to the secondary RLC entity.

Agreements:

1	 At PDCP re-establishment, when retransmitting PDCP SDUs for AM DRBs, duplicated PDUs are submitted to both associated RLC entities when PDCP duplication is activated.

2	 At PDCP data recovery, when retransmitting PDCP SDUs for AM DRBs, duplicated PDUs are submitted to both associated RLC entities when PDCP duplication is activated.

3	For packet duplication via CA, meeting the maximum number of RLC retransmissions for the secondary leg does not trigger RLF.

Agreements:

1	SRB duplication activation/deactivation via MAC CE is not supported in LTE. Once the duplication for SRB is configured, it is always activated.

2	RLC UM split bearer, i.e. two UM mode RLCs configured for one PDCP-config needs to be supported in LTE for the packet duplication.

3	For DC, when DRB duplication is deactivated via MAC CE, the UE falls back to the split bearer operation. For CA, when DRB duplication is deactivated via MAC CE, the UE falls back to the single RLC operation.

4	RRC can be used to de-configure the packet duplication directly.

Agreement:

1	Maintain the current LTE prioritization rules for handling MAC CE and LCHs configured with enhanced reliability requirements.

Agreements

1	A variable number of consecutive repetitions in time for SPS UL should be supported and is to be controlled by the eNB through RRC signalling.

UAV:

Agreement:

1	RAN2 confirm the following work/responsibility split between WGs:

RAN2:

-	To clarify and confirm the usage of the subscription based identification and the overall stage2 behaviour of UAV identification signaling.

RAN3:

-	To define the signaling/IE for subscription based UAV identification.

SA2:

-	To define the subscription based UAV identification and the necessary signaling in interfaces between CN nodes (e.g., MME – HSS, etc.)

2	RAN2 to confirm that the subscription based information is signaled from the MME to the eNB

Agreement:

UE location information are included in the measurement report for Aerial UE based on the existing location information IE and reporting mechanism. Any parameters for reporting can be further studied.

Agreement:

Provide reference altitude information (including threshold) to UAV UE provided by eNB to assist UE to identify its status (i.e., airborne status).