3GPP TSG-RAN WG2 #101 NR	Tdoc R2-1803857

Athens, Greece, 26 Feb - 2 March 2018

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	configuredGrantTimer values

Document for:	Discussion, Decision

# configuredGrantTimer values

The Periodicity and the configuredGrantTimer discussion concluded:

R2-1802205 Miscellaneous corrections to calculation of configured grant   Huawei, HiSilicon        discussion   Rel-15 38.321 NR_newRAT-Core

-     Nokia thinks that we should change the RRC if anything is needed.  Huawei thinks that RAN1 has a table.  Ericsson understands that RRC would need to signal something.

-     Interdigital explains that they are still discussing in RAN1 the table

-     Nokia thinks that we anyways need to configure the periodicity in RRC

=>  The value for configured grant should be in symbols and needs to be updated in RRC

=>  The change on adding timedomainallocation and × numberOfSymbolsPerSlot to the formula is agreeable.  Rapporteur will merge it in main CR

=>  Not agreed

The values below would then to be updated w.r.t to the periodicity:

UL transmission without UL grant for type 1 and type 2:

periodicity                           ENUMERATED {sym2, sym7, ms0dot125, ms0dot25, ms0dot5, ms1, ms2, ms5, ms10, ms20,

ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640},

What is captured in ASN.1 is then the values in the RAN1 table without translation, e.g.  2,7, n*14, n= {1,…}:

Table 6.1.2.3-1: Allowed periodicities P for uplink transmission without grant

|  | CP | Possible values of periodicities P [symbols] |
| --- | --- | --- |
| 0 | Normal | 2, 7, n*14, where n={1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640} |
| 1 | Normal | 2, 7, n*14, where n={1, 2, 4, 10, 20, 40, 64, 80, 128, 160, 256, 320, 640, 1280} |
| 2 | Normal | 2, 7, n*14, where n={1, 2, 4, 8, 20, 40, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} |
| 2 | Extended | 2, 6, n*12, where n={1, 2, 4, 8, 20, 40, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} |
| 3 | Normal | 2, 7, n*14, where n={1, 2, 4, 8, 16, 40, 80, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} |

With the assumption from the configuredGranttimer agreements (to be updated), below:

=>  configuredGrantTimer value unit is OFDM symbol if the configured grant periodicity is in number of OFDM symbols.

=>  configuredGrantTimer value unit is slot if the configured grant periodicity is in number of milliseconds.

=>  A candidate value list is further proposed below:

configuredGrantTimer value list can be down select from [1, 2, 4, 8, 12, 16, 20, 24, 28, 32, 40, 64, 80, 128, 160, 320, 640, 1280] [CB – Ericsson consider number of bits and values?]

Use configuredGrantTimer values from 14 x n/ 12 x n [2, 4, 8, 12, 16, 20, 24, 28, 32, 40, 64, 80, 128, 160, 320, 640, 1280, …] in the same as they are used for the CG periodicity in ASN.1

I.e

 For normal CP: 2, 7, {1, 2, 4, 5, 8, 10, 20, 32, 40, 64, 80, 128, 160, 256, 512, 640 } x 14

 For extended CP: 2, 6, {1, 2, 4, 8, 20, 40, 80, 128, 160, 256, 320, 512, 640 } x 12

This would be simple and match the periodicities up to 640 slots and excluding the largest values (same structure).

The larger values is not needed for this timer.