3GPP TSG-RAN WG2 #101	Tdoc R2-1803898

Athens, Greece, 26th Feb – 2nd Mar, 2018

Agenda Item:	10.2.14

Source:	Ericsson

Title:	TP on 38.300 for RNA Configuration

Document for:	Decision

# Introduction

This contribution contains the updated text proposal for RAN-based Notification Area configuration based on offline discussion [RAN2#101 offline#19].

# Text Proposal

#### 9.2.2.3	RAN-Based Notification Area

A UE in the RRC_INACTIVE state can be configured with an RAN-based Notification Area (RNA), where:

-	the RNA can cover a single or multiple cells, and shall be contained within the CN registration area;

-	a RAN-based notification area update (RNAU) is periodically sent by the UE and is also sent when the cell reselection procedure of the UE selects a cell that does not belong to the configured RNA.

There are several different alternatives on how the RNA can be configured:

-	List of cells:

-	A UE is provided an explicit list of cells (one or more) that constitute the RNA.

-	List of RAN areas:

-	A UE is provided (at least one) RAN area ID, where a RAN area is a subset of a CN Tracking Area or equal to a CN Tracking Area. A RAN area is specified by one RAN area ID, which consists of a TAI and optionally a RAN area Code;

-	A cell broadcasts a RAN area ID in the system information.

NG RAN may provide different RNA definitions to different UEs but not mix different definitions to the same UE at the same time. UE shall support all RNA configuration options listed above.