3GPP TSG-RAN WG2 Meeting #101 	R2-1804036

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:

Source:	Ericsson

Title:	Offline discussion on Multiplicity definitions in 38331 (E359)

Document for:	Discussion, Decision

# Introduction

This document lists constants (multiplicity definitions) in TS38.331, together with values discussed/agreed during RAN2#101 ASN.1 review session.

The list was originally submitted in R2-1803553.

| Area | Name | Current spec value | Proposed value | Description from 38.331 | Comments |
| --- | --- | --- | --- | --- | --- |
| L1 | maxCoReSetDuration | 3 |  | -- Max number of OFDM symbols in a control resource set |  |
| L1 | maxEARFCN | 262143 |  | -- Highest value of extended E-UTRA EARFCN range Note: there might be impact of offset discussed in RAN1 |  |
| L1 | maxINT-DCI-PayloadSize |  |  | -- Max number payload of a DCI scrambled with INT-RNTI | RAN1 information needed. |
| L1 | maxINT-DCI-PayloadSize-1 |  |  | -- Max number payload of a DCI scrambled with INT-RNTI minus 1 | RAN1 information needed. |
| L1 | maxNrofBWPs | 4 |  | -- Maximum number of BWPs per serving cell |  |
| L1 | maxNrofControlResourceSets | 12 |  | -- Max number of CoReSets configurable on a serving cell |  |
| L1 | maxNrofControlResourceSets-1 | 11 |  |  | RAN1 information needed. |
| L1 | maxNrofCSI-IM-Resources |  |  | -- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214. | RAN1 information needed. |
| L1 | maxNrofCSI-IM-Resources-1 |  |  | -- Maximum number of CSI-IM resources minus 1. See CSI-IM-ResourceMax in 38.214. | RAN1 information needed. |
| L1 | maxNrofCSI-IM-ResourcesPerSet |  |  | -- Maximum number of CSI-IM resources per set. See CSI-IM-ResourcePerSetMax in 38.214 | RAN1 information needed. |
| L1 | maxNrofCSI-MeasId |  |  | -- Maximum number of link configurations | RAN1 information needed. |
| L1 | maxNrofCSI-MeasId-1 |  |  | -- Maximum number of link configurations minus 1 | RAN1 information needed. |
| L1 | maxNrofCSI-ResourceConfigurations |  |  | -- Maximum number of resource configurations | RAN1 information needed. |
| L1 | maxNrofCSI-ResourceConfigurations-1 |  |  | -- Maximum number of resource configurations minus 1 | RAN1 information needed. |
| L1 | maxNrofCSI-ResourceSets |  |  | -- Maximum number of resource sets per resource configuration | RAN1 information needed. |
| L1 | maxNrofCSI-ResourceSets-1 |  |  | -- Maximum number of resource sets per resource configuration minus 1 | RAN1 information needed. |
| L1 | maxNrofCSI-ResrouceConfigurations |  |  |  | RAN1 information needed. |
| L1 | maxNrofCSI-RS |  |  |  | RAN1 information needed. |
| L1 | maxNrofCSI-RS-ResourcesPerSet | 8 |  | - Maximum number of CSI-RS resources per resource set |  |
| L1 | maxNrofDL-Allocations |  |  |  | RAN1 information needed. |
| L1 | maxNrofNZP-CSI-RS-Resources-1 |  |  | -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1 | RAN1 information needed. |
| L1 | maxNrofP0-PUSCH-AlphaSets | 30 |  | -- Maximum number of P0-pusch-alpha-sets (see 38,213, section 7.1) |  |
| L1 | maxNrofP0-PUSCH-AlphaSets-1 | 29 |  | -- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, section 7.1) |  |
| L1 | maxNrofPCIsPerSMTC |  |  |  | RAN1 information needed. |
| L1 | maxNrofPhysicalResourceBlocks | 275 |  | -- Maximum number of PRBs |  |
| L1 | maxNrofPhysicalResourceBlocks-1 | 274 |  | -- Maximum number of PRBs |  |
| L1 | maxNrofPUCCH-P0-PerSet | 8 |  | -- Maximum number of P0-pucch present in a p0-pucch set |  |
| L1 | maxNrofPUCCH-PathlossReferenceRSs | 4 |  | -- Maximum number of RSs used as pathloss reference for PUCCH power control. |  |
| L1 | maxNrofPUCCH-PathlossReferenceRSs-1 | 3 |  | -- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1. |  |
| L1 | maxNrofPUCCH-ResourceSets | 4 |  | -- Maximum number of PUCCH Resource Sets |  |
| L1 | maxNrofPUCCH-ResourceSets-1 | 3 |  | -- Maximum number of PUCCH Resource Sets minus 1. |  |
| L1 | maxNrofPUCCH-ResourcesPerSet | 8 |  | -- Maximum number of PUCCH Resources per PUCCH-ResourceSet |  |
| L1 | maxNrofPUCCH-ResourcesPerSet-17 |  |  |  | RAN1 information needed. |
| L1 | maxNrofPUSCH-PathlossReferenceRSs | 4 |  | -- Maximum number of RSs used as pathloss reference for PUSCH power control. |  |
| L1 | maxNrofPUSCH-PathlossReferenceRSs-1 | 3 |  | -- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1. |  |
| L1 | maxNrofQFIs |  |  |  | RAN1 information needed. |
| L1 | maxNrofRateMatchPatterns | 4 |  | -- Max number of rate matching patterns that may be configured |  |
| L1 | maxNrofRateMatchPatterns-1 | 3 |  | -- Max number of rate matching patterns that may be configured minus 1 |  |
| L1 | maxNrofSearchSpaces | 40 |  | -- Max number of Search Spaces |  |
| L1 | maxNrofSearchSpaces-1 | 339 |  | -- Max number of Search Spaces minus 1 |  |
| L1 | maxNrofSlotFormatCombinationsPerSet |  |  | -- Maximum number of Slot Format Combinations in a SF-Set. | RAN1 information needed. |
| L1 | maxNrofSlotFormatCombinationsPerSet-1 |  |  | -- Maximum number of Slot Format Combinations in a SF-Set minus 1. | RAN1 information needed. |
| L1 | maxNrofSlotFormatsPerCombination |  |  |  | RAN1 information needed. |
| L1 | maxNrofSlots | 320 |  | -- Maximum number of slots in a 10 ms period |  |
| L1 | maxNrofSlots-1 | 319 |  | -- Maximum number of slots in a 10 ms period minus 1 |  |
| L1 | maxNrofSpatialRelationInfos |  |  |  | RAN1 information needed. |
| L1 | maxNrofSR-ConfigPerCellGroup | 8 |  | -- Maximum number of SR configurations per cell group | RAN1 information needed. |
| L1 | maxNrofSR-Resoruces |  |  |  | RAN1 information needed. |
| L1 | maxNrofSRS-Resources |  |  | -- Maximum number of SRS resources in an SRS resource set. | RAN1 information needed. |
| L1 | maxNrofSRS-Resources-1 |  |  | -- Maximum number of SRS resources in an SRS resource set minus 1. | RAN1 information needed. |
| L1 | maxNrofSRS-ResourceSets |  |  | -- Maximum number of SRS resource sets. | RAN1 information needed. |
| L1 | maxNrofSRS-ResourceSets-1 |  |  | -- Maximum number of SRS resource sets minus 1. | RAN1 information needed. |
| L1 | maxNrofSRS-ResourcesPerSet |  |  |  | RAN1 information needed. |
| L1 | maxNrofSRS-TriggerStates-1 | 3 |  | -- Maximum number of SRS trigger states minus 1, i.e., the largest code point. |  |
| L1 | maxNrofSSBs -1 | 63 | 63 | -- Maximum number of SSB resources in a resource set minus 1 |  |
| L1 | maxNrofSSBs |  | 64 |  | RAN1 information needed. |
| L1 | maxNrofSymbols-1 | 13 |  | - Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13) |  |
| L1 | maxNrofTCI-States | 64 |  |  |  |
| L1 | maxNrofTCI-States-1 | 63 |  |  |  |
| L1 | maxNrofTCI-StatesPDCCH |  |  |  | RAN1 information needed. |
| L1 | maxNrofUL-Allocations |  |  |  | RAN1 information needed. |
| L1 | maxNrofZP-CSI-RS-Resources | 3 |  | -- Maximum number of Zero-Power (NZP) CSI-RS resources |  |
| L1 | maxNrofZP-CSI-RS-Resources-1 | 2 |  | -- Maximum number of Zero-Power (NZP) CSI-RS resources minus 1 |  |
| L1 | maxQFI |  |  |  | RAN1 information needed. |
| L1 | maxRA-CSIRS-Resources |  |  |  | RAN1 information needed. |
| L1 | maxRA-SSB-Resources |  |  |  | RAN1 information needed. |
| L1 | maxSCSs |  |  |  | RAN1 information needed. |
| L1 | maxSFI-DCI-PayloadSize |  |  | -- Max number payload of a DCI scrambled with SFI-RNTI | RAN1 information needed. |
| L1 | maxSFI-DCI-PayloadSize-1 |  |  | -- Max number payload of a DCI scrambled with SFI-RNTI minus 1 | RAN1 information needed. |
| L2 | maxCellSCG |  | - |  | Same as maxNrofSCells. Can be deleted. |
| L2 | maxDRB |  | 16 | Draft: Max mumber of DRBs (that can be added in DRB-ToAddModLIst) | Currently, this sets the max number of DRBs that can be added/modified/deleted. Proposal: 16 Offline on this so needs to be aligned |
| L2 | maxLCG-ID | 7 |  | -- Maximum value of LCG ID |  |
| L2 | maxLCH |  | - |  | Currently, this sets the max number of LCHs (RLC Bearers) that can be added/modified/deleted. Deleted in email discussion #5, since same as maxLC-ID. |
| L2 | maxLC-ID |  | 20 | -- Maximum value of Logical Channel ID | Proposed to be set to “max number of DRBs” + “max number of SRBs” = 16 + 4 = 20 To be updated based on offline, this will be either 20 or 36 |
| L2 | maxNrofTAGs | 4 |  | -- Maximum number of Timing Advance Groups |  |
| L2 | maxNrofTAGs-1 | 3 |  | -- Maximum number of Timing Advance Groups minus 1 |  |
| UE Cap | maxBandComb |  | 2^16 | - Maximum number of DL band combinations |  |
| UE Cap | maxBands |  | 1024 |  |  |
| UE Cap | maxBasebandProcComb |  | 2^16 | -- Maximum number of base band processing combinations |  |
| UE Cap | maxRAT-CapabilityContainers |  | 8 | -- Maximum number of interworking RAT containers (incl NR and MRDC) |  |
| UE Cap | maxSecondaryCellGroups |  | 3 |  | Currently, defines the max value of CellGroupID. Proposal: rename to maxCellGroupID (3), meaning values 0-3 are used (4 Cell Groups |
| UE Cap | maxSimultaneousBands |  | 32 | -- Maximum number of simultaneously aggregated bands | Defines the max number of cells (component carriers) in a CA/DC band combination. In LTE, value is used, although 32 is the upper limit after Rel-13 B5C work item. There can be max 16 cells in a cell group. |
| Cell | maxNrofSCells | 31 | 31 | -- Max number of secondary serving cells per cell group | Note: max number of cells is 16 in NR CA but 32 selected for future proofness |
| Cell | maxNrofServingCells |  | 32 | -- Max number of serving cells (SpCell + SCells) per cell group |  |
| Cell | maxNrofServingCells-1 | 15 | 31 | -- Max number of serving cells (SpCell + SCells) per cell group minus 1 |  |
| RRM | maxCellPrep |  | 32 |  | In LTE, the corresponding parameter was maxReestabInfo (32). Only used in inter-node message HandoverPreparationInformation. Proposal: - Reuse LTE name and value (32) - use constant local in Inter-node module |
| RRM | maxCellReport |  | 8 |  | Current LTE value. |
| RRM | maxFreq |  | 8 | DRAFT: Max number of non-serving frequencies possible to include in MeasResultSCG-Failure. | Rename to maxNonServFreqReport, or(preferred) include directly in SIZE definition. |
| RRM | maxNrofAggregatedCellsPerCellGroup |  | 32 | This is related to L1 parameter. Can use same value as maxNrofServingCells (16). | This is related to L1 parameter. Can use same value as maxNrofServingCells (16), or (preferred) reuse this constant. |
| RRM | maxNrofCandidateBeams |  | 16 |  | Defines the max number of PRACH-ResourceDedicatedBFR that can be signalled in BFR config. Expected RAN4 defines performance requirement. RAN1/RAN4 feedback needed. |
| RRM | maxNrofCellMeas |  | 32 | -- Maximum number of entries in each of the cell lists in a measurement object. | 32 is used in LTE. Same can be used here |
| RRM | maxNrofCSI-ReportConfig-1 |  |  |  | Need further RAN2 discussion - check with RAN1 (Huawei) |
| RRM | maxNrofCSI-Reports |  |  | -- Maximum number of report configurations | Need further RAN2 discussion |
| RRM | maxNrofCSI-RS-CellsRRM |  | 96 | -- Maximum number of FFS | Need further RAN2 discussion |
| RRM | maxNrofCSI-RS-ResourcesRRM |  | 96 | -- Maximum number of CSI-RS resources for an RRM measurement object | Change CSI-RS-ResourceId-RRM ::= INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1) To CSI-RS-ResourceId-RRM ::= INTEGER (1..maxNrofCSI-RS-ResourcesRRM) |
| RRM | maxNrofCSI-RS-ResourcesRRM-1 |  |  | -- Maximum number of CSI-RS resources for an RRM measurement object minus 1 | Delete. See above |
| RRM | maxNrofCSI-RS-ResourcesToAverage |  | FFS | -- Max number for the (max) number of CSI-RS to average to determine cel measurement | Need further RAN2 discussion Huawei to check |
| RRM | maxNrofIndexesToReport |  | 32 |  | Need further RAN2 discussion |
| RRM | maxNrofMeasId |  | 64 | -- Maximum number of configured measurements | 64 is used in LTE |
| RRM | maxNrofObjectId |  | 64 | -- Maximum number of configured measurement objects | 64 is used in LTE |
| RRM | maxNrofPCI-Ranges |  | 8 | -- Maximum number of PCI ranges | In LTE, 4 is used for UTRA PCI ranges |
| RRM | maxNrofQuantityConfig | 2 |  | -- Maximum number of quantity configurations |  |
| RRM | maxNrofReportConfigIdsPerTrigger | 16 |  | -- Maximum number of report configurations per reportTrigger |  |
| RRM | maxNrofSS-BlocksToAverage |  | 16 | -- Max number for the (max) number of SS blocks to average to determine cell measurement | Need further RAN2 discussion |
| RRM | maxReportConfigId |  | 64 | -- Maximum number of reporting configurations | 32 is used in LTE |
| RRM | maxNrofFailureDetectionResources |  | 8 | -- Maximum number of failure detection resources | Need further RAN2 discussion Is this same as RLM-RS resources? |