3GPP TSG-RAN WG2 Meeting #101 	R2-1804524

# Sanya, China, 16 th – 20 th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 55 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PHR MAC CE for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently, the length of CC bitmap within the Multiple Entry PHR MAC CE depends on the highest SCellIndex of SCell with configured uplink. If it is less than 8, one octet will be used; otherwise four octets are used. If each node allocates SCellIndex for its own cells (from a set of SCellIndices available to that node) without involvement of the other node at the time of allocation of the SCell, the node with small SCellIndex range will not know the highest SCellIndex configured for UE. Hence it cannot determine which CC bitmap length will be used by UE in the Multiple Entry PHR MAC CE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two LCIDs are introduced for Multiple Entry PHR MAC CE, one for 1 octet CC bitmap and one for 4 octets CC bitmap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network is unclear which CC bitmap length is used in the Multiple Entry PHR MAC CE reported by the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs and padding. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs and padding. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to zero.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 000000 | CCCH |
| 000001–100000 | Identity of the logical channel |
| 100001–101111 | Reserved |
| 110000 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 110001 | PUCCH spatial relation Activation/Deactivation |
| 110010 | SP SRS Activation/Deactivation |
| 110011 | SP CSI reporting on PUCCH Activation/Deactivation |
| 110100 | TCI State Indication for UE-specific PDCCH |
| 110101 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 110110 | Aperiodic CSI Trigger State Subselection |
| 110111 | SP CSI-RS / CSI-IM Resource Set Activation/Deactivation |
| 111000 | Duplication Activation/Deactivation |
| 111001 | SCell Activation/Deactivation (4 octet) |
| 111010 | SCell Activation/Deactivation (1 octet) |
| 111011 | Long DRX Command |
| 111100 | DRX Command |
| 111101 | Timing Advance Command |
| 111110 | UE Contention Resolution Identity |
| 111111 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 000000 | CCCH |
| 000001–100000 | Identity of the logical channel |
| 100001–110101 | Reserved |
| 110110 | Configured Grant Confirmation |
| 110111 | Multiple Entry PHR with one octet bitmap |
| 111000 | Multiple Entry PHR with four octets bitmap |
| 111001 | Single Entry PHR |
| 111010 | C-RNTI |
| 111011 | Short Truncated BSR |
| 111100 | Long Truncated BSR |
| 111101 | Short BSR |
| 111110 | Long BSR |
| 111111 | Padding |