3GPP TSG RAN2 Meeting #101bis	R2-18 1805334

# Sanya, China, 16th - 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.305 | CR | DRAFT | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of additional sensor methods (IMU) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE_acc_enh-Core |  |  |  |  |  |  | Date: |  | 2018-04-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To provide support for a new positioning method based on Additional Sensors |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for Additional Sensor measurements such as IMU, light sensors has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel.15 support for IMU measurement requirements can not be fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 36.355... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of change

## 4.3	Standard UE Positioning Methods

The standard positioning methods supported for E-UTRAN access are:

-	network-assisted GNSS methods;

-	downlink positioning;

-	enhanced cell ID method;

-	uplink positioning;

-	barometric pressure sensor method;

-	WLAN method;

-	Bluetooth method;

-	Terrestrial Beacon System method.

-	additional sensors method

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

These positioning methods may be supported in UE-based, UE-assisted/E-SMLC-based, eNB-assisted, and LMU-assisted/E-SMLC-based versions. Table 4.3-1 indicates which of these versions are supported in this version of the specification for the standardised positioning methods.

Table 4.3-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, E-SMLC-based | eNB- assisted | LMU-assisted/ E-SMLC-based | SUPL |
| --- | --- | --- | --- | --- | --- |
| - A-GNSS | - Yes | - Yes | - No | - No | - Yes - (UE-based and UE-assisted) |
| - Downlink Note1 | - No | - Yes | - No | - No | - Yes (UE-assisted) |
| - E-CID | - No | - Yes | - Yes | - No | - Yes (UE-assisted) |
| - Uplink | - No | - No | - No | - Yes | - No |
| - Barometric | - Yes | - Yes | - No | - No | - No |
| - WLAN | - Yes | - Yes | - No | - No | - Yes |
| - Bluetooth | - No | - Yes | - No | - No | - No |
| - TBS Note 2 | - Yes | - Yes | - No | - No | - Yes (MBS) |
| - Additional sensors | - Yes | - Yes | - No | - No | - No |
| - NOTE 1: This includes TBS positioning based on PRS signals. - NOTE 2: In this version of the specification only for TBS positioning based on MBS signals. |  |  |  |  |  |

Barometric pressure sensor, and additional sensors (e.g. IMU), WLAN, Bluetooth, and TBS positioning methods based on MBS signals are also supported in standalone mode, as described in the corresponding sections.

Next Change

### 4.3.x	Additional sensors

The additional sensors method makes use of different sensors such as accelerometers, gyros, magnetometers, light sensors to calculate the relative acceleration, velocity and displacement of UE. The UE could use the data to report it to the location server as auxiliary information for hybrid positioning.

This method should be combined with other positioning methods who can get an exact position to determine the 3D position of the UE.

# The operation of the sensor positioning method is described in clause 8.x.

Next Change

### 5.3.1	User Equipment (UE)

The UE may transmit the needed signals for uplink-based UE Positioning measurements and may make measurements of downlink signals from E-UTRAN and other sources such as different GNSS and TBS systems, WLAN access points, Bluetooth beacons, UE barometric sensors, accelerometer sensors, gyroscope sensors, magnetometer sensors and light sensors. The measurements to be made will be determined by the chosen positioning method.

The UE may also contain LCS applications, or access an LCS application either through communication with a network accessed by the UE or through another application residing in the UE. This LCS application may include the needed measurement and calculation functions to determine the UE's position with or without network assistance. This is outside of the scope of this specification.

The UE may also, for example, contain an independent positioning function (e.g., GPS) and thus be able to report its position, independent of the E-UTRAN transmissions. The UE with an independent positioning function may also make use of assistance information obtained from the network.

Next Change

## 8.x	Additional sensors positioning method

### 8.x.1	General

Additional sensors including Inertial measurement unit (IMU) and light sensors in respect to the barometric pressure sensor can be used to estimate the location of the UE, the UE velocity, altitude, acceleration and displacement, and indoor-to-outdoor movement. Inertial measurement unit (IMU) which is also referred as inertial navigation system (INS) is based on motion sensors (accelerometers), rotation sensors (gyroscopes), and occasionally magnetic sensors (magnetometers) that continuously calculate via dead reckoning the position, orientation, and velocity (direction and speed of movement) of the UE. With the result of other positioning methods which can get the exact position, the final position of the UE is computed.

The additional sensor positioning method are support in three modes:

UE-Assisted: The UE provides the additional sensors measurements (velocity, acceleration, relative displacement or processed sensor output ) to the network side (i.e. E-SMLC) to compute the final exact position of the UE. The calculation of the final exact position take place on the network side. The additional sensor measurement information includes any of

- Relative positioning information in relation to a reference position, or reference time

- The velocity or acceleration in earth-bounded coordinate system including a timestamp

- Relative transition information, indoor-outdoor or outdoor-indoor movement with certainty

- Statistical information about UE movement, rotations and vibrations reported in a UE bounded coordinate system. Also including light sensor measurements.

-	UE-Based: The UE performs the additional sensors measurements (velocity, acceleration, relative displacement) and calculates its position combined with other positioning methods with network assistance. The UE obtains a reference position at a reference time and can estimate an absolute position (earth-bounded) at a later time using additional sensors. The reference position can be obtained from the E-SMLC, or from the UE (e.g. using GNSS). The calculation of the final exact position takes place on the UE side.

-	Standalone: The UE performs additional sensors measurements and calculates its position combined with other positioning methods, None of the hybrid positioning methods require network assistance.

### 8.x.2	Information to be transferred between E-UTRAN Elements

This subclause defines the information (e.g., assistance data, position and/or measurement data) that may be transferred between E-UTRAN elements.

#### 8.x.2.1	Information that may be transferred from the UE to E-SMLC

The information transferred from the UE to the E-SMLC consists of capability information and location measurements or UE position. The supported information elements are given in Table 8.x.2.1-1.

Table 8.x.2.1-1: Sensor Measurement Information that may be transferred from UE to the E-SMLC

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Timestamp | Yes | Yes |
| Statistical Sensor Information |  |  |
| Accelerometer sensor measurements | Yes | No |
| Gyroscope sensor measurements | Yes | No |
| Magnetometer sensor measurements | Yes | No |
| Light sensor measurements | Yes | No |
| UE Displacement and Movement Information |  |  |
| UE relative position | Yes | No |
| Displacement Information | Yes | No |
| Acceleration Information | Yes | No |
| Velocity Information | Yes | No |
| UE Location Information |  |  |
| UE absolute position with uncertainty shape | No | Yes |
| Location Source (method(s) used to compute location) | No | Yes |

##### 8.x.2.1.1	UE-assisted mode

In the UE-assisted mode, the UE reports velocity, displacements and sensor measurements together with associated quality estimates.

##### 8.x.2.1.1.1	Statistical Sensor Information

The UE may report sensor statistics in a UE-based coordinate system, the statistics include the average and variance of each or the magnitude for each sensor measurement.

##### 8.x.2.1.1.2	UE Displacement and Movement Information

The UE may report movement and displacement information divided into segments, where each segment is for example a piece-wise linear acceleration between two points in time. Given a relative linear acceleration between multiple time points, a UE trajectory can be estimated at the E-SMLC. Likewise, the UE shall report the velocity in multiple segments, where each segment details the velocity between two points in time. The UE may translate the sensor measurements into earth-bounded relative displacements and report a sequence of displacements, where the first item corresponds to the starting position.

- x.2.1.2 UE-based mode

In the UE-based mode, the UE has obtained a reference position at a reference time and can estimate an absolute position (in earth-bounded   coordinates) at a later time using sensors. The reference position can be obtained from the E-SMLC, or from the UE (e.g. using GNSS). The UE will indicate together with the position information that the position estimate is based on sensor information/integration, possibly also indicating the time of the most recent position from which the sensor integration has been ongoing. The UE reports an indication of additional sensors positioning method used to calculate a fix.

##### 8.x.2.1.3	Standalone mode

In Standalone mode, the UE is not required to report any indication of using additional sensors positioning method to the E-SMLC.

#### 8.x.2.2	Information that may be transferred from the E-SMLC to the UE

Table 8.x.2.2-1 lists assistance data for UE-assisted and UE-based modes that may be sent from the E-SMLC to the UE.

Table 8.x.2.2-1: Information that may be transferred from the E-SMLC to the UE

Assistance Data

- Reference Position

- Light reference data

TBD

The reference position can be reported based on

- Absolute position, for example provided via another positioning method.

- Absolute time. The reference position is associated to a specific absolute time. An example is the absolute time when a specific location information report was sent from the UE to the E-SMLC, for example containing RSTD measurements of the OTDOA positioning method. The UE can relate to the reference position at this absolute time without being aware of the actual absolute position at this time.

### 8.x.3	Additional Sensors Location Information Transfer Procedure

The purpose of this procedure is to enable the E-SMLC to request additional sensor measurements or to enable the UE to provide sensor measurements to the E-SMLC for position calculation.

#### 8.x.3.1	E-SMLC initiated Location Information Transfer Procedure

Figure 8.x.3.1-1 shows the Location Information Transfer operations when the procedure is initiated by the E-SMLC.

Figure 8.x.3.1-1: E-SMLC-initiated Location Information Transfer Procedure

(1)	The E-SMLC sends a LPP Request Location Information message to the UE for invocation of sensor positioning. This request includes positioning instructions such as the positioning mode (UE-assisted, UE-based), specific requested UE measurements if any, and quality of service parameters (accuracy, response time), with possibility of indicating the reference position for the UE relative displacement report.

(2)	The UE performs the requested measurements and possibly calculates its own position. The UE sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) elapsed. If the UE is unable to perform the requested measurements, or if the Response Time provided in step (1) elapsed before any of the requested measurements have been obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

#### 8.x.3.2	UE-initiated Location Information Delivery Procedure

Figure 8.x.3.2-1 shows the Location Information delivery operations for additional sensor method when the procedure is initiated by the UE.

Figure 8.x.3.2-1: UE-initiated Location Information Delivery Procedure

(1)	The UE sends an LPP Provide Location Information message to the E-SMLC. The Provide Location Information message may include UE sensor measurements or location estimate already available at the UE.

#### 8.x.3.3	Assistance Data Transfer Procedure

The purpose of this procedure is to enable the E-SMLC to provide assistance data to the UE (e.g., as part of a positioning procedure) and the UE to request assistance data from the E-SMLC (e.g., as part of a positioning procedure or for autonomous self-localization (i.e., UE determines its own location)).

##### 8.x.3.3.1		E-SMLC initiated Assistance Data Delivery

Figure 8.x.3.3.1-1 shows the Assistance Data Delivery operations for the network-assisted sensor method when the procedure is initiated by the E-SMLC.

Figure 8.x.3.3.1-1: E-SMLC-initiated Assistance Data Delivery Procedure

(1)	The E-SMLC determines that assistance data needs to be provided to the UE (e.g., as part of a positioning procedure) and sends an LPP Provide Assistance Data message to the UE. This message may include any of the sensor assistance data defined in subclause 8.x.2.2.

##### 8.x.3.3.2	UE initiated Assistance Data Transfer

Figure 8.x.3.3.2-1 shows the Assistance Data Transfer operations for the network-assisted sensor method when the procedure is initiated by the UE.

Figure 8.x.3.3.2-1: UE-initiated Assistance Data Transfer Procedure

(1)	The UE determines that certain sensor assistance data is desired (e.g., in case the UE requires its own location with autonomous self-location or as part of a positioning procedure when the E-SMLC provided assistance data are not sufficient for the UE to fulfil the request) and sends a LPP Request Assistance Data message to the E-SMLC.  This request includes an indication of which specific sensor assistance data is requested.

(2)	The E-SMLC provides the requested assistance data in a LPP Provide Assistance Data message, if available at the E-SMLC. The entire set of assistance data may be delivered in one or several LPP messages. In this case, this step may be repeated by the E-SMLC several times. If any of the UE requested assistance data in step (1) are not provided in step (2), the UE shall assume that the requested assistance data are not supported, or currently not available at the E-SMLC. If none of the UE requested assistance data in step (1) can be provided by the E-SMLC, return any information that can be provided in an LPP message of type Provide Assistance Data which includes a cause indication for the not provided assistance data.

Next Change

## Annex B (informative): Use of LPP with SUPL

The design goal of LPP is to enable it to be used in user plane location solutions such as OMA SUPL ([17], [18]) and this informative annex shows how LPP can be used in SUPL 2.0.

# B.1	SUPL 2.0 Positioning Methods and Positioning Protocols

The following table shows how the 3GPP positioning protocols are supported in SUPL 2.0.

Table B.1-1: SUPL support of positioning methods

| Positioning Protocol: | RRLP (GSM/GPRS/WCDMA/ LTE/WLAN/WiMAX) | RRC (WCDMA) | LPP (LTE) |
| --- | --- | --- | --- |
| Positioning Method: |  |  |  |
| A-GPS (A-GANSS) SET Assisted |  |  |  |
| A-GPS (A-GANSS) SET Based |  |  |  |
| Autonomous GPS/GANSS |  |  |  |
| Enhanced Cell ID |  |  |  |
| Enhanced Observed Time Difference (E-OTD) | (GSM only) | NA | NA |
| Observed Time Difference of Arrival (OTDOA) NOTE 1 | NA |  |  |
| Barometric Pressure Sensors | NA |  |  |
| WLAN | NA |  |  |
| Bluetooth | NA |  |  |
| TBS NOTE 2 | NA |  |  |
| Additional sensors NOTE 3 | NA | NA |  |
| NOTE 1: This includes TBS positioning based on PRS signals, which is only supported in LPP (LTE). NOTE 2: TBS positioning based on MBS signals. NOTE 3: Additional sensors include IMU sensors, and any other sensor embedded in the UE. |  |  |  |

End of changes