3GPP TSG-RAN WG2#102 	R2-1806992

# Busan, Korea, 21th-25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0117 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further corrections with switching of bandwidth part and random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The latest agreed CR defining UE’s behaviour regarding BWP switching upon Random Access procedure initiation applies equally to CBRA and CFRA. However, it should not apply to a Random Access procedure triggered by the beam failure recovery procedure, since BeamFailureRecoveryConfig I.E. can configure, in paired spectrum, a DL BWP for running the BFR procedure (via explicitly search space and CORESET where to monitor the gNB response) with a different BWP index than the active UL BWP. It makes sense since there is no point in requesting the UE with on-going DL activity to switch its DL BWP for the only purpose of monitoring the gNB response to BFR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Section 5.15, switching the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP is not performed if the Random Access procedure was initiated for beam failure recocery. Impact analysis Impacted functionality: BWP operation. Inter-operability: - If the network is implemented according to the CR and the UE is not, the NW restricts its configuration flexibility by configuring, for such UEs, the search space and CORESET where to monitor the gNB response in the DL BWP with same index as the active UL BWP. - If the UE is implemented according to the CR and the network is not, there is no impact since the network can only configure the search space and CORESET where to monitor the gNB response in the DL BWP with same index as the active UL BWP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE operating in paired spectrum in an active DL BWP with a different index than the active UL BWP is unnecessarily forced to switch its DL BWP upon BFR trigger although it may have some on-going DL activity and NW has no ambiguity about its current active DL BWP. Also NW can only configure the search space and CORESET where to monitor the gNB response in the DL BWP with same index as the active UL BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR is implemented on top of rapporteur’s latest running CR 57 rev2 in R2-1806229 |  |  |  |  |  |  |  |  |

Start of change

5.15	Bandwidth Part (BWP) operation

In addition to clause 12 of TS 38.213 [6], this subclause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bandwidthPartInactivityTimer, or by the MAC entity itself upon initiation of Random Access procedure. Upon addition of SpCell or activation of an SCell, one BWP is initially active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

On the active BWP for each activated Serving Cell configured with a BWP, the MAC entity shall apply normal operations including:

1>	transmit on UL-SCH;

1>	transmit on RACH;

1>	monitor the PDCCH;

1>	transmit PUCCH;

1>	transmit SRS;

1>	receive DL-SCH;

1>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2.

On the inactive BWP for each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	not transmit on UL-SCH;

1>	not transmit on RACH;

1>	not monitor the PDCCH;

1>	not transmit PUCCH;

1>	not transmit SRS;

1>	not receive DL-SCH;

1>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2;

1>	suspend any configured uplink grant of configured Type 1.

Upon initiation of the Random Access procedure, the MAC entity shall:

1>	if PRACH occasions are

not configured for the active UL BWP):

2>	switch to initial DL BWP and UL BWP;

1> else:

2> if the active DL BWP does not have the same bwp-Id as the active UL BWP; and

2> the Random Access was not initiated for beam failure recovery (as specified in subclause 5.17):

3> switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP;

1>	perform the Random Access procedure on the active DL BWP and UL BWP.

If the MAC entity receives a PDCCH for BWP switching of a serving cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching while a Random Access procedure is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in subclauses 5.1.4 and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure on the new activated BWP; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the active BWP.

If the bwp-InactivityTimer is configured, the MAC entity shall for each activated Serving Cell:

1>	if the default-DL-BWP is configured, and the active DL BWP is not the BWP indicated by the default-DL-BWP; or

1>	if the default-DL-BWP is not configured, and the active DL BWP is not the initial BWP:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

3>	if there is no ongoing random access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

4>	start or restart the bandwidthPartInactivityTimer associated with the active DL BWP.

2>	if a PDCCH for BWP switching is received on the active DL BWP, and the MAC entity switches the active BWP:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if Random Access procedure is initiated on this Serving Cell:

3>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell.

3>	if the Serving Cell is SCell (other than PSCell):

4>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the default-DL-BWP is configured:

4>	perform BWP switching to a BWP indicated by the default-DL-BWP.

3>	else:

4>	perform BWP switching to the initial DL BWP.

End of change