3GPP TSG-RAN WG2 #102	R2-1807040

Busan, S. Korea, 21st – 25th May 2018	(Resubmission of R2-1804797)

Agenda Item:	10.4.2.4

Source:	Ericsson

Title:	TP to 38.331 for inter-RAT mobility between NR and E-UTRA

Document for:	Discussion, Decision

# Introduction

During RAN2#101, the following agreements were made related to inter-RAT mobility:

Agreements for inter-RAT HO from E-UTRA to NR

1	Modify the legacy MobilityFromEUTRACommand to allow transfer of NR RRC message e.g. RRCReconfiguration to enable inter-RAT handover to NR (applicable to LTE/EPC and LTE/5GC)

2	Inter-system, inter-RAT handover from E-UTRA/EPC to NR will require a full configuration to reset the radio bearers.

3	In order to keep the SDAP and PDCP configurations during intra-system, inter-RAT handover from E-UTRA/5GC to NR, delta-configuration for the radio bearer configuration can be used.

4	Align the handling of PDCP/SDAP and security parameters in all intra-5GC handover scenarios (i.e. intra-NR, intra-E-UTRA, and inter-RAT where both source and target node use 5GC).

Agreements

1	MobilityFromNRCommand allows transfer of LTE RRC message e.g. RRCConnectionReconfiguration to enable inter-RAT handover to LTE (applicable to handover to LTE/EPC and LTE/5GC)

2	For inter-system inter-RAT handover from NR to E-UTRA/EPC, the UE is configured via the RRCConnectionReconfiguration message (full configuration) carried by NR, as in legacy inter-RAT handover procedure.

3	In order to keep the SDAP and PDCP configurations during intra-system, inter-RAT handover from NR to E-UTRA/5GC, delta-configuration for the radio bearer configuration can be used.

4	Security procedures for inter-system handover to E-UTRA/EPC should follow legacy inter-RAT handover procedures i.e. be configured with 36.331 SecurityConfigHO.

This contribution contains a text proposal to TS 38.331 regarding inter-system and intra-system, inter-RAT mobility procedures. The corresponding changes to LTE RRC TS 36.331 can be found in [1]. The required changes to the security related aspects of inter-RAT handover are further discussed in [2].

# Text proposal

START OF FIRST CHANGE

## 5.4	Inter-RAT mobility

#### 5.4.1	Introduction

NR support network controlled inter-RAT handover to/from NR from/to E-UTRA connected to EPC or 5GC.

#### 5.4.2	Handover to NR

##### 5.4.2.1	General

Figure 5.4.2.1-1: Handover to NR, successful

The purpose of this procedure is to, under the control of the network, transfer a connection between the UE and another Radio Access Network (e.g. E-UTRAN) to NR.

The handover to NR procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from E-UTRA to NR applies only after integrity has been activated in E-UTRA.

##### 5.4.2.2	Initiation

The RAN using another RAT initiates the handover to NR procedure, in accordance with the specifications applicable for the other RAT, by sending the RRCReconfiguration message via the radio access technology from which the inter-RAT handover is performed.

The network applies the procedure as follows:

-	to activate ciphering, possibly using NULL algorithm, if not yet activated in the other RAT;

-	to re-establish SRBs and one or more DRBs;

##### 5.4.2.3	Reception of the RRCReconfiguration by the UE

The UE shall:

1>	if the received RRCReconfiguration includes the securityConfig:

2>	perform key update procedure as specified in 5.3.5.x;

Editor’s Note: The section 5.3.5.x (‘Security key update’) need to be updated.

- perform the radio configuration procedure as specified in 5.3.5.

##### 5.4.2.4	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message:

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in x.x specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.

#### 5.4.3	Mobility from NR

##### 5.4.3.1	General

Figure 5.4.3.1-1: Mobility from NR, successful

Figure 5.4.3.1-2: Mobility from NR, failure

The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using E-UTRA Radio Access Technology (RAT). The mobility from NR procedure covers the following type of mobility:

-	handover, i.e. the MobilityFromNRCommand message includes radio resources that have been allocated for the UE in the target cell;

##### 5.4.3.2	Initiation

The network initiates the mobility from NR procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport message, by sending a MobilityFromNRCommand message. The network applies the procedure as follows:

-	the procedure is initiated only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended;

##### 5.4.3.3	Reception of the MobilityFromNRCommand by the UE

The UE shall be able to receive a MobilityFromNRCommand message, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	consider inter-RAT mobility as initiated towards RAT indicated by the targetRAT-Type included in the MobilityFromNRCommand message;

1>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

##### 5.4.3.4	Successful completion of the mobility from NR

Upon successfully completing the handover the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in x.x.x.x, with release cause 'other';

Editor’s Note: It is FFS whether other release cause than ‘other’ will be applicable.

##### 5.4.3.5	Mobility from NR failure

The UE shall:

1>	if the UE does not succeed in establishing the connection to the target radio access technology; or

1>	if the UE is unable to comply with (part of) the configuration included in the MobilityFromNRCommand message; or

1>	if there is a protocol error in the inter RAT information included in the MobilityFromNRCommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT:

2>	revert back to the configuration used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in x.x.x;

END OF FIRST CHANGE

START OF SECOND CHANGE

#### –	MobilityFromNRCommand

The MobilityFromNRCommand message is used to command handover from NR to E-UTRA (connected to EPC or 5GC).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

MobilityFromNRCommand message

-- ASN1START

MobilityFromNRCommand ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

mobilityFromNRCommand				MobilityFromNRCommand-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

MobilityFromNRCommand-IEs ::=	SEQUENCE {

targetRAT-Type						ENUMERATED {

eutra,	spare3, spare2, spare1, ...},

targetRAT-MessageContainer			OCTET STRING

lateNonCriticalExtension				OCTET STRING								OPTIONAL,

nonCriticalExtension					SEQUENCE {}									OPTIONAL

}

-- ASN1STOP

MobilityFromNRCommand field descriptions

targetRAT-Type Indicates the target RAT type.

targetRAT-MessageContainer The field contains a message specified in another standard, as indicated by the targetRAT-Type, and carries information about the target cell identifier(s) and radio parameters relevant for the target radio access technology. NOTE 1. A complete message is included, as specified in the other standard.

NOTE 1:	The correspondence between the value of the targetRAT-Type, the standard to apply and the message contained within the targetRAT-MessageContainer is shown in the table below:

| targetRAT-Type | Standard to apply | targetRAT-MessageContainer |
| --- | --- | --- |
| eutra | 3GPP TS 36.331 (clause 5.4.2) | RRCCONNECTIONreconfiguration |

# References

R2-1807039, TP to 36.331 for inter-RAT mobility between NR and E-UTRA, Ericsson, RAN2#102, Busan, S. Korea, 21-25 May, 2018

R2-1807043, Security key update for NR SA and MR-DC, Ericsson, RAN2#102, Busan, S. Korea, 21-25 May, 2018