3GPP TSG-RAN WG2 #102	R2-1807194

# Busan, Republic of Korea, 21st – 25th May 2018

Agenda Item:	10.3.3.3

Source:	Ericsson

Title:	PDCP duplication

Document for:	Discussion, Decision

# Introduction

In RAN2#101 and #101bis, PDCP duplication was discussed and the following agreement were reached related to submission time of duplicated PDCP PDUs:

Agreements

=>	The duplication is done at the time of submission.  When to submit the PDCP PDU is up to UE implementation.  The PDCP modelling assumes that the submission to different RLC entity is done at the same time and a single PDCP data volume

In this contribution we continue the discussion on the remaining issue of submission time of duplicate PDUs.

# Discussion

## Submission time of duplicate PDUs

In the current Rel-15 NR TS 38.323 15.1.0, the PDCP duplication function is captured in the transmitter side as follows:

[…]

-	if pdcp-Duplication is configured and activated:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities;

-	else, if pdcp-Duplication is configured but not activated:

-	submit the PDCP Data PDU to the primary RLC entity;

[…]

This way, for each PDCP SDU, the corresponding PDCP Data PDU is duplicated and provided to each RLC entity – irrespective of transmission opportunities and their ratio on the transmission legs. For the split bearer operation, the following NOTE was added:

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity.

indicating that submission of PDCP PDUs to lower layers before transmission opportunity is indicated should be kept minimal by UE implementations. This way, low skew times below the UL transmission legs are ensured, since data transmitted to lower layers is more according to the UL grant ratio. This way, for split operation, eventually lower reordering times in the receiving PDCP entity are ensured.

For duplication operation, similar considerations must be done: instead of considering minimizing the reordering time among the transmission legs, the reception time differences of the duplicates must be considered, since PDCP data duplication can only be considered useful for e.g. URLLC traffic if the reception time differences are not too high, i.e. the duplicate is not already outdated when received too late after the original.

Furthermore, for PDCP data duplication, pushing down all the data immediately to both RLCs may lead to this data being stuck in an RLC for which no grant will arrive (for some time). It is noted that at HARQ and RLC layer, potential retransmission among the duplicates are not synchronized. This will lead to the further risk of duplicate transmissions drifting apart, due to different amounts of granted resources or retransmissions on the links.

In RAN2 AH#1801, it had been agreed that for packet duplication, when to submit PDCP PDUs to lower layers is up to UE implementation. Furthermore, in RAN2#101 it was agreed that for sake of simpler PDCP modelling (for the data volume calculation), it is assumed for PDCP modelling that submission of the duplicates happens at the same time. Nevertheless, the submission time is agreed to be up to UE implementation.

Same as for split bearer operation we thus need to specify a NOTE limiting potentially bad UE behaviour associated with this, as described above. Thus, we propose:

- In duplication, the UE transmitter minimizes the amount of PDCP PDUs submitted to lower layers before receiving a request from lower layers to minimize the reception time difference of the duplicates in the receiving PDCP entity.

A corresponding text proposal can be found below.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	In duplication, the UE transmitter minimizes the amount of PDCP PDUs submitted to lower layers before receiving a request from lower layers to minimize the reception time difference of the duplicates in the receiving PDCP entity.

# Text proposal

For TS 38.323, v15.1.0, Section 5.2.1 Transit operation:

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity, or if pdcp-Duplication is configured to minimize the PDCP PDU reception time difference of the duplicates in the receiving PDCP entity.