# 3GPP TSG-RAN WG2 Meeting #102 	R2-1807294

# Busan, South Korea, May 21-25, 2018	revision of R2-1806040

Agenda item:	9.14.2

Source: 	Qualcomm Incorporated

Title: 	ASN.1 for PRACH partitioning

Document for:     Discussion/Decision

# Introduction

The WI description for even further enhanced MTC for LTE is given in [1]. Current RRC running CR is given in [2].

One of the remaining items is ASN.1 for PRACH partitioning for eMTC. Post-meeting email discussion [101#59] tried to capture the ASN.1 for PRACH resource partitioning based on contributions and discussion from RAN2#101. There were some unaddressed comments.

Compared to previous version, the NB-IoT part is removed in this revision of the Tdoc as agreements were reached in RAN2#101bis.

# Discussion

For eMTC, PRACH partitioning involves at least two aspects:

- PRACH time-frequency resources partitioning

- PRACH preamble partitioning

In addition, the above is applicable for per CE level.

## PRACH time-frequency resources partitioning for EDT

This is already captured in the running CR with changes in PRACH-Config.

## PRACH preamble partitioning for EDT

Currently, within a PRACH resource (configured by PRACH-Config), the partitioning is

- between group A and B, or

- when different CE levels are configured, between different CE levels (i.e., no group B)

For EDT, we need to add groups for EDT within each CE level. The following NOTE in TS 36.321 subclause 5.1.1 should be applicable for EDT PRACH partitioning also:

NOTE: When a PRACH resource is shared for multiple enhanced coverage levels, and enhanced coverage levels are differentiated by different preamble indices, Group A and Group B is not used for this PRACH resource.

The following NOTE from TS 36.321 subclause 5.1.1 is applicable for EDT PRACH partitioning: “When a PRACH resource is shared for multiple enhanced coverage levels, and enhanced coverage levels are differentiated by different preamble indices, Group A and Group B is not used for this PRACH resource.”

Following modification (red text) are currently captured in the RRC running CR [2]. As the changes were not fully agreed, an Editor’s Note was also captured.

RACH-ConfigCommon ::=		SEQUENCE {

<<skip>>

...,

[[	preambleTransMax-CE-r13			PreambleTransMax					OPTIONAL,	-- Need OR

rach-CE-LevelInfoList-r13		RACH-CE-LevelInfoList-r13			OPTIONAL	-- Need OR

]]

}

<<skip>>

RACH-CE-LevelInfoList-r13 ::=	SEQUENCE (SIZE (1..maxCE-Level-r13)) OF RACH-CE-LevelInfo-r13

RACH-CE-LevelInfo-r13 ::=		SEQUENCE {

preambleMappingInfo-r13				SEQUENCE {

firstPreamble-r13					INTEGER(0..63),

lastPreamble-r13					INTEGER(0..63)

},

ra-ResponseWindowSize-r13			ENUMERATED {sf20, sf50, sf80, sf120, sf180,

sf240, sf320, sf400},

mac-ContentionResolutionTimer-r13	ENUMERATED {sf80, sf100, sf120,

sf160, sf200, sf240, sf480, sf960},

rar-HoppingConfig-r13				ENUMERATED {on,off},

...,

[[	edt-parameters-r15			SEQUENCE {

edt-TBS-r15					ENUMERATED {b328, b408, b504, b600, b712,

b808, b936, b1000or456},

edt-lastPreamble-r15		INTEGER(0..63),

mac-ContentionResolutionTimer-r15	ENUMERATED {sfVal1, sfVal2,

sfVal3, TBDNumAndValues}

}	OPTIONAL		-- Cond EDT

]]

}

<<skip>>

Editor’s Note: ASN.1 for preamble partitioning (edt-lastPreamble-r15) is a placeholder showing one possible way. Final agreements on how to capture in ASN.1 is FFS.

Editor’s Note: FFS: the candidate values and number of values for mac-ContentionResolutionTimer-r15.

| RACH-ConfigCommon field descriptions |  |
| --- | --- |
| edt-lastPreamble Provides the mapping of preambles to groups for each CE level for EDT, as specified in TS 36.321 [6]. The preambles for EDT are the preambles lastPreamble-r13 +1 to edt-lastPreamble-r15. E-UTRAN configures this field only when random access preambles group B is not used. |  |
| edt-TBS Largest TBS for Msg3 for a CE level applicable to a UE performing EDT. Value in bits. Value b328 corresponds to 328 bits, b408 corresponds to 408 bits and so on. Additionally, value b1000or456 corresponds to 1000 bits for CE levels 0 and 1, and 456 bits for CE levels 2 and 3. See TS 36.213 [23]. |
| preambleMappingInfo Provides the mapping of preambles to groups for each CE level, except for EDT, as specified in TS 36.321 [6]. When random access preambles group B is used, firstPreamble-r13 is set to 0 and lastPreamble-r13 is set to numberOfRA-Preambles-1. |

In our view, instead of signalling both first preamble and last preamble for EDT preamble pool, it is sufficient to signal edt-lastPreamble considering first preamble for EDT to be immediately next preamble to the non-EDT last preamble for the CE level.

Confirm that current changes in RACH-ConfigCommon for preamble partitioning from RRC running CR is adopted. Remove corresponding Editor’s note.

# Conclusion

In this contribution, we discussed on some FFSes from previous agreements on PRACH partitioning for EDT. Based on the above discussion, we made the following proposals:

Proposal 1.	The following NOTE from TS 36.321 subclause 5.1.1 is applicable for EDT PRACH partitioning: “When a PRACH resource is shared for multiple enhanced coverage levels, and enhanced coverage levels are differentiated by different preamble indices, Group A and Group B is not used for this PRACH resource.”

Proposal 2.	Confirm that current changes in RACH-ConfigCommon for preamble partitioning from RRC running CR is adopted. Remove corresponding Editor’s note.

# References

- RP-170732, New WID on Even further enhanced MTC for LTE, Ericsson, Qualcomm

- R2-1807276, Introduction of EDT for eMTC and NB-IoT enhancements, Huawei, HiSilicon, Qualcomm