3GPP TSG-RAN WG2 #102	Tdoc R2-1807921

Busan, Korea, 21st – 25th May 2018

Agenda Item:	10.4.1.3.6

Source:	Ericsson

Title:	Timer for RRC Resume Request (T300X)

Document for:	Discussion

# Introduction

In RAN2 #101 in Athens, it has been agreed to use a protection timer (like T300 in LTE [1]) to handle the case of failure during the resume procedure.

Agreements

3a	Resume procedure is protected by a timer (similar to T300 for connection establishment procedure). UE consider resume failure upon timer expiry.

This contribution explains the assumptions concerning these timers as captured in the latest version of the TP on Connection Control [2]. A companion paper for RRC Setup Request (concerning timer T300) has been provided in [3].

# Discussion

Starting criteria

In a first email discussion to draft the initial version of the RRC connection control messages and procedure [2], T300X timer was defined for connection resume. In the updated version of TP discussed after the RAN2#101 meeting that was named timer T319. Similar to LTE, the TP on Connection Control [2] assumes that T319 is started upon the transmission of RRCResumeRequest.

T319 is started upon the transmission of an RRCResumeRequest.

Stopping criteria (successful case)

Upon transmitting an RRCResumeRequest and starting a timer T319 (previously called T300X) [2], the UE may receive an RRCSetup (successful case), or an RRCReject (overload protection); or an RRC Release with Suspend configuration (indicating that the UE should remain in RRC_INACTIVE); or an RRC Release (indicating that the UE should transit from RRC_INACTIVE to RRC_IDLE). The reception of any of these is an indication of a successful reception at the network of the resume request, hence timer T319 is stopped.

T319 is stopped at least upon the reception of an RRCSetup, or RRCReject, or RRCRelease to idle or RRCRelease with suspend configuration to inactive.

Cell reselection while timer is running

As in LTE, it is assumed in [2] that the UE continues to perform cell reselection after the transmission of an RRCResumeRequest. That is because the UE according to 36.331 and 38.331 the UE remains in RRC_INACTIVE until it receives an RRCResume. That is beneficial as network-controlled mobility has not started yet and, as UE may find a better cell while trying to resume or setup a connection, remaining in a much worse cell could lead to a failure that could anyway lead to a cell reselection.

Hence, after transmitting an RRCResumeRequest, cell reselection can occur. In LTE, cell reselection while the equivalent timer is running is considered as a failure case and, the defined UE behaviour is that UE notifies the higher layers and performs NAS recovery. In RAN2#101bis, the following has been agreed as a working assumption:

Working assumption

1	If cell re-selection occurs during T300X is running then the UE initiates resume procedure in the new cell. This implies that T300X (from new cell) is started when the procedure is re initiated.

In a companion paper we analyse the implications in terms of performance and security concerning the abovementioned working assumption [4]. However, regardless if that working assumption is confirmed or not, the timer T319 (T300X in the working assumption) should also be stopped upon cell reselection, regardless of subsequent actions (e.g. possibly a re-start of T300X).

T319 is also stopped upon cell reselection if running.

Actions upon expiry of T319

The expiry of T319 is an indication that the UE has transmitted the request message but either network has not been able to receive it, or the UE has not been able to detect the network response, which could occur due to a coverage problem. In LTE, it is defined that the UE performs NAS recovery, both for Resume or Establishment. As in the case of the expiry of T300 [4], for T319 it also makes sense to perform NAS recovery upon the expiry of T319, as captured in [2].

Upon the expiry of T319 or cell reselection while T319 is running, the UE performs Resume again.

All the described assumptions have been captured in the latest version of the TP on Connection Control [3].

Confirm the assumptions of T319 as in the TP on Connection Control.

# Conclusion

Based on the discussion in section 2 we propose the following:

- Confirm the assumptions of T319 as in the TP on Connection Control.

# References

3GPP TS 36.331, Radio Resource Control (RRC), V15.0.1 (2018-01)

R2-1805355, CR on Connection Control, Ericsson, 3GPP TSG-RAN WG2 #101bis, Sanya, P.R. of China, 16th – 20th April 2018

R2-1807915, Timer for RRC Setup establishment procedure (T300), Ericsson, 3GPP TSG-RAN WG2 #102, Busan, Korea, 21st – 25th May 2018.

R2-1807920, Actions upon the expiry of T300X or cell reselection while T300X is running, Ericsson, 3GPP TSG-RAN WG2 #102, Busan, Korea, 21st – 25th May 2018.