3GPP TSG-RAN WG2 #102	Tdoc R2-1807930

Busan, Korea, 21st – 25th May 2018

Agenda Item:	10.4.1.3.9 (Connection Control – Other)

Source:	Ericsson

Title:	Conditional presence of MCC in PLMN-Identity

Document for:	Discussion, Decision

# Introduction

This contribution addresses the following FFS in the TP on Connection Control [1]

#### PLMN-Identity

The IE PLMN-Identity identifies a Public Land Mobile Network. Further information regarding how to set the IE is specified in TS 23.003 [20].

PLMN-Identityinformation element

-- ASN1START

PLMN-Identity ::=					SEQUENCE {

mcc									MCC					OPTIONAL,					-- Cond MCC

mnc									MNC

}

MCC ::=								SEQUENCE (SIZE (3)) OF

MCC-MNC-Digit

MNC ::=								SEQUENCE (SIZE (2..3)) OF

MCC-MNC-Digit

MCC-MNC-Digit ::=					INTEGER (0..9)

-- ASN1STOP

PLMN-Identity field descriptions

mcc The first element contains the first MCC digit, the second element the second MCC digit and so on. If the field is absent, it takes the same value as the mcc of the immediately preceding IE PLMN-Identity. See TS 23.003 [20].

mnc The first element contains the first MNC digit, the second element the second MNC digit and so on. See TS 23.003 [20].

Editor’s Note: FFS Whether there is a conditional presence in the case of MCC e.g. whne PLMN identity is included in the Cell Global Id NR.

# Discussion

In 36.331, there is a conditional inclusion of an MCC (Mobile Country Code) field in the PLMN identity field.

The conditions for including MCC is described as:

| Conditional presence | Explanation |
| --- | --- |
| MCC | This IE is mandatory when PLMN-Identity is included in CellGlobalIdEUTRA, in CellGlobalIdUTRA, in CellGlobalIdGERAN or in RegisteredMME. This IE is also mandatory in the first occurrence of the IE PLMN-Identity within the IE PLMN-IdentityList. Otherwise it is optional, need OP. |

Dependent on when and where the plmn-Identity IE is included it may or may not be necessary to include MCC. A commonly deployed principle is to try to avoid repetitions. Thus, the MCC can be optionally included with a few exceptions.

Similarly as for LTE, it is necessary to include the full PLMN identifiers in the first occurrence of the IE plmn-Identity within the IE PLMN-IdentityList.

Inherit from LTE that MCC is mandatory in the first occurrence of the IE plmn-Identity within the IE PLMN-IdentityList.

Continuing, a cell global identifier, for it to be globally unique, needs to include the full PLMN. Thus, it is, also at least formally required to include the full PLMN, i.e., also the MCC. It can be discussed if, e.g., lists of global cell ID’s could also deploy the same principle that MCC is only included in the first entry. The principles in LTE however seem to suggest that the global Cell ID’s include all fields. In cases there are lists of cells of the same plmn, then the term CellGlobalIdxxx should not be used. Instead the cellIdentity fields should be used and grouped together for the same PLMN. We believe that the principles from LTE shall apply also for NR.

Inherit from LTE that MCC is mandatory in IE plmn-Identity when part of cell global ID in NR. If many cells of same PLMN, use cellIdentity fields grouped per PLMN.

The plmn-Identity is also sometimes included in RegisteredAMF (when different than selected PLMN), which corresponds to the RegisteredMME for LTE above. The same principle for RegisteredAMF must apply as for RegisteredMME.

Inherit from LTE that MCC is mandatory in IE plmn-Identity when part of RegisteredAMF.

To summarize, it is proposed to include, in connection to the PLMN-Identity definition in 38.331:

| Conditional presence | Explanation |
| --- | --- |
| MCC | This IE is mandatory when PLMN-Identity is included in global cell IDs in NR [FFS - exact name] and CellGlobalIdEUTRA. MCC is also mandatory in PLMN-Identity if included in RegisteredAMF. The IE is also mandatory in the first occurrence of the IE PLMN-Identity within the IE PLMN-IdentityList. Otherwise it is optional, need OP. |

Include the conditional presence explanation above in 38.331 definition of plmn-Identity.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Inherit from LTE that MCC is mandatory in the first occurrence of the IE plmn-Identity within the IE PLMN-IdentityList.

Proposal 2	Inherit from LTE that MCC is mandatory in IE plmn-Identity when part of cell global ID in NR. If many cells of same PLMN, use cellIdentity fields grouped per PLMN.

Proposal 3	Inherit from LTE that MCC is mandatory in IE plmn-Identity when part of RegisteredAMF.

Proposal 4	Include the conditional presence explanation above in 38.331 definition of plmn-Identity.

# References

R2-1805355, CR on Connection Control, Ericsson, Ericsson, 3GPP TSG-RAN WG2 #101bis, Sanya, P.R. of China, 16th – 20th April 2018