3GPP TSG-RAN WG2#101bis	R2-1808110

Busan, Korea, 21st – 25th May 2018

Agenda Item:	9.8.2

Source:	Ericsson

Title:	Transfer of unicast reference station ambiguity level

Document for:	Discussion, Decision

# Introduction

At the RAN2#101bis meeting, some aspects of ensuring compact messages for broadcast of GNSS RTK assistance data were discussed. One example is the use of the MAC representation of observations when broadcasting assistance data associated to multiple physical or non-physical reference stations, since it is more compact compared to using separate information elements for each reference station observation.

The main reason for broadcasting observations from multiple reference stations is to allow the device to move in the area covered by the multiple reference stations.

In unicast, the device may need observations from multiple reference stations to ensure a maintained service quality despite mobility. In this paper, we discuss the unicast challenge in more detail, and show that much of the overhead can be avoided by some extra information at the time instant when the device desires to switch from one reference station to the next.

# Multiple Reference Station Observations to Mobile UEs

For mobile UEs, served with RTK corrections via LPP, it is attractive to adapt the scope of the observations based on the crude UE position, for example to only provide observations from the most relevant physical or non-physical reference station. However, at some point, a different reference station from the current becomes most relevant. With an ambition to minimize the information to the UE, the location server could simply swap the reference station from one to another. The would probably mean that the UE needs to restart the integer ambiguity resolution, resulting in a fall back to GNSS positioning based on code phase measurements in the meantime with much worse positioning accuracy.

An alternative would be to send observations from several physical or non-physical reference stations in parallel to the UE, and the UE could thereby initiate the integer ambiguity resolution in advance for alternative reference stations. This would require additional processing in the device, and also a multiplication of the needed bandwidth to provide the GNSS RTK observations via LPP.

Observation 1	Mobile UEs can be supported by providing GNSS RTK observations from multiple reference stations at the expense of additional processing in the device and a multiplication of the needed LPP signalling costs.

Assume that the reference stations are placed on a uniform grid as illustrated by Figure 1, where the most favourable reference station is marked with a circle. To ensure that any kind of mobility patterns can be supported, observations from at least the four closest reference stations (marked by the dashed line) should be provided to the device.

Figure 1. Device in a region with a grid of reference stations, the closest marked by a circle, and the closest four reference stations marked by a dashed line.

Observation 2	With a grid of reference stations, the device could be provided with observations from at least the closest four reference stations in order to support mobility

An alternative is to provide some additional information to UE at the instants when the location server swaps the reference station, which the observation stems from. As it turns out, the needed information can be provided by utilizing the existing signalling for MAC in combination with the observations from physical or non-physical reference stations.

From Navpedia [3] we have the following generic description of the phase measurement, the integer ambiguity N and the error contributions:

Where:

is the signal path delay due to the ionosphere;

is the signal path delay due to the troposphere;

is the receiver clock offset from the reference (GPS) time;

is the satellite clock offset from the reference (GPS) time;

is the vacuum speed of light;

is the carrier nominal wavelength;

is the ambiguity of the carrier-phase (integer number);

are the measurement noise components, including multipath and other effects;

is the geometrical range between the satellite and the receiver, computed as a function of the satellite  and receiver  coordinates as:

.

Receivers then form the double difference equation

For two receivers a and b making simultaneous measurements at the same nominal time to satellites 1 and 2, the double difference observable is:

Where

∅a12=∅a1-∅a2 	(4)

The integer ambiguity solution solved for Nab12 etc, where
Nab12=Na1-Na2-(Nb1-Nb2)	(5)

For a UE denoted r (rover), and a current reference station c, the UE maintains the integer solutions Nrcij for the two satellites i and j. Eventually, when the UE has transferred to a new reference station n, the UE needs to solve for Nrnij instead. This will require some initialization time etc.

Fortunately, given (5), there is a relation between Nrcij and Nrcij that can be exploited:

Nrnij=Nrcij+Ncnij=Nrcij+Nci-Ncj-Nni-Nnj	(6)

The information the UE needs to translate from  Nrcij to Nrnij is readily available as the GNSS RTK observations at from the current and the new reference station, for pairs of satellites, or essentially all satellites but one in relation to one reference satellite.

Proposal 1	Define signalling to enable the UE to translate its integer ambiguity solution associated to a current reference station to an integer solution associated to a new reference station

The double difference can be formed by the difference of data (integer-ms + rough-range + fine-PhaseRange, where each term is added with the correct scale factor) for a satellite j and a reference satellite i, for on the one hand the current reference station, and one the other hand the new reference station, and then the difference between these two differences. The integer part of the result is the Nrnij. In a well-maintained reference network, this integer is within +/- 5, but it can be good to have some margin for less maintained networks.

A draft text proposal with a suggested representation of the needed information can be found in Appendix A.

Proposal 2	Introduce the text proposal in Appendix A into the running LPP CR

# Conclusion

Based on the discussion in section 2 we propose the following

Proposal 1	Define signalling to enable the UE to translate its integer ambiguity solution associated to a current reference station to an integer solution associated to a new reference station

Proposal 2	Introduce the text proposal in Appendix A into the running LPP CR

# References

R2-1802710 Additional Stage-2 Considerations for RTK Assistance Data, Deutsche Telecom

R2-1802690 Running LPP CR for RTK GNSS positioning, Qualcomm

http://www.navipedia.net/index.php/RTK_Fundamentals

# Appendix A Text Proposal for TS 36.355

### 6.5.2	A-GNSS Positioning

#### 6.5.2.1	GNSS Assistance Data

#### […]

- GNSS-GenericAssistData

The IE GNSS-GenericAssistData is used by the location server to provide assistance data for a specific GNSS (e.g., GPS, Galileo, GLONASS, BDS, etc.). The specific GNSS for which the provided assistance data are applicable is indicated by the IE GNSS-ID and (if applicable) by the IE SBAS-ID. Assistance for up to 16 GNSSs can be provided.

-- ASN1START

# GNSS-GenericAssistData ::= SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataElement

# GNSS-GenericAssistDataElement ::= SEQUENCE {

gnss-ID 						GNSS-ID,

sbas-ID							SBAS-ID 						OPTIONAL, -- Cond GNSS-ID-SBAS

gnss-TimeModels					GNSS-TimeModelList				OPTIONAL,	-- Need ON

gnss-DifferentialCorrections	GNSS-DifferentialCorrections	OPTIONAL,	-- Need ON

gnss-NavigationModel			GNSS-NavigationModel			OPTIONAL,	-- Need ON

gnss-RealTimeIntegrity			GNSS-RealTimeIntegrity			OPTIONAL,	-- Need ON

gnss-DataBitAssistance			GNSS-DataBitAssistance			OPTIONAL,	-- Need ON

gnss-AcquisitionAssistance		GNSS-AcquisitionAssistance 		OPTIONAL,	-- Need ON

gnss-Almanac					GNSS-Almanac					OPTIONAL,	-- Need ON

gnss-UTC-Model					GNSS-UTC-Model					OPTIONAL,	-- Need ON

gnss-AuxiliaryInformation		GNSS-AuxiliaryInformation		OPTIONAL,	-- Need ON

...,

[[

bds-DifferentialCorrections-r12

BDS-DifferentialCorrections-r12	OPTIONAL,	-- Cond	GNSS-ID-BDS

bds-GridModel-r12			BDS-GridModelParameter-r12		OPTIONAL	-- Cond	GNSS-ID-BDS

]],

[[

gnss-RTK-Observations-r15	GNSS-RTK-Observations-r15		OPTIONAL,	-- Need ON

gnss-RTK-RStransferInfo-r15	GNSS-RTK-RStransferInfo-r15		OPTIONAL,	-- Cond RTK-RST

glo-RTK-BiasInformation-r15	GLO-RTK-BiasInformation-r15		OPTIONAL,	-- Cond GNSS-ID-GLO

gnss-RTK-MAC-CorrectionDifferences-r15

GNSS-RTK-MAC-CorrectionDifferences-r15

OPTIONAL,	-- Need ON

gnss-RTK-Residuals-r15		GNSS-RTK-Residuals-r15			OPTIONAL,	-- Need ON

gnss-RTK-FKP-Gradients-r15	GNSS-RTK-FKP-Gradients-r15		OPTIONAL,	-- Need ON

gnss-SSR-OrbitCorrections-r15

GNSS-SSR-OrbitCorrections-r15	OPTIONAL,	-- Need ON

gnss-SSR-ClockCorrections-r15

GNSS-SSR-ClockCorrections-r15	OPTIONAL,	-- Need ON

gnss-SSR-CodeBias-r15		GNSS-SSR-CodeBias-r15			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| GNSS-ID-BDS | The field may be present if the GNSS-ID = bds; otherwise it is not present. |
| GNSS-ID-GLO | The field may be present if the GNSS ID = glonass; otherwise it is not present. |
| RTK-RST | The field may be present if gnss-RTK-Observations is present, otherwise it is not present |

#### [..]

#### 6.5.2.2	GNSS Assistance Data Elements

#### […]

- GNSS-RTK-Observations

The IE GNSS-RTK-Observations is used by the location server to provide GNSS reference station observables (pseudorange, phaserange, phaserange-rate (Doppler), and carrier-to-noise ratio) of the GNSS signals. Essentially, these parameters describe the range and derivatives from respective satellites to the reference station location provided in IE GNSS-RTK-ReferenceStationInfo at the reference time GNSS-SystemTime provided in IE GNSS-ReferenceTime. Whenever GNSS-RTK-Observations is provided by the location server, the IE GNSS-ReferenceTime shall be provided as well.

The parameters provided in IE GNSS-RTK-Observations are used as specified for message type 1071-1127 in [30].

-- ASN1START

GNSS-RTK-Observations-r15 ::= SEQUENCE (SIZE(1..64)) OF GNSS-RTK-SatelliteDataElement-r15

GNSS-RTK-SatelliteDataElement-r15 ::= SEQUENCE{

svID-r15								SV-ID,

integer-ms-r15							INTEGER (0..254),

rough-range-r15							INTEGER (0..1023),

rough-phase-range-rate-r15				INTEGER (-8192..8191)					OPTIONAL,

gnss-rtk-SatelliteSignalDataList-r15	GNSS-RTK-SatelliteSignalDataList-r15,

...

}

GNSS-RTK-SatelliteSignalDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-SatelliteSignalDataElement-r15

GNSS-RTK-SatelliteSignalDataElement-r15 ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

fine-PseudoRange-r15				INTEGER (-524288..524287),

fine-PhaseRange-r15					INTEGER (-8388608..8388607),

lockTimeIndicator-r15				INTEGER (0..1023),

halfCycleAmbiguityIndicator-r15		BIT STRING (SIZE (1)),

carrier-to-noise-ratio-r15			INTEGER (0..1023)						OPTIONAL,

fine-PhaseRangeRate-r15				INTEGER (-16384..16383)					OPTIONAL,

...

}

-- ASN1STOP

GNSS-RTK-Observations field descriptions

svID This field specifies the GNSS SV-ID of the satellite for which the GNSS Observations are provided.

integer-ms This field contains the integer number of milliseconds in the satellite rough range. Rough range can be used to restore complete observables for a given satellite. Scale factor 1 milli-second in the range from 0 to 254 milli-seconds.

rough-range This field contains the sub-milliseconds in the satellite rough range (modulo 1 millisecond). Scale factor 2-10 milli-seconds in the range from 0 to (1-2-10) milli-seconds.

rough-phase-range-rate This field contains the GNSS satellite rough phaserange rate. Scale factor 1 m/s. Range ±8191 m/s.

gnss-SignalID This field specifies the GNSS signal for which the GNSS observations are provided.

fine-PseudoRange This field contains the GNSS signal fine pseudorange. Being added to fields integer-ms and rough-range allows getting the full pseudorange observable corresponding to given signal. NOTE 1. Scale factor 2–29 milli-seconds. Range ±(2–10–2–29) milli-seconds.

fine-PhaseRange This field contains the GNSS signal fine phaserange. Being added to fields integer-ms and rough-range allows getting the full phaserange observable corresponding to given signal. NOTE 2. Scale factor 2–31 milli-seconds. Range ±(2–8–2–31) milli-seconds.

lockTimeIndicator This field provides a measure of the amount of time during which the receiver has maintained continuous lock on that satellite signal. If a cycle slip occurs during the previous measurement cycle, the lock time indicator shall be reset to zero. Mapping according to the table lockTimeIndicator value to interpretation lock-time relation shown below.

halfCycleAmbiguityIndicator Value 0 indicates no half-cycle ambiguity. Value 1 indicates half-cycle ambiguity. When providing phaserange with unresolved polarity encoding this bit shall be set to 1. A target device that is not capable of handling half-cycle ambiguities shall skip such phaserange observables. If polarity resolution forced phaserange to be corrected by half-a-cycle, then the lockTimeIndicator must be reset to zero, indicating that despite continuous tracking the final phaserange experienced non-continuity.

carrier-to-noise-ratio This field provides the GNSS signal carrier-to-noise-ratio in dB-Hz. Scale factor 2–4 dB-Hz in the range from 0.0625 to 63.9375 dB-Hz.

fine-PhaseRangeRate This field contains the GNSS signal fine Phase Range Rate. Full phaserange rate is the sum of this field and the rough-phase-range-rate field. NOTE 3. Scale factor 0.0001 m/s. Range ±1.6383 m/s.

NOTE 1:	Complete Pseudorange for each signal (i) of given satellite can be restored as follows: 
Pseudorange(i) = c/1000 × (integer-ms + rough_range/1024 + 2–29 × fine_Pseudorange(i)), meter.

NOTE 2: 	Complete Phaserange for each signal (i) of given satellite can be restored as follows:
Phaserange(i) = c/1000 × (integer-ms + rough_range/1024 + 2–31 × fine_Phaserange(i)), meter.

NOTE 3: 	Complete PhaseRangeRate for each signal (i) of given satellite can be restored as follows:
PhaseRangeRate(i) = rough-phase-range-rate + 0.0001*fine-PhaseRangeRate (i), meter/sec.

NOTE 4:	The speed of light c is 299,792,458 meters per second.

lockTimeIndicator value to interpretation lock-time relation

| Indicator (i) | Supplementary coefficient (k) | Minimum Lock Time (ms) | Range of Indicated Lock Times (t) (ms) |
| --- | --- | --- | --- |
| 0 – 63 | 1 | i | 0 ≤ t < 64 |
| 64 – 95 | 2 | 2 × i – 64 | 64 ≤ t < 128 |
| 96 – 127 | 4 | 4 × i – 256 | 128 ≤ t < 256 |
| 128 – 159 | 8 | 8 × i – 768 | 256 ≤ t < 512 |
| 160 – 191 | 16 | 16 × i – 2048 | 512 ≤ t < 1024 |
| 192 – 223 | 32 | 32 × i – 5120 | 1024 ≤ t < 2048 |
| 224 – 255 | 64 | 64 × i – 12288 | 2048 ≤ t < 4096 |
| 256 – 287 | 128 | 128 × i – 28672 | 4096 ≤ t < 8192 |
| 288 – 319 | 256 | 256 × i – 65536 | 8192 ≤ t < 16384 |
| 320 – 351 | 512 | 512 × i – 147456 | 16384 ≤ t < 32768 |
| 352 – 383 | 1024 | 1024 × i – 327680 | 32768 ≤ t < 65536 |
| 384 – 415 | 2048 | 2048 × i – 720896 | 65536 ≤ t < 131072 |
| 416 – 447 | 4096 | 4096 × i – 1572864 | 131072 ≤ t < 262144 |
| 448 – 479 | 8192 | 8192 × i – 3407872 | 262144 ≤ t < 524288 |
| 480 – 511 | 16384 | 16384 × i – 7340032 | 524288 ≤ t < 1048576 |
| 512 – 543 | 32768 | 32768 × i – 15728640 | 1048576 ≤ t < 2097152 |
| 544 – 575 | 65536 | 65536 × i – 33554432 | 2097152 ≤ t < 4194304 |
| 576 – 607 | 131072 | 131072 × i – 71303168 | 4194304 ≤ t < 8388608 |
| 608 – 639 | 262144 | 262144 × i – 150994944 | 8388608 ≤ t < 16777216 |
| 640 – 671 | 524288 | 524288 × i – 318767104 | 16777216 ≤ t < 33554432 |
| 672 – 703 | 1048576 | 1048576 × i – 671088640 | 33554432 ≤ t < 67108864 |
| 704 | 2097152 | 2097152 × i – 1409286144 | 67108864 ≤ t |
| 705 – 1023 | Reserved |  |  |

- GNSS-RTK-RStransferInfo

The IE GNSS-RTK-RStransferInfo is used by the location server to provide GNSS reference station transfer information to a target device to enable the device to transfer the integer ambiguity solution associated to a current reference station to a new reference station. The transfer information is provided as a double difference of the integer parts associated to both the current and new reference stations as well as for each satellite and a reference satellite.

-- ASN1START

GNSS-RTK-RStransferInfo-r15 ::= SEQUENCE{

currentStationID-r15					GNSS-ReferenceStationID-r15,

newStationID-r15						GNSS-ReferenceStationID-r15,

svIDref-r15								SV-ID,

gnss-rtk-RStransferInfoList-r15			GNSS-RTK-RStransferInfoList-r15,

...

}

GNSS-RTK-RStransferInfoList-r15 ::= SEQUENCE (SIZE(1..63)) OF

GNSS-RTK-RStransferInfoListElement-r15

GNSS-RTK-RStransferInfoListElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

gnss-rtk-RStransferDataList-r15		GNSS-RTK-RStransferDataList-r15

...

}

GNSS-RTK-RStransferDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-RStransferDataListElement-r15

GNSS-RTK-RStransferDataListElement ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

rsTransferInteger					INTEGER (-127..128),

...

}

-- ASN1STOP

GNSS-RTK-Observations field descriptions

svIDref This field specifies the GNSS SV-ID of the satellite used as reference of the reference station transfer information for the integer ambiguity solution.

svID This field specifies the GNSS SV-ID of any satellite except the reference satellite and is used to identify the satellite to which the reference station transfer information is associated to.

currentStationID The ID of the current reference stations, i.e. the reference station the target device is transferring from

newStationID The ID of the new reference stations, i.e. the reference station the target device is transferring to

gnss-SignalID This field specifies the GNSS signal for which the reference station transfer information is associated to.

rsTransferInteger The double difference integer ambiguity solution transfer information, that transfers the double difference integer ambiguity solution associated to the current reference station and the device to the double difference integer ambiguity solution associated to the new reference station.

#### […]

#### 6.5.2.4	GNSS Assistance Data Request Elements

[…]

#### –	GNSS-RTK-ObservationsReq

The IE GNSS-RTK-ObservationsReq is used by the target device to request the GNSS-RTK-Observations assistance from the location server.

-- ASN1START

GNSS-RTK-ObservationsReq-r15::= 	SEQUENCE {

gnss-RTK-SignalsReq-r15				GNSS-SignalIDs,

gnss-RTK-PhaseRangeRateReq-r15		BOOLEAN,

gnss-RTK-CNR-Req-r15				BOOLEAN,

stationID-r15						GNSS-ReferenceStationID-r15		OPTIONAL,

currStationID-r15					GNSS-ReferenceStationID-r15		OPTIONAL,

gnss-RTK-RStransferInfoReq-r15		BOOLEAN							OPTIONAL,

...

}

-- ASN1STOP

GNSS-RTK-ObservationsReq field descriptions

gnss-RTK-SignalsReq This field specifies the GNSS Signal(s) for which the GNSS-RTK-Observations are requested. A one-value at a bit position means RTK observations for the specific signal are requested; a zero-value means not requested.

gnss-RTK-PhaseRangeRateReq This field specifies whether the rough-phase-range-rate and fine-PhaseRangeRate are requested or not. TRUE means requested.

gnss-RTK-CNR-Req This field specifies whether the carrier-to-noise-ratio is requested or not. TRUE means requested.

stationID This field specifies the Station ID for which the GNSS-RTK-Observations are requested.

currStationID This field specifies the Station ID for which the GNSS-RTK-Observations have been requested up until now.

gnss-RTK-RStransferInfoReq This field specifies whether the reference station transfer information is requested or not TRUE means requested.

[…]

#### 6.5.2.10	GNSS Capability Information Elements

[…]

- –	GNSS-GenericAssistanceDataSupport

The IE GNSS-GenericAssistanceDataSupport is used by the target device to provide information on supported GNSS generic assistance data types to the location server for each supported GNSS.

-- ASN1START

# GNSS-GenericAssistanceDataSupport ::=

SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataSupportElement

# GNSS-GenericAssistDataSupportElement ::= SEQUENCE {

gnss-ID 							GNSS-ID,

sbas-ID								SBAS-ID 					OPTIONAL, -- Cond GNSS-ID-SBAS

gnss-TimeModelsSupport				GNSS-TimeModelListSupport

OPTIONAL, -- Cond TimeModSup

gnss-DifferentialCorrectionsSupport	GNSS-DifferentialCorrectionsSupport

OPTIONAL, -- Cond DGNSS-Sup

gnss-NavigationModelSupport			GNSS-NavigationModelSupport

OPTIONAL, -- Cond NavModSup

gnss-RealTimeIntegritySupport		GNSS-RealTimeIntegritySupport

OPTIONAL, -- Cond RTISup

gnss-DataBitAssistanceSupport		GNSS-DataBitAssistanceSupport

OPTIONAL, -- Cond DataBitsSup

gnss-AcquisitionAssistanceSupport	GNSS-AcquisitionAssistanceSupport

OPTIONAL, -- Cond AcquAssistSup

gnss-AlmanacSupport					GNSS-AlmanacSupport

OPTIONAL, -- Cond AlmanacSup

gnss-UTC-ModelSupport				GNSS-UTC-ModelSupport

OPTIONAL, -- Cond UTCModSup

gnss-AuxiliaryInformationSupport	GNSS-AuxiliaryInformationSupport

OPTIONAL, -- Cond AuxInfoSup

...,

[[

bds-DifferentialCorrectionsSupport-r12

BDS-DifferentialCorrectionsSupport-r12

OPTIONAL, -- Cond DBDS-Sup

bds-GridModelSupport-r12		BDS-GridModelSupport-r12

OPTIONAL	 -- Cond BDS-GridModSup

]],

[[

gnss-RTK-ObservationsSupport-r15

GNSS-RTK-ObservationsSupport-r15

OPTIONAL,	-- Cond RTK-OSR-Sup

gnss-RTK-RStransferSupport-r15	GNSS-RTK-RStransferSupport-r15

OPTIONAL,	-- Cond RTK-RST-Sup

glo-RTK-BiasInformationSupport-r15

GLO-RTK-BiasInformationSupport-r15

OPTIONAL,	-- Cond GLO-CPB-Sup

gnss-RTK-MAC-CorrectionDifferencesSupport-r15

GNSS-RTK-MAC-CorrectionDifferencesSupport-r15

OPTIONAL,	-- Cond MAC-Sup

gnss-RTK-ResidualsSupport-r15	GNSS-RTK-ResidualsSupport-r15

OPTIONAL,	-- Cond Res-Sup

gnss-RTK-FKP-GradientsSupport-r15

GNSS-RTK-FKP-GradientsSupport-r15

OPTIONAL,	-- Cond FKP-Sup

gnss-SSR-OrbitCorrectionsSupport-r15

GNSS-SSR-OrbitCorrectionsSupport-r15

OPTIONAL, -- Cond OC-Sup

gnss-SSR-ClockCorrectionsSupport-r15

GNSS-SSR-ClockCorrectionsSupport-r15

OPTIONAL, -- Cond CC-Sup

gnss-SSR-CodeBiasSupport-r15	GNSS-SSR-CodeBiasSupport-r15

OPTIONAL  -- Cond CB-Sup

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| TimeModSup | The field is mandatory present if the target device supports GNSS-TimeModelList; otherwise it is not present. |
| DGNSS-Sup | The field is mandatory present if the target device supports GNSS-DifferentialCorrections; otherwise it is not present. |
| NavModSup | The field is mandatory present if the target device supports GNSS-NavigationModel; otherwise it is not present. |
| RTISup | The field is mandatory present if the target device supports GNSS-RealTimeIntegrity; otherwise it is not present. |
| DataBitsSup | The field is mandatory present if the target device supports GNSS-DataBitAssistance; otherwise it is not present. |
| AcquAssistSup | The field is mandatory present if the target device supports GNSS-AcquisitionAssistance; otherwise it is not present. |
| AlmanacSup | The field is mandatory present if the target device supports GNSS-Almanac; otherwise it is not present. |
| UTCModSup | The field is mandatory present if the target device supports GNSS-UTC-Model; otherwise it is not present. |
| AuxInfoSup | The field is mandatory present if the target device supports GNSS-AuxiliaryInformation; otherwise it is not present. |
| DBDS-Sup | The field is mandatory present if the target device supports BDS-DifferentialCorrections; otherwise it is not present. This field may only be present if gnss-ID indicates ‘bds’. |
| BDS-GridModSup | The field is mandatory present if the target device supports BDS-GridModel; otherwise it is not present. This field may only be present if gnss-ID indicates ‘bds’. |
| RTK-OSR-Sup | The field is mandatory present if the target device supports GNSS-RTK-Observations; otherwise it is not present. Note, support for GNSS-RTK-Observations implies support for GNSS-RTK-CommonObservationInfo as well. |
| RTK-RST-Sup | The field is mandatory present if the target device supports GNSS-RTK-RStransferInfo; otherwise it is not present. Note, support for GNSS-RTK-RStransferInfo implies support for GNSS-RTK-Observations and GNSS-RTK-CommonObservationInfo as well. |
| GLO-CPB-Sup | The field is mandatory present if the target device supports GLO-RTK-BiasInformation; otherwise it is not present. This field may only be present if gnss-ID indicates ‘glonass’. |
| MAC-Sup | The field is mandatory present if the target device supports GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| Res-Sup | The field is mandatory present if the target device supports GNSS-RTK-Residuals; otherwise it is not present. |
| FKP-Sup | The field is mandatory present if the target device supports GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| OC-Sup | The field is mandatory present if the target device supports GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| CC-Sup | The field is mandatory present if the target device supports GNSS-SSR-ClockCorrections; otherwise it is not present. |
| CB-Sup | The field is mandatory present if the target device supports GNSS-SSR-CodeBias; otherwise it is not present. |

[…]

#### –	GNSS-RTK-ObservationsSupport

-- ASN1START

GNSS-RTK-ObservationsSupport-r15 ::= 	SEQUENCE {

gnssSignalIDs-r15			GNSS-SignalIDs,

...

}

-- ASN1STOP

GNSS-RTK-ObservationsSupport field descriptions

gnssSignalIDs This field specifies the GNSS signal types for which GNSS-RTK-Observations are supported by the target device. This is represented by a bit string in GNSS-SignalIDs, with a one-value at the bit position means GNSS-RTK-Observations for the particular GNSS signal type is supported; a zero-value means not supported.

#### –	GNSS-RTK-RStransferSupport

-- ASN1START

GNSS-RTK-RStransferSupport-r15 ::= 	SEQUENCE {

...

}

-- ASN1STOP