3GPP TSG-RAN WG2 #102	Tdoc R2-1808119

Busan, Korea, 21st – 25th May 2018

Agenda Item:	9.7.3

Source:	Ericsson

Title:	Resume Request for LTE connected to 5GC

Document for:	Discussion

# Introduction

The present contribution includes a proposal for an RRC Connection Resume Request message in LTE

# Discussion

For LTE connected to 5GC, we think it is important to, to the extent possible, make clear separations in existing messages and not interleave too much the messages that are intended for UE’s connecting to EPC vs. UEs that are connecting to 5GC. The reason for this is manifold.

First, it goes to clarity. It is much easier to follow and not jeopardize the quality of a standard with clear differentiators.

Second, we think that there will be evolution of one that doesn’t necessarily follow the other. At least in these two aspects, it is generally a good idea to separate EPC-specific procedures from 5GC-specific ones, even though they are utilizing the same messages and procedures.

For RRCConnectionResumeRequest in particular there are a couple of things we note, that we think makes it worthwhile to separate and make a different set of release 15 IE’s.

We don’t want to coordinate ResumeID’s for resuming from RRC_IDLE (release 13) with the I-RNTI’s used for resuming from RRC_INACTIVE.

We don’t consider that there will be truncated versions of I-RNTI. At least this is not yet agreed to support. RAN2 should further discuss the support of truncated resume identifiers and this should also be aligned with a similar discussion for NR.

We need to add another cause value for resuming a Release 15 UE from RRC_INACTIVE – The RAN Area Update. This cause value is not relevant for UE’s that are resuming from RRC_IDLE (see, e.g.,[1]).

We think that this together, or actually even each of these isolated, would justify that we in the RRCConnectionResumeRequest message include a release-15 extension

Add a release 15 rrcConnectionResumeRequest extension to the resume message in LTE, for specifying LTE identifiers and causes relevant for LTE connected to 5GC.

# Text Proposal

RRCConnectionResumeRequest message

-- ASN1START

RRCConnectionResumeRequest-r13 ::=	SEQUENCE {

criticalExtensions						CHOICE {

rrcConnectionResumeRequest-r13			RRCConnectionResumeRequest-r13-IEs,

criticalExtensionsFuture				RRCConnectionResumeRequest-r15-IEs

}

}

RRCConnectionResumeRequest-r13-IEs ::=		SEQUENCE {

resumeIdentity-r13								CHOICE {

resumeID-r13									ResumeIdentity-r13,

truncatedResumeID-r13							BIT STRING (SIZE (24))

},

shortResumeMAC-I-r13								BIT STRING (SIZE (16)),

resumeCause-r13									ResumeCause,

spare											BIT STRING (SIZE (1))

}

RRCConnectionResumeRequest-r15-IEs ::=		SEQUENCE {

I-RNTI-r15										BIT STRING (SIZE (40)),

shortResumeMAC-I-r13							BIT STRING (SIZE (16)),

resumeCause-r15									ResumeCause-R15,

spare											BIT STRING (SIZE (FFS))

}

ResumeCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1}

ResumeCause-R15 ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, RNA-Update, mo-VoiceCall-v1280, spare1}

-- ASN1STOP

# Conclusion

In section 2 we made the following observations:

Observation 1	We don’t want to coordinate ResumeID’s for resuming from RRC_IDLE (release 13) with the I-RNTI’s used for resuming from RRC_INACTIVE.

Observation 2	We don’t consider that there will be truncated versions of I-RNTI. At least this is not yet agreed to support. RAN2 should further discuss the support of truncated resume identifiers and this should also be aligned with a similar discussion for NR.

Observation 3	We need to add another cause value for resuming a Release 15 UE from RRC_INACTIVE – The RAN Area Update. This cause value is not relevant for UE’s that are resuming from RRC_IDLE (see, e.g.,[1]).

Based on the discussion in section 2 we propose the following:

Proposal 1	Add a release 15 rrcConnectionResumeRequest extension to the resume message in LTE, for specifying LTE identifiers and causes relevant for LTE connected to 5GC.

# References

R2-1806758, RRC resume cause values, Ericsson, 3GPP TSG-RAN WG2 #102 Busan, Korea, 21st – 25th May 2018