3GPP TSG-RAN WG2 #102	R2-1808162

Busan, Republic of Korea, 21st - 25th May2018	Revision of R2-1805827

Agenda Item:	9.9.2

Source:	Ericsson

Title:	Fallback to long CQI reporting period

Document for:	Discussion, Decision

# Introduction

In Reno RAN2 agreed to have a short CQI reporting period after SCell activation but it is FFS how long this shorter periodicity is applied:

Agreements:

1	The short period of the CQI reporting resource could be only available upon receiving the SCell activation command. FFS when the UE fall back to longer periodicity.

# Discussion

If the UE is configured to apply the short CQI reporting periodicity for SCell activation RAN2 needs to decide when the UE falls back to the longer periodicity. We see two alternatives:

- the UE uses the shorter periodicity a fixed time (e.g. max activation time (24/34 ms)).

- the UE uses the shorter periodicity until the UE gets scheduled.

Alternative B will cause less CQI reporting if the eNB schedules the UE quickly, while more CQI reporting otherwise. One may discuss what is best from CQI-reporting perspective, but probably the amount of CQI reporting comparing A and B is not a critical problem. On the other hand, from an implementation point of view (both UE and eNB) it seems to use simpler to have a fixed time rather than a time depending on scheduling so A would be simpler so we therefore prefer A.

The eNB can sometimes not know if the SCell is known or unknown for the UE, thus eNB does not always know if the SCell activation will take 24 or 34 ms for the UE. If we chose 34 ms, we might have some extra CQI transmissions during the last 10 ms. However, that is a minor issue compared to chosing 24 ms and maybe missing the first correct CQI report during the last 10 ms of the activation. Thus we suggest to go with the 34 ms fixed time after activation.

The UE falls back to the longer CQI periodicity 34 ms after activation.

# Conclusion

We ask RAN2 to agree on the following:

- The UE falls back to the longer CQI periodicity 34 ms after activation.