3GPP TSG-RAN WG2 Meeting #102			R2-1809203

Busan, South Korea, 21st - 25th May 2018

Title:	LS for UE capability of measurement gap for inter-RAT NR measurement not yet configured with EN-DC

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT

Source:	RAN2

To:	RAN4

Cc:

Contact Person:

#### Name:	Jing He

E-mail Address:	jing.1.he@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

To configure inter-RAT NR measurement to the LTE UE not yet configured with EN-DC, the LTE network needs to know whether the UE needs gaps for such measurements. Adding NeedForGap capability to each NR bands into LTE RRC signaling hasn’t reach consensus.

RAN2 wonders if the MR-DC band combination information reported in UE capability can be used to identify some cases where gapless measurements can be performed. For example, the network could assume that the UE is able to perform measurements without gaps on all carriers that could (according to the advertised band combinations in the UE capabilities) be configured as a serving cell in addition to the currently configured serving cells of this UE.

For Rel-15 RAN2 currently does not intend to introduce any specific UE capability to indicate the need for gaps for inter-RAT NR measurements. Consequently, if MR-DC band combination information could not be used e.g. as suggested above, apart from gapless SFTD measurements, when configuring inter-RAT NR measurements to an EN-DC capable UE not yet configured with EN-DC, the LTE network would always configure measurement gaps to the UE.

2. Actions:

To RAN4 group.

ACTION: 	RAN2 respectfully asks RAN4 whether the MR-DC band combinations reported in UE capability can allow the network to identify the cases where the LTE UE not yet configured with EN-DC can perform gapless measurements.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2 Adhoc	2018.07.02 – 2018.07.06		Montreal

3GPP TSG RAN WG2 #103	2018.08.20 – 2018.08.24		Gothenburg