3GPP TSG-RAN WG2 Meeting #103	R2-1811296

Gothenburg, Sweden, 20-24 August 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.324 | CR | draftCR | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on flow remapping from default DRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current specification constructs and submits end marker control PDU to default DRB during flow remapping as long as there is no stored QoS flow to DRB mapping for the QoS flow and default DRB is configured. Per previous agreement, if a DRB is not configured with SDAP header, the end marker cannot be transmitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify end marker is transmitted when flow remapping from default DRB happens only if default DRB is configured with SDAP header Impact Analysis: Impacted functionality: QoS flow to DRB mapping Interoperability issue: No interoperability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - End marker may be transmitted when network does not configure SDAP header for default DRB causing misinterpretation at receiver side. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.4, 8.1, 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

### 5.3.1	Configuration

When RRC [3] configures an UL QoS flow to DRB mapping rule for a QoS flow, the SDAP entity shall:

-	if the SDAP entity has already been establishment and there is no stored QoS flow to DRB mapping rule for the QoS flow and a default DRB with the presence of SDAP header is configured:

-	construct an end-marker control PDU, as specified in subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the default DRB;

-	submit the end-marker control PDU to the lower layers.

-	if the stored UL QoS flow to DRB mapping rule is different from the configured QoS flow to DRB mapping rule for the QoS flow and the DRB according to the stored QoS flow to DRB mapping rule is configured by RRC [3] with the presence of SDAP header:

-	construct an end-marker control PDU, as specified in subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	submit the end-marker control PDU to the lower layers.

-	store the configured UL QoS flow to DRB mapping rule for the QoS flow.

When RRC [3] releases an UL QoS flow to DRB mapping rule for a QoS flow, the SDAP entity shall:

-	remove the UL QoS flow to DRB mapping rule for the QoS flow.

### 5.3.2	Reflective mapping

For each received DL SDAP PDU with RDI set to 1, the SDAP entity shall:

-	process the QFI field in the SDAP header and determine the QoS flow;

-	if there is no stored QoS flow to DRB mapping rule for the QoS flow and a default DRB with the presence of SDAP header is configured:

-	construct an end-marker control PDU, as specified in subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the default DRB;

-	submit the end-marker control PDU to the lower layers;

-	if the stored QoS flow to DRB mapping rule for the QoS flow is different from the QoS flow to DRB mapping of the DL SDAP data PDU and the DRB according to the stored QoS flow to DRB mapping rule is configured by RRC [3] with the presence of SDAP header:

-	construct an end-marker control PDU, as specified in subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	submit the end-marker control PDU to the lower layers;

-	store the QoS flow to DRB mapping of the DL SDAP data PDU as the QoS flow to DRB mapping rule for the UL.

End of changes