3GPP TSG-RAN WG2 AH 1807	R2-1811498

# Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | Draft | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 36.331 to introduce ANR reporting for EUTRA connected to 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE corporation, Sanechips, Qualcomm, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enhancement to the EUTRA CGI reporting for ANR when a neighbour cell is connected to 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this CR, we propose to include cellAccessRelatedInfoList-5GC-r15 including plmn-IdentityList-r15, trackingAreaCode-5GC-r15, ran-AreaCode-r15 (if available) and cellIdentity -5GC-r15 for each entry of the cellAccessRelatedInfoList-5GC-r15 to indicate the PLMN lists, TAC, RANAC and cell identity when a neighbouring EUTRAN cell is connected to 5GC. The supported CN type is also reported implicitly via the 5GC PLMN list and TAC value per PLMN. The legacy cgi-Info is also reported along with the new report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNB is unable to decide the supported CN type of the reported EUTRA cell (EPC, 5GC or both) and is unable to know the 5GC PLMN lists, TAC, RANAC and cell identity when a neighbouring EUTRAN cell is connected to 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.5, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of first change

### 5.5.3	Performing measurements

#### 5.5.3.1	General

For all measurements, except for UE Rx–Tx time difference measurements, RSSI, UL PDCP Packet Delay per QCI measurement, channel occupancy measurements, CBR measurement, and except for WLAN measurements of Band, Carrier Info, Available Admission Capacity, Backhaul Bandwidth, Channel Utilization, and Station Count, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria or for measurement reporting. When performing measurements on NR carriers, the UE derives the cell quality as specified in 5.5.3.3 and the beam quality as specified in 5.5.3.4.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell as follows:

2>	for the PCell, apply the time domain measurement resource restriction in accordance with measSubframePatternPCell, if configured;

2>	if the UE supports CRS based discovery signals measurement:

3>	for each SCell in deactivated state, apply the discovery signals measurement timing configuration in accordance with measDS-Config, if configured within the measObject corresponding to the frequency of the SCell;

1>	if the UE has a measConfig with rs-sinr-Config configured, perform RS-SINR (as indicated in the associated reportConfig) measurements as follows:

2>	perform the corresponding measurements on the frequency indicated in the associated measObject using available idle periods or using autonomous gaps as necessary;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the purpose for the associated reportConfig is set to reportCGI:

3>	if si-RequestForHO is configured for the associated reportConfig:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using autonomous gaps as necessary;

3>	else:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using available idle periods or using autonomous gaps as necessary;

NOTE 1:	If autonomous gaps are used to perform measurements, the UE is allowed to temporarily abort communication with all serving cell(s), i.e. create autonomous gaps to perform the corresponding measurements within the limits specified in TS 36.133 [16]. Otherwise, the UE only supports the measurements with the purpose set to reportCGI only if E-UTRAN has provided sufficient idle periods.

3>	try to acquire the global cell identity of the cell indicated by the cellForWhichToReportCGI in the associated measObject by acquiring the relevant system information from the concerned cell;

3>	if an entry in the cellAccessRelatedInfoList includes the selected PLMN, acquire the relevant system information from the concerned cell;

3>	if the cell indicated by the cellForWhichToReportCGI included in the associated measObject is an E-UTRAN cell:

4>	try to acquire the CSG identity, if the CSG identity is broadcast in the concerned cell;

4>	try to acquire the trackingAreaCode in the concerned cell;

4>	try to acquire the list of additional PLMN Identities, as included in the plmn-IdentityList, if multiple PLMN identities are broadcast in the concerned cell;

4>	if cellAccessRelatedInfoList is included, use trackingAreaCode and plmn-IdentityList from the entry of cellAccessRelatedInfoList containing the selected PLMN;

4>	if the includeMultiBandInfo is configured:

5>	try to acquire the freqBandIndicator in the SystemInformationBlockType1of the concerned cell;

5>	try to acquire the list of additional frequency band indicators, as included in the multiBandInfoList, if multiple frequency band indicators are included in the SystemInformationBlockType1of the concerned cell;

5>	try to acquire the freqBandIndicatorPriority, if the freqBandIndicatorPriority is included in the SystemInformationBlockType1of the concerned cell;

4>	if report5GCInfo-r15 is configured; and

4> if cellAccessRelatedInfoList-5GC is broadcast in the concerned cell

5> try to acquire the cellAccessRelatedInfoList-5GC;

next change

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to E-UTRAN. The UE shall initiate this procedure only after successful security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultPCell to include the quantities of the PCell;

1>	set the measResultServFreqList to include for each E-UTRA SCell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in [16], except if purpose for the reportConfig associated with the measId that triggered the measurement reporting is set to reportLocation;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2>	for each E-UTRA serving frequency for which measObjectId is referenced in the measIdList, other than the frequency corresponding with the measId that triggered the measurement reporting:

3>	set the measResultServFreqList to include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	if the triggerType is set to event; and if the corresponding measObject concerns NR; and if eventId is set to eventB1 or eventB2; or

1>	if the triggerType is set to event; and if eventId is set to eventA3 or eventA4 or eventA5:

2>	if purpose for the reportConfig associated with the measId that triggered the measurement reporting is set to a value other than reportLocation:

3>	set the measResultServFreqListNR to include for each NR serving frequency, if any, the following:

4>	set measResultSCell to include the available results of the NR serving cell, as specified in 5.5.5.2;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set measResultBestNeighCell to include the available results, as specified in 5.5.5.2, of the best non-serving cell, ordered based on the quantity determined as specified in 5.5.5.3;

5>	for each (serving or neighbouring) cell for which the UE reports results according to the previous, additionally include available beam results according to the following:

6>	if maxReportRS-Index is configured, set measResultCellRS-Index to include available results, as specified in 5.5.5.2, of up to maxReportRS-Index beams, ordered based on the quantity determined as specified in 5.5.5.3;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the triggerType is set to event:

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE 1:	The reliability of the report (i.e. the certainty it contains the strongest cells on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in TS 36.133 [16].

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the triggerType is set to event; or the purpose is set to reportStrongestCells or to reportStrongestCellsForSON:

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns E-UTRA:

6>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfig in order of decreasing triggerQuantity, i.e. the best cell is included first;

5>	if the measObject associated with this measId concerns NR:

6>	set the measResultCell to include the quantity(ies) indicated in the reportQuantityCellNR within the concerned reportConfig in order of decreasing quantity according to bN-ThresholdYNR, i.e. the best cell is included first;

6>	if maxReportRS-Index is configured, set measResultCellRS-Index to include results of up to maxReportRS-Index beams whose quantity is above threshRS-Index defined in the VarMeasConfig for the corresponding measObject and in order of decreasing quantity, same as used for cell reporting, and as follows:

7>	order beams based on the reporting quantity determined as specified in 5.5.5.3;

7>	include ssbIndex;

7>	if reportQuantityRS-IndexNR and reportRS-IndexResultsNR are configured, for each quantity indicated, include the corresponding measurement result;

5>	if the measObject associated with this measId concerns UTRA FDD and if ReportConfigInterRAT includes the reportQuantityUTRA-FDD:

6>	set the measResult to include the quantities indicated by the reportQuantityUTRA-FDD in order of decreasing measQuantityUTRA-FDD within the quantityConfig, i.e. the best cell is included first;

5>	if the measObject associated with this measId concerns UTRA FDD and if ReportConfigInterRAT does not include the reportQuantityUTRA-FDD; or

5>	if the measObject associated with this measId concerns UTRA TDD, GERAN or CDMA2000:

6>	set the measResult to the quantity as configured for the concerned RAT within the quantityConfig in order of either decreasing quantity for UTRA and GERAN or increasing quantity for CDMA2000 pilotStrength, i.e. the best cell is included first;

3>	else if the purpose is set to reportCGI and the corresponding measObject concerns a RAT other than NR:

4>	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5>		if the includeMultiBandInfo is configured:

6>	include the freqBandIndicator;

6>		if the cell broadcasts the multiBandInfoList, include the multiBandInfoList;

6>	if the cell broadcasts the freqBandIndicatorPriority, include the freqBandIndicatorPriority;

5>	if the cell broadcasts a CSG identity:

6>	include the csg-Identity;

6>	include the csg-MemberStatus and set it to member if the cell is a CSG member cell;

5>	if the si-RequestForHO is configured within the reportConfig associated with this measId:

6>	include the cgi-Info containing all the fields other than the plmn-IdentityList that have been successfully acquired;

6>	include, within the cgi-Info, the field plmn-IdentityList in accordance with the following:

7>	if the cell is a CSG member cell, determine the subset of the PLMN identities, starting from the second entry of PLMN identities in the broadcast information, that meet the following conditions:

a)	equal to the RPLMN or an EPLMN; and

b)	the CSG whitelist of the UE includes an entry comprising of the concerned PLMN identity and the CSG identity broadcast by the cell;

7>	if the subset of PLMN identities determined according to the previous includes at least one PLMN identity, include the plmn-IdentityList and set it to include this subset of the PLMN identities;

7>	if the cell is a CSG member cell, include the primaryPLMN-Suitable if the primary PLMN meets conditions a) and b) specified above;

7>	if the cell does not broadcast csg-Identity and the UE is capable of reporting the plmn-IdentityList from cells not broadcasting csg-Identity:

8>	include in the plmn-IdentityList the list of identities starting from the second entry of PLMN identities in the broadcast information;

5>	else:

6>	include the cgi-Info containing all the fields that have been successfully acquired and in accordance with the following:

7>	include in the plmn-IdentityList the list of identities starting from the second entry of PLMN Identities in the broadcast information;

4>  if report5GCInfo-r15 is configured; and

4> if the cellAccessRelatedInfoList-5GC is broadcast and the UE is capable of decoding the cellAccessRelatedInfoList-5GC in the broadcast information:

5>	include cgi-Info-5GC;

<Cut until modified section>

### 6.2.2	Message definitions

<Cut until modified section>

SystemInformationBlockType1 message

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SystemInformationBlockType1-v890-IEs::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs)			OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v920-IEs	OPTIONAL

}

-- Late non critical extensions

SystemInformationBlockType1-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList					MultiBandInfoList		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL,	-- Cond FBI-max

multiBandInfoList-v9e0				MultiBandInfoList-v9e0		OPTIONAL,	-- Cond mFBI-max

nonCriticalExtension				SystemInformationBlockType1-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType1-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non critical extensions

SystemInformationBlockType1-v920-IEs ::=	SEQUENCE {

ims-EmergencySupport-r9				ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfo-v920				CellSelectionInfo-v920		OPTIONAL,	-- Cond RSRQ

nonCriticalExtension				SystemInformationBlockType1-v1130-IEs	OPTIONAL

}

SystemInformationBlockType1-v1130-IEs ::=	SEQUENCE {

tdd-Config-v1130				TDD-Config-v1130			OPTIONAL,	-- Cond TDD-OR

cellSelectionInfo-v1130			CellSelectionInfo-v1130		OPTIONAL,	-- Cond WB-RSRQ

nonCriticalExtension			SystemInformationBlockType1-v1250-IEs	OPTIONAL

}

SystemInformationBlockType1-v1250-IEs ::=	SEQUENCE {

cellAccessRelatedInfo-v1250					SEQUENCE {

category0Allowed-r12						ENUMERATED {true}		OPTIONAL	-- Need OP

},

cellSelectionInfo-v1250					CellSelectionInfo-v1250		OPTIONAL,	-- Cond RSRQ2

freqBandIndicatorPriority-r12			ENUMERATED {true}			OPTIONAL,	-- Cond mFBI

nonCriticalExtension			SystemInformationBlockType1-v1310-IEs	OPTIONAL

}

SystemInformationBlockType1-v1310-IEs ::=	SEQUENCE {

hyperSFN-r13								BIT STRING (SIZE (10))		OPTIONAL,	-- Need OR

eDRX-Allowed-r13							ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13	OPTIONAL,	-- Need OP

bandwidthReducedAccessRelatedInfo-r13	SEQUENCE {

si-WindowLength-BR-r13					ENUMERATED {

ms20, ms40, ms60, ms80, ms120,

ms160, ms200, spare},

si-RepetitionPattern-r13				ENUMERATED {everyRF, every2ndRF, every4thRF,

every8thRF},

schedulingInfoList-BR-r13				SchedulingInfoList-BR-r13	OPTIONAL,	-- Cond SI-BR

fdd-DownlinkOrTddSubframeBitmapBR-r13	CHOICE {

subframePattern10-r13					BIT STRING (SIZE (10)),

subframePattern40-r13					BIT STRING (SIZE (40))

}																	OPTIONAL,	-- Need OP

fdd-UplinkSubframeBitmapBR-r13			BIT STRING (SIZE (10))		OPTIONAL,	-- Need OP

startSymbolBR-r13						INTEGER (1..4),

si-HoppingConfigCommon-r13				ENUMERATED {on,off},

si-ValidityTime-r13						ENUMERATED {true}	OPTIONAL,			-- Need OP

systemInfoValueTagList-r13				SystemInfoValueTagList-r13	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond BW-reduced

nonCriticalExtension						SystemInformationBlockType1-v1320-IEs	OPTIONAL

}

SystemInformationBlockType1-v1320-IEs ::=	SEQUENCE {

freqHoppingParametersDL-r13				SEQUENCE {

mpdcch-pdsch-HoppingNB-r13				ENUMERATED {nb2, nb4}		OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13					ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13					ENUMERATED { int5, int10, int20, int40}

}																	OPTIONAL,	-- Need OR

mpdcch-pdsch-HoppingOffset-r13			INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Hopping

nonCriticalExtension						SystemInformationBlockType1-v1350-IEs					OPTIONAL

}

SystemInformationBlockType1-v1350-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL,	-- Need OP

nonCriticalExtension					SystemInformationBlockType1-v1360-IEs				OPTIONAL

}

SystemInformationBlockType1-v1360-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-v1360				CellSelectionInfoCE1-v1360	OPTIONAL,	-- Cond QrxlevminCE1

nonCriticalExtension						SystemInformationBlockType1-v1430-IEs		OPTIONAL

}

SystemInformationBlockType1-v1430-IEs ::=	SEQUENCE {

eCallOverIMS-Support-r14				ENUMERATED {true}			OPTIONAL,	-- Need OR

tdd-Config-v1430						TDD-Config-v1430			OPTIONAL,	-- Cond TDD-OR

cellAccessRelatedInfoList-r14			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14	OPTIONAL,	-- Need OR

nonCriticalExtension					SystemInformationBlockType1-v1450-IEs				OPTIONAL

}

SystemInformationBlockType1-v1450-IEs ::=	SEQUENCE {

tdd-Config-v1450						TDD-Config-v1450		OPTIONAL,	-- Cond TDD-OR

nonCriticalExtension					SystemInformationBlockType1-v15x0-IEs					OPTIONAL

}

SystemInformationBlockType1-v15x0-IEs ::=	SEQUENCE {

hsdn-Cell-r15							ENUMERATED{true}			OPTIONAL,

pos-schedulingInfoList-r15				Pos-SchedulingInfoList-r15		OPTIONAL,

nw-BasedCRS-InterferenceMitigation-r15	ENUMERATED {enabled}			OPTIONAL,	-- Need OR

cellBarred-CRS-r15						ENUMERATED {barred, notBarred}	OPTIONAL,	-- Need OR

plmn-IdentityList-r15					PLMN-IdentityList-r15 			OPTIONAL,	-- Need OR

cellSelectionInfoCE-v15x0 				CellSelectionInfoCE-v15x0		OPTIONAL,	-- Need OP

cellBarred-CRS-CE-r15					ENUMERATED {barred, notBarred},

crs-MutingNumPRBs-r15					ENUMERATED {n6, n24}			OPTIONAL,	-- Need OR

cellAccessRelatedInfoFor-5GC-r15			SEQUENCE {

cellBarred-5GC-r15						ENUMERATED {barred, notBarred},

cellAccessRelatedInfoList-5GC-r15			SEQUENCE (SIZE (1..maxPLMN-r11)) OF

CellAccessRelatedInfo-5GC-r15

}				OPTIONAL,	-- NEED OP

ims-EmergencySupport5GC-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

eCallOverIMS-Support5GC-r15			ENUMERATED {true}			OPTIONAL,	-- Need OR

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

PLMN-IdentityInfoList-r15 ::=				SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-r15

PLMN-IdentityInfo-r15 ::=				SEQUENCE {

plmn-Identity-r15						PLMN-Identity,

cellReservedForOperatorUse-CRS-r15		ENUMERATED {reserved, notReserved}

}

CellAccessRelatedInfo-5GC-r15 ::=	SEQUENCE {

plmn-IdentityList-r15			PLMN-IdentityList2-r15,

ran-AreaCode-r15					RAN-AreaCode-r15 OPTIONAL,	-- NEED OP

trackingAreaCode-5GC-r15			TrackingAreaCode-5GC-r15,

cellIdentity-5GC-r15				CellIdentity-5GC-r15

}

CellIdentity-5GC-r15 ::=			CHOICE{

cellIdentity-r15					CellIdentity,

cellId-Index-r15					INTEGER (1..maxPLMN-r11)

}

PLMN-IdentityList2-r15::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo2-r15

PLMN-IdentityInfo2-r15 ::=				SEQUENCE {

plmn-Identity-5GC-r15					CHOICE{

plmn-Identity-r15						PLMN-Identity,

plmn-Index-r15							INTEGER (1..maxPLMN-r11)

},

cellReservedForOperatorUse-r15			ENUMERATED {reserved, notReserved}

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfo ::=	SEQUENCE {

si-Periodicity						ENUMERATED {

rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo						SIB-MappingInfo

}

SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13

SchedulingInfo-BR-r13 ::=	SEQUENCE {

si-Narrowband-r13				INTEGER (1..maxAvailNarrowBands-r13),

si-TBS-r13						ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712,

b808, b936}

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310, sibType21-v1430,

sibType22-v15x0, sibType24-v15x0}

SystemInfoValueTagList-r13 ::=		SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13

SystemInfoValueTagSI-r13 ::=		INTEGER (0..3)

CellSelectionInfo-v920 ::=			SEQUENCE {

q-QualMin-r9						Q-QualMin-r9,

q-QualMinOffset-r9					INTEGER (1..8)						OPTIONAL	-- Need OP

}

CellSelectionInfo-v1130 ::=			SEQUENCE {

q-QualMinWB-r11						Q-QualMin-r9

}

CellSelectionInfo-v1250 ::=			SEQUENCE {

q-QualMinRSRQ-OnAllSymbols-r12						Q-QualMin-r9

}

CellAccessRelatedInfo-r14 ::=	SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList,

trackingAreaCode-r14				TrackingAreaCode,

cellIdentity-r14					CellIdentity

}

Pos-SchedulingInfoList-r15 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF Pos-SchedulingInfo-r15

Pos-SchedulingInfo-r15 ::=	SEQUENCE {

pos-si-Periodicity-r15		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

pos-sib-MappingInfo-r15		Pos-SIB-MappingInfo-r15

}

Pos-SIB-MappingInfo-r15 ::= SEQUENCE (SIZE (1..maxSIB)) OF Pos-SIB-Type-r15

Pos-SIB-Type-r15 ::= SEQUENCE {

encrypted-r15		ENUMERATED { true }				OPTIONAL,		-- Need OP

gnss-id-r15			GNSS-ID-r15						OPTIONAL,		-- Need OP

sbas-id-r15			SBAS-ID-r15						OPTIONAL,		-- Need OP

pos-sib-type-r15	ENUMERATED { 	posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType3-1,

... }

}

-- ASN1STOP

| SystemInformationBlockType1 field descriptions |
| --- |
| bandwithReducedAccessRelatedInfo Access related information for BL UEs and UEs in CE. NOTE 3. |
| category0Allowed The presence of this field indicates category 0 UEs are allowed to access the cell. |
| cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 4. |
| cellAccessRelatedInfoList-5GC This field contains a PLMN list and a list allowing signalling of access related information per PLMN for PLMNs that provides connectivity to 5GC. One PLMN can be included in only one entry of this list. NOTE4 |
| cellBarred barred means the cell is barred, as defined in TS 36.304 [4]. This field is ignored by the UE if the UE supports network-based CRS interference mitigation and nw-BasedCRS-InterferenceMitigation is included in SystemInformationBlockType1. |
| cellBarred-5GC barred means the cell is barred for connectivity to 5GC, as defined in TS 36.304 [4]. |
| cellBarred-CRS barred means the cell is barred for UEs capable of network-based CRS interference mitigation while nw-BasedCRS-InterferenceMitigation is included in SystemInformationBlockType1, as defined in TS 36.304 [4]. This field is ignored by the UE if the UE does not support network-based CRS interference mitigation. |
| cellIdentity Indicates the cell identity. NOTE 2. |
| cellId-index the index of the cell ID in the PLMN lists for EPC, indicates UE the corresponding cell ID is used for 5GC. Value 1 indicates the cell ID of the 1st PLMN list for EPC in the SIB1. Value 2 indicates the cell ID of the 2nd PLMN list for EPC, and so on. |
| cellReservedForOperatorUse As defined in TS 36.304 [4]. This field is ignored by the UE if the UE supports network-based CRS interference mitigation and nw-BasedCRS-InterferenceMitigation is included in SystemInformationBlockType1. |
| cellReservedForOperatorUse-CRS As defined in TS 36.304 [4]. This field is ignored by the UE if the UE does not support network-based CRS interference mitigation. |
| cellSelectionInfoCE Cell selection information for BL UEs and UEs in CE. If absent, coverage enhancement S criteria is not applicable. NOTE 3. |
| cellSelectionInfoCE1 Cell selection information for BL UEs and UEs in CE supporting CE Mode B. E-UTRAN includes this IE only if cellSelectionInfoCE is present in SystemInformationBlockType1-BR. NOTE 3. |  |
| crs-MutingNumPRBs Indicates number of PRBs, i.e. 6 or 24 PRBs, for CRS transmission in the central cell BW when CRS muting is enabled for BL UEs or UEs in CE supporting ce-CRS-Muting. Presence of this field indicates CRS muting is enabled in the cell, as specified in TS 36.133 [16]. If this field is configured and the cell is not barred for this UE (as specified in TS 36.304 [4]), the behaviour is undefined for UEs not supporting this feature. |  |
| csg-Identity Identity of the Closed Subscriber Group the cell belongs to. |
| csg-Indication If set to TRUE the UE is only allowed to access the cell if it is a CSG member cell, if selected during manual CSG selection or to obtain limited service, see TS 36.304 [4]. |
| eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services via EPC for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS via EPC is not supported by the network in the cell. NOTE 2. |
| eCallOverIMS-Support5GC Indicates whether the cell supports eCall over IMS services via 5GC as defined in TS 23.401 [41]. If absent, eCall over IMS via 5GC is not supported by the network in the cell. NOTE 2. |
| eDRX-Allowed The presence of this field indicates if idle mode extended DRX is allowed in the cell. The UE shall stop using extended DRX in idle mode if eDRX-Allowed is not present. |
| encrypted The presence of this field indicates that the pos-sib-type is encrypted as specified in TS 36.355 [54]. |
| fdd-DownlinkOrTddSubframeBitmapBR The set of valid subframes for FDD downlink or TDD transmissions, see TS 36.213 [23]. If this field is present, SystemInformationBlockType1-BR-r13 is transmitted in RRCConnectionReconfiguration, and if RRCConnectionReconfiguration does not include systemInformationBlockType2Dedicated, UE may assume the valid subframes in fdd-DownlinkOrTddSubframeBitmapBR are not indicated as MBSFN subframes. If this field is not present, the set of valid subframes is the set of non-MBSFN subframes as indicated by mbsfn-SubframeConfigList. If neither this field nor mbsfn-SubframeConfigList is present, all subframes are considered as valid subframes for FDD downlink transmission, all DL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD DL transmission, and all UL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD UL transmission. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| fdd-UplinkSubframeBitmapBR The set of valid subframes for FDD uplink transmissions for BL UEs, see TS 36.213 [23]. If the field is not present, then UE considers all uplink subframes as valid subframes for FDD uplink transmissions. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| freqBandIndicatorPriority If the field is present and supported by the UE, the UE shall prioritize the frequency bands in the multiBandInfoList field in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator field. Otherwise, the UE applies frequency band according to the rules defined in multiBandInfoList. NOTE 2. |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency band in freqBandIndicator. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| freqHoppingParametersDL Downlink frequency hopping parameters for BR versions of SI messages, MPDCCH/PDSCH of paging, MPDCCH/PDSCH of RAR/Msg4 and unicast MPDCCH/PDSCH. If not present, the UE is not configured downlink frequency hopping. |
| gnss-ID The presence of this field indicates that the positioning SIB type is for a specific GNSS. |
| hsdn-Cell This field indicates this is a HSDN cell as specified in TS 36.304 [4]. |
| hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. |
| ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services via EPC for UEs in limited service mode. If absent, IMS emergency call via EPC is not supported by the network in the cell for UEs in limited service mode. NOTE 2. |
| ims-EmergencySupport5GC Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode via 5GC. If absent, IMS emergency call via 5GC is not supported by the network in the cell for UEs in limited service mode. NOTE 2. |
| intraFreqReselection Used to control cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 36.304 [4]. NOTE 2. |
| multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42, table 5.5-1] that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE. |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency bands in multiBandInfoList (i.e. without suffix) and multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| nw-BasedCRS-InterferenceMitigation This field indicates whether network-based CRS interference mitigation is enabled for the cell, as specified in TS 36.133 [16]. If this field is configured and the cell is not barred for this UE (as specified in TS 36.304), the behaviour is undefined for UEs not supporting this feature. |
| plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN. If plmn-IdentityList-r15 is included it includes the same number of entries as plmn-IdentityList (without suffix). NOTE 2. |
| plmn-index Index of the PLMN in the plmn-IdentityList fields included in SIB1for EPC, indicating the same PLMN ID is connected to 5GC too. 1 if the 1st PLMN is from the 1st plmn-IdentityList included in SIB1, 2 if the 2nd PLMN is from the same plmn-IdentityList, or when no more PLMN are present within the same plmn-IdentityList, then the PLMN listed 1st in the subsequent plmn-IdentityList within the same SIB1 and so on. NOTE 6. |
| p-Max Value applicable for the cell. If absent the UE applies the maximum power according to its capability as specified in TS 36.101 [42, 6.2.2]. NOTE 2. |
| pos-sib-MappingInfo List of the posSIBs mapped to this SystemInformation message. |
| pos-sib-type The positioning SIB type is defined in TS 36.355 [54]. |
| q-QualMin Parameter "Qqualmin" in TS 36.304 [4]. If cellSelectionInfo-v920 is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinOffset Parameter "Qqualminoffset" in TS 36.304 [4]. Actual value Qqualminoffset = field value [dB]. If cellSelectionInfo-v920 is not present or the field is not present, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| q-RxLevMinOffset Parameter Qrxlevminoffset in TS 36.304 [4]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell. |
| sbas-ID The presence of this field indicates that the positioning SIB type is for a specific SBAS. |
| sib-MappingInfo List of the SIBs mapped to this SystemInformation message. There is no mapping information of SIB2; it is always present in the first SystemInformation message listed in the schedulingInfoList list. |
| si-HoppingConfigCommon Frequency hopping activation/deactivation for BR versions of SI messages and MPDCCH/PDSCH of paging. |
| si-Narrowband This field indicates the index of a narrowband used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.211 [21, 6.4.1] and TS 36.213 [23, 7.1.6]. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..[maxAvailNarrowBands-r13-1]) as specified in TS 36.211 [21]. |
| si-RepetitionPattern Indicates the radio frames within the SI window used for SI message transmission. Value everyRF corresponds to every radio frame, value every2ndRF corresponds to every 2 radio frames, and so on. The first transmission of the SI message is transmitted from the first radio frame of the SI window. |
| si-Periodicity, pos-si-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on. |
| si-TBS This field indicates the transport block size information used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.213 [23, Table 7.1.7.2.1-1] for a 6 PRB bandwidth and a QPSK modulation. |
| schedulingInfoList-BR Indicates additional scheduling information of SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h. |
| si-WindowLength, si-WindowLength-BR Common SI scheduling window for all SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on. In case si-WindowLength-BR-r13 is present and the UE is a BL UE or a UE in CE, the UE shall use si-WindowLength-BR-r13 and ignore the original field si-WindowLength (without suffix). UEs other than BL UEs or UEs in CE shall ignore the extension field si-WindowLength-BR-r13. |
| startSymbolBR For BL UEs and UEs in CE, indicates the OFDM starting symbol for any MPDCCH, PDSCH scheduled on the same cell except the PDSCH carrying SystemInformationBlockType1-BR, see TS 36.213 [23]. Values 1, 2, and 3 are applicable for dl-Bandwidth greater than 10 resource blocks. Values 2, 3, and 4 are applicable otherwise. |
| systemInfoValueTagList Indicates SI message specific value tags for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| systemInfoValueTagSI SI message specific value tag as specified in subclause 5.2.1.3. Common for all SIBs within the SI message other than MIB, SIB1, SIB10, SIB11, SIB12 and SIB14. |
| systemInfoValueTag Common for all SIBs other than MIB, MIB-MBMS, SIB1, SIB1-MBMS, SIB10, SIB11, SIB12 and SIB14. Change of MIB, MIB-MBMS, SIB1 and SIB1-MBMS is detected by acquisition of the corresponding message. |
| tdd-Config Specifies the TDD specific physical channel configurations. NOTE 2. |
| trackingAreaCode/trackingAreaCode-5GC A trackingAreaCode that is common for all the PLMNs listed. NOTE2. NOTE 5. |

NOTE 1:	The value the UE applies for parameter "Qqualmin" in TS 36.304 [4] depends on the q-QualMin fields signalled by E-UTRAN and supported by the UE. In case multiple candidate options are available, the UE shall select the highest priority candidate option according to the priority order indicated by the following table (top row is highest priority).

| q-QualMinRSRQ-OnAllSymbols | q-QualMinWB | Value of parameter "Qqualmin" in TS 36.304 [4] |
| --- | --- | --- |
| Included | Included | q-QualMinRSRQ-OnAllSymbols – (q-QualMin – q-QualMinWB) |
| Included | Not included | q-QualMinRSRQ-OnAllSymbols |
| Not included | Included | q-QualMinWB |
| Not included | Not included | q-QualMin |

NOTE 2:	E-UTRAN sets this field to the same value for all instances of SIB1 message that are broadcasted within the same cell.

NOTE 3:	E-UTRAN configures this field only in the BR version of SIB1 message.

NOTE 4:	E-UTRAN configures at most 6 EPC PLMNs in total (i.e. across all the PLMN lists except for PLMN lists in cellAccessRelatedInfoList-5GC in SIB1). E-UTRAN configures at most 6 5GC PLMNs in total (i.e. across all the PLMN lists in cellAccessRelatedInfoList-5GC in SIB1).

NOTE 5:	E-UTRAN configures only one value for this parameter per PLMN.

NOTE 6: E-UTRAN configures plmn-index only if the cellBarred is set to notBarred

<Cut until modified section>

### 6.3.5	Measurement information elements

<Cut until modified section>

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency and inter- RAT mobility.

MeasResults information element

-- ASN1START

MeasResults ::=						SEQUENCE {

measId								MeasId,

measResultPCell						SEQUENCE {

rsrpResult							RSRP-Range,

rsrqResult							RSRQ-Range

},

measResultNeighCells				CHOICE {

measResultListEUTRA					MeasResultListEUTRA,

measResultListUTRA					MeasResultListUTRA,

measResultListGERAN					MeasResultListGERAN,

measResultsCDMA2000					MeasResultsCDMA2000,

...,

measResultNeighCellListNR-r15			MeasResultCellListNR-r15

}																		OPTIONAL,

...,

[[	measResultForECID-r9				MeasResultForECID-r9			OPTIONAL

]],

[[	locationInfo-r10					LocationInfo-r10				OPTIONAL,

measResultServFreqList-r10			MeasResultServFreqList-r10		OPTIONAL

]],

[[	measId-v1250						MeasId-v1250					OPTIONAL,

measResultPCell-v1250				RSRQ-Range-v1250				OPTIONAL,

measResultCSI-RS-List-r12			MeasResultCSI-RS-List-r12		OPTIONAL

]],

[[	measResultForRSSI-r13				MeasResultForRSSI-r13			OPTIONAL,

measResultServFreqListExt-r13		MeasResultServFreqListExt-r13	OPTIONAL,

measResultSSTD-r13					MeasResultSSTD-r13				OPTIONAL,

measResultPCell-v1310				SEQUENCE {

rs-sinr-Result-r13					RS-SINR-Range-r13

}																	OPTIONAL,

ul-PDCP-DelayResultList-r13			UL-PDCP-DelayResultList-r13		OPTIONAL,

measResultListWLAN-r13				MeasResultListWLAN-r13			OPTIONAL

]],

[[	measResultPCell-v1360				RSRP-Range-v1360				OPTIONAL

]],

[[	measResultListCBR-r14				MeasResultListCBR-r14			OPTIONAL,

measResultListWLAN-r14				MeasResultListWLAN-r14			OPTIONAL

]],

[[	measResultServFreqListNR-r15		MeasResultServFreqListNR-r15	OPTIONAL,

measResultCellListSFTD-r15		MeasResultCellListSFTD-r15			OPTIONAL

]],

[[	measResultListBT-r15				MeasResultListBT-r15			OPTIONAL,

measResultListWLAN-r15			MeasResultListWLAN-r15			OPTIONAL,

heightUE-r15						BIT STRING (SIZE (15))		OPTIONAL

]]

}

MeasResultListEUTRA ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=	SEQUENCE {

physCellId							PhysCellId,

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdEUTRA,

trackingAreaCode					TrackingAreaCode,

plmn-IdentityList					PLMN-IdentityList2				OPTIONAL

}															OPTIONAL,

measResult							SEQUENCE {

rsrpResult							RSRP-Range						OPTIONAL,

rsrqResult							RSRQ-Range						OPTIONAL,

...,

[[	additionalSI-Info-r9				AdditionalSI-Info-r9		OPTIONAL

]],

[[	primaryPLMN-Suitable-r12			ENUMERATED {true}			OPTIONAL,

measResult-v1250					RSRQ-Range-v1250			OPTIONAL

]],

[[	rs-sinr-Result-r13					RS-SINR-Range-r13			OPTIONAL,

cgi-Info-v1310						SEQUENCE {

freqBandIndicator-r13				FreqBandIndicator-r11		OPTIONAL,

multiBandInfoList-r13				MultiBandInfoList-r11		OPTIONAL,

freqBandIndicatorPriority-r13		ENUMERATED {true}			OPTIONAL

}																OPTIONAL

]],

[[

measResult-v1360					RSRP-Range-v1360					OPTIONAL

]] ,

[[

cgi-Info-5GC-r15						SEQUENCE (SIZE (1..maxPLMN-r11)) OF		CellAccessRelatedInfo-5GC-r15									OPTIONAL								]]

}

}

MeasResultsIdle-r15	::= SEQUENCE {

measResultServingCell-r15					SEQUENCE {

rsrpResult-r15					RSRP-Range,

rsrqResult-r15					RSRQ-Range-r13

},

measResultNeighCells-r15		CHOICE {

idleMeasResultListEUTRA-r15		IdleMeasResultListEUTRA-r15,

...

}																	OPTIONAL,

...

}

IdleMeasResultListEUTRA-r15 ::=	SEQUENCE (SIZE (1..maxCellMeasIdle-r15)) OF IdleMeasResultEUTRA-r15

IdleMeasResultEUTRA-r15 ::=	SEQUENCE {

carrierFreq-r15						ARFCN-ValueEUTRA-r9,

physCellId-r15						PhysCellId,

measResult-r15						SEQUENCE {

rsrpResult-r15						RSRP-Range,

rsrqResult-r15						RSRQ-Range-r13

},

...

}

MeasResultServFreqListNR-r15 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF MeasResultServFreqNR-r15

MeasResultServFreqNR-r15 ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

measResultSCell-r15					MeasResultCellNR-r15				OPTIONAL,

measResultBestNeighCell-r15			MeasResultCellNR-r15				OPTIONAL,

...

}

MeasResultCellListNR-r15::=		SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultCellNR-r15

MeasResultCellNR-r15 ::=			SEQUENCE {

pci-r15								PhysCellIdNR-r15,

measResultCell-r15					MeasResultNR-r15,

measResultRS-IndexList-r15			MeasResultSSB-IndexList-r15				OPTIONAL,

...,

[[	cgi-Info-r15						CGI-InfoNR-r15				OPTIONAL

]]

}

MeasResultNR-r15 ::=				SEQUENCE {

rsrpResult-r15						RSRP-RangeNR-r15						OPTIONAL,

rsrqResult-r15						RSRQ-RangeNR-r15						OPTIONAL,

rs-sinr-Result-r15					RS-SINR-RangeNR-r15						OPTIONAL,

...

}

MeasResultSSB-IndexList-r15::=		SEQUENCE (SIZE (1..maxRS-IndexReport-r15)) OF MeasResultSSB-Index-r15

MeasResultSSB-Index-r15 ::=		SEQUENCE {

ssb-Index-r15						RS-IndexNR-r15,

measResultSSB-Index-r15				MeasResultNR-r15					OPTIONAL,

...

}

RS-IndexNR-r15 ::=			INTEGER (0.. maxRS-Index-1-r15)

MeasResultServFreqList-r10 ::=	SEQUENCE (SIZE (1..maxServCell-r10)) OF MeasResultServFreq-r10

MeasResultServFreqListExt-r13 ::=	SEQUENCE (SIZE (1..maxServCell-r13)) OF MeasResultServFreq-r13

MeasResultServFreq-r10 ::=			SEQUENCE {

servFreqId-r10						ServCellIndex-r10,

measResultSCell-r10					SEQUENCE {

rsrpResultSCell-r10					RSRP-Range,

rsrqResultSCell-r10					RSRQ-Range

}															OPTIONAL,

measResultBestNeighCell-r10			SEQUENCE {

physCellId-r10						PhysCellId,

rsrpResultNCell-r10					RSRP-Range,

rsrqResultNCell-r10					RSRQ-Range

}															OPTIONAL,

...,

[[	measResultSCell-v1250				RSRQ-Range-v1250	OPTIONAL,

measResultBestNeighCell-v1250		RSRQ-Range-v1250	OPTIONAL

]],

[[	measResultSCell-v1310				SEQUENCE {

rs-sinr-Result-r13					RS-SINR-Range-r13

}		OPTIONAL,

measResultBestNeighCell-v1310		SEQUENCE {

rs-sinr-Result-r13					RS-SINR-Range-r13

}		OPTIONAL

]]

}

MeasResultServFreq-r13 ::=			SEQUENCE {

servFreqId-r13						ServCellIndex-r13,

measResultSCell-r13					SEQUENCE {

rsrpResultSCell-r13					RSRP-Range,

rsrqResultSCell-r13					RSRQ-Range-r13,

rs-sinr-Result-r13					RS-SINR-Range-r13	OPTIONAL

}															OPTIONAL,

measResultBestNeighCell-r13			SEQUENCE {

physCellId-r13						PhysCellId,

rsrpResultNCell-r13					RSRP-Range,

rsrqResultNCell-r13					RSRQ-Range-r13,

rs-sinr-Result-r13					RS-SINR-Range-r13	OPTIONAL

}															OPTIONAL,

...,

[[	measResultBestNeighCell-v1360		SEQUENCE {

rsrpResultNCell-v1360				RSRP-Range-v1360

}														OPTIONAL

]]

}

MeasResultCSI-RS-List-r12 ::=	SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultCSI-RS-r12

MeasResultCSI-RS-r12 ::=		SEQUENCE {

measCSI-RS-Id-r12				MeasCSI-RS-Id-r12,

csi-RSRP-Result-r12				CSI-RSRP-Range-r12,

...

}

MeasResultListUTRA ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultUTRA

MeasResultUTRA ::=	SEQUENCE {

physCellId							CHOICE {

fdd									PhysCellIdUTRA-FDD,

tdd									PhysCellIdUTRA-TDD

},

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdUTRA,

locationAreaCode					BIT STRING (SIZE (16))			OPTIONAL,

routingAreaCode						BIT STRING (SIZE (8))			OPTIONAL,

plmn-IdentityList					PLMN-IdentityList2				OPTIONAL

}															OPTIONAL,

measResult							SEQUENCE {

utra-RSCP							INTEGER (-5..91)				OPTIONAL,

utra-EcN0							INTEGER (0..49)					OPTIONAL,

...,

[[	additionalSI-Info-r9				AdditionalSI-Info-r9				OPTIONAL

]],

[[	primaryPLMN-Suitable-r12			ENUMERATED {true}			OPTIONAL

]]

}

}

MeasResultListGERAN ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultGERAN

MeasResultGERAN ::=	SEQUENCE {

carrierFreq							CarrierFreqGERAN,

physCellId							PhysCellIdGERAN,

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdGERAN,

routingAreaCode						BIT STRING (SIZE (8))			OPTIONAL

}																		OPTIONAL,

measResult							SEQUENCE {

rssi								INTEGER (0..63),

...

}

}

MeasResultsCDMA2000 ::=				SEQUENCE {

preRegistrationStatusHRPD			BOOLEAN,

measResultListCDMA2000				MeasResultListCDMA2000

}

MeasResultListCDMA2000 ::=			SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultCDMA2000

MeasResultCDMA2000 ::=	SEQUENCE {

physCellId							PhysCellIdCDMA2000,

cgi-Info							CellGlobalIdCDMA2000				OPTIONAL,

measResult							SEQUENCE {

pilotPnPhase						INTEGER	(0..32767)				OPTIONAL,

pilotStrength						INTEGER (0..63),

...

}

}

MeasResultListWLAN-r13 ::=		SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultWLAN-r13

MeasResultListWLAN-r14 ::=		SEQUENCE (SIZE (1..maxWLAN-Id-Report-r14)) OF MeasResultWLAN-r13

MeasResultWLAN-r13 ::=	SEQUENCE {

wlan-Identifiers-r13					WLAN-Identifiers-r12,

carrierInfoWLAN-r13						WLAN-CarrierInfo-r13	OPTIONAL,

bandWLAN-r13							WLAN-BandIndicator-r13	OPTIONAL,

rssiWLAN-r13							WLAN-RSSI-Range-r13,

availableAdmissionCapacityWLAN-r13		INTEGER (0..31250)		OPTIONAL,

backhaulDL-BandwidthWLAN-r13			WLAN-backhaulRate-r12	OPTIONAL,

backhaulUL-BandwidthWLAN-r13			WLAN-backhaulRate-r12	OPTIONAL,

channelUtilizationWLAN-r13				INTEGER (0..255)		OPTIONAL,

stationCountWLAN-r13					INTEGER (0..65535)		OPTIONAL,

connectedWLAN-r13						ENUMERATED {true}		OPTIONAL,

...

}

MeasResultListCBR-r14 ::=			SEQUENCE (SIZE (1..maxCBR-Report-r14)) OF MeasResultCBR-r14

MeasResultCBR-r14 ::=	SEQUENCE {

poolIdentity-r14		SL-V2X-TxPoolReportIdentity-r14,

cbr-PSSCH-r14			SL-CBR-r14,

cbr-PSCCH-r14			SL-CBR-r14				OPTIONAL

}

MeasResultForECID-r9 ::=		SEQUENCE {

ue-RxTxTimeDiffResult-r9				INTEGER (0..4095),

currentSFN-r9							BIT STRING (SIZE (10))

}

PLMN-IdentityList2 ::=				SEQUENCE (SIZE (1..5)) OF PLMN-Identity

AdditionalSI-Info-r9 ::=			SEQUENCE {

csg-MemberStatus-r9				ENUMERATED {member}				OPTIONAL,

csg-Identity-r9						CSG-Identity						OPTIONAL

}

MeasResultForRSSI-r13 ::=			SEQUENCE {

rssi-Result-r13							RSSI-Range-r13,

channelOccupancy-r13					INTEGER (0..100),

...

}

UL-PDCP-DelayResultList-r13 ::=		SEQUENCE (SIZE (1..maxQCI-r13)) OF UL-PDCP-DelayResult-r13

UL-PDCP-DelayResult-r13 ::=			SEQUENCE {

qci-Id-r13							ENUMERATED {qci1, qci2, qci3, qci4, spare4, spare3, spare2, spare1},

excessDelay-r13						INTEGER (0..31),

...

}

CGI-InfoNR-r15 ::=					SEQUENCE {

plmn-IdentityInfoList-r15			PLMN-IdentityInfoListNR-r15			OPTIONAL,

ran-NotificationAreaCode-r15		RAN-NotificationAreaCode-r15		OPTIONAL,

frequencyBandList-15				MultiFrequencyBandListNR-r15		OPTIONAL,

noSIB1-r15							ENUMERATED {true}					OPTIONAL,

...

}

CellIdentityNR-r15 ::=					BIT STRING (SIZE (36))

PLMN-IdentityList-r15 ::=				SEQUENCE (SIZE (1..maxPLMN-NR-r15)) OF PLMN-Identity

PLMN-IdentityInfoListNR-r15 ::=		SEQUENCE (SIZE (1..maxPLMN-NR-r15)) OF PLMN-IdentityInfoNR-r15

PLMN-IdentityInfoNR-r15 ::=			SEQUENCE {

plmn-IdentityList-r15				PLMN-IdentityList-r15,

trackingAreaCode-r15				TrackingAreaCodeNR-r15			OPTIONAL,

cellIdentity-r15					CellIdentityNR-r15

}

RAN-NotificationAreaCode-r15 ::=		BIT STRING (SIZE (6))

TrackingAreaCodeNR-r15 ::=				BIT STRING (SIZE (24))

-- ASN1STOP

<Cut until modified section>

#### –	ReportConfigEUTRA

The IE ReportConfigEUTRA specifies criteria for triggering of an E-UTRA measurement reporting event. The E-UTRA measurement reporting events concerning CRS are labelled AN with N equal to 1, 2 and so on.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/ PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/ PSCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell.

The E-UTRA measurement reporting events concerning CSI-RS are labelled CN with N equal to 1 and 2.

Event C1:	CSI-RS resource becomes better than absolute threshold;

Event C2:	CSI-RS resource becomes amount of offset better than reference CSI-RS resource.

The E-UTRA measurement reporting events concerning CBR are labelled VN with N equal to 1 and 2.

Event V1:	CBR becomes larger than absolute threshold;

Event V2:	CBR becomes smaller than absolute threshold.

The E-UTRA reporting events concerning Aerial UE height are labelled HN with N equal to 1 and 2.

Event H1:	Aerial UE height becomes higher than absolute threshold;

Event H2:	Aerial UE height becomes lower than absolute threshold.

ReportConfigEUTRA information element

-- ASN1START

ReportConfigEUTRA ::=				SEQUENCE {

triggerType							CHOICE {

event								SEQUENCE {

eventId								CHOICE {

eventA1								SEQUENCE {

a1-Threshold						ThresholdEUTRA

},

eventA2								SEQUENCE {

a2-Threshold						ThresholdEUTRA

},

eventA3								SEQUENCE {

a3-Offset							INTEGER (-30..30),

reportOnLeave						BOOLEAN

},

eventA4								SEQUENCE {

a4-Threshold						ThresholdEUTRA

},

eventA5								SEQUENCE {

a5-Threshold1						ThresholdEUTRA,

a5-Threshold2						ThresholdEUTRA

},

...,

eventA6-r10							SEQUENCE {

a6-Offset-r10						INTEGER (-30..30),

a6-ReportOnLeave-r10				BOOLEAN

},

eventC1-r12							SEQUENCE {

c1-Threshold-r12					ThresholdEUTRA-v1250,

c1-ReportOnLeave-r12				BOOLEAN

},

eventC2-r12							SEQUENCE {

c2-RefCSI-RS-r12					MeasCSI-RS-Id-r12,

c2-Offset-r12						INTEGER (-30..30),

c2-ReportOnLeave-r12				BOOLEAN

},

eventV1-r14							SEQUENCE {

v1-Threshold-r14					SL-CBR-r14

},

eventV2-r14							SEQUENCE {

v2-Threshold-r14					SL-CBR-r14

},

eventH1-r15							SEQUENCE	 {

h1-ThresholdOffset-r15				INTEGER (0..300),

h1-Hysteresis-15						INTEGER (1..16)

},

eventH2-r15							SEQUENCE	 {

h2-ThresholdOffset-r15				INTEGER (0..300),

h2-Hysteresis-15						INTEGER (1..16)

}

},

hysteresis							Hysteresis,

timeToTrigger						TimeToTrigger

},

periodical								SEQUENCE {

purpose									ENUMERATED {

reportStrongestCells, reportCGI}

}

},

triggerQuantity						ENUMERATED {rsrp, rsrq},

reportQuantity						ENUMERATED {sameAsTriggerQuantity, both},

maxReportCells						INTEGER (1..maxCellReport),

reportInterval						ReportInterval,

reportAmount						ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

...,

[[	si-RequestForHO-r9					ENUMERATED {setup}		OPTIONAL,	-- Cond reportCGI

ue-RxTxTimeDiffPeriodical-r9		ENUMERATED {setup}		OPTIONAL	-- Need OR

]],

[[	includeLocationInfo-r10				ENUMERATED {true}		OPTIONAL,	-- Need OR

reportAddNeighMeas-r10				ENUMERATED {setup}		OPTIONAL	-- Need OR

]],

[[	alternativeTimeToTrigger-r12		CHOICE {

release								NULL,

setup								TimeToTrigger

}													OPTIONAL,	-- Need ON

useT312-r12							BOOLEAN			OPTIONAL,	-- Need ON

usePSCell-r12						BOOLEAN			OPTIONAL,	-- Need ON

aN-Threshold1-v1250					RSRQ-RangeConfig-r12		OPTIONAL,	-- Need ON

a5-Threshold2-v1250					RSRQ-RangeConfig-r12		OPTIONAL,	-- Need ON

reportStrongestCSI-RSs-r12			BOOLEAN			OPTIONAL,	-- Need ON

reportCRS-Meas-r12					BOOLEAN			OPTIONAL,	-- Need ON

triggerQuantityCSI-RS-r12			BOOLEAN			OPTIONAL		-- Need ON

]],

[[	reportSSTD-Meas-r13					BOOLEAN			OPTIONAL,		-- Need ON

rs-sinr-Config-r13					CHOICE {

release								NULL,

setup								SEQUENCE {

triggerQuantity-v1310				ENUMERATED {sinr}		OPTIONAL,	-- Need ON

aN-Threshold1-r13					RS-SINR-Range-r13		OPTIONAL,	-- Need ON

a5-Threshold2-r13					RS-SINR-Range-r13		OPTIONAL,	-- Need ON

reportQuantity-v1310				ENUMERATED {rsrpANDsinr, rsrqANDsinr, all}

}

}																OPTIONAL,	-- Need ON

useWhiteCellList-r13				BOOLEAN						OPTIONAL,	-- Need ON

measRSSI-ReportConfig-r13			MeasRSSI-ReportConfig-r13	OPTIONAL,	-- Need ON

includeMultiBandInfo-r13			ENUMERATED {true}			OPTIONAL,	-- Cond reportCGI

ul-DelayConfig-r13					UL-DelayConfig-r13			OPTIONAL	-- Need ON

]],

[[	ue-RxTxTimeDiffPeriodicalTDD-r13	BOOLEAN						OPTIONAL	-- Need ON

]],

[[

purpose-v1430			ENUMERATED {reportLocation, sidelink, spare2, spare1}

OPTIONAL	-- Need ON

]],

[[

maxReportRS-Index-r15		INTEGER (0..maxRS-IndexReport-r15)	OPTIONAL	-- Need ON

]],

[[	includeBT-Meas-r15				IncludeBT-Meas-r15			OPTIONAL,	-- Need OR

includeWLAN-Meas-r15				IncludeWLAN-Meas-r15			OPTIONAL,		-- Need OR

numberOfTriggeringCells-r15			INTEGER	(2..maxCellReport)	OPTIONAL,		-- Cond a3a4a5

a4-a5-ReportOnLeave-r15				BOOLEAN						OPTIONAL		-- Cond a4a5

]],

[[

report5GCInfo-r15				ENUMERATED {true}			OPTIONAL	-- Cond reportCGI

]]

}

RSRQ-RangeConfig-r12 ::=			CHOICE {

release								NULL,

setup								RSRQ-Range-v1250

}

ThresholdEUTRA ::=					CHOICE{

threshold-RSRP						RSRP-Range,

threshold-RSRQ						RSRQ-Range

}

ThresholdEUTRA-v1250 ::=			CSI-RSRP-Range-r12

MeasRSSI-ReportConfig-r13 ::=	SEQUENCE {

channelOccupancyThreshold-r13			RSSI-Range-r13				OPTIONAL	-- Need OR

}

IncludeBT-Meas-r15 ::=		SEQUENCE{

bt-MeasConfig-r15			ENUMERATED {setup},

bt-MeasConfigNameList-r15		BT-MeasConfigNameList-r15			OPTIONAL

}

IncludeWLAN-Meas-r15 ::=		SEQUENCE{

wlan-MeasCconfig-r15			ENUMERATED {setup},

wlan-MeasConfigNameList-r15		WLAN-MeasConfigNameList-r15			OPTIONAL

}

-- ASN1STOP

| ReportConfigEUTRA field descriptions |
| --- |
| a3-Offset/ a6-Offset/ c2-Offset Offset value to be used in EUTRA measurement report triggering condition for event a3/ a6/ c2. The actual value is field value * 0.5 dB. |
| alternativeTimeToTrigger Indicates the time to trigger applicable for cells specified in altTTT-CellsToAddModList of the associated measurement object, if configured |
| aN-ThresholdM/ cN-ThresholdM Threshold to be used in EUTRA measurement report triggering condition for event number aN/ cN. If multiple thresholds are defined for event number aN/ cN, the thresholds are differentiated by M. E-UTRAN configures aN-Threshold1 only for events A1, A2, A4, A5 and a5-Threshold2 only for event A5. |
| bt-MeasConfig The value true indicates that the UE shall include Bluetooth measurements, if available. |
| bt-MeasConfigNameList If configured, the UE only performs Bluetooth measurements according to names identified in this list. For each name, it refers to LOCAL NAME defined in Bluetooth specification [91]. |
| c1-ReportOnLeave/ c2-ReportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a CSI-RS resource in csi-RS-TriggeredList, as specified in 5.5.4.1. |
| c2-RefCSI-RS Identity of the CSI-RS resource from the measCSI-RS-ToAddModList of the associated measObject, to be used as the reference CSI-RS resource in EUTRA measurement report triggering condition for event c2. |
| channelOccupancyThreshold RSSI threshold which is used for channel occupancy evaluation. |
| eventId Choice of E-UTRA event triggered reporting criteria. EUTRAN may set this field to eventC1 or eventC2 only if measDS-Config is configured in the associated measObject with one or more CSI-RS resources. The eventC1 and eventC2 are not applicable for the eventId if RS-SINR is configured as triggerQuantity or reportQuantity. |
| h1-Hysteresis, h2-Hysteresis This parameter is used within the entry and leave condition of an event triggered reporting condition for event H1. The actual value is field value. If this field is configured UE shall ignore parameter hysteresis. |
| h1-ThresholdOffset, h2-ThresholdOffset An offset value to heightThresholdReferenceValue to obtain the threshold to be used in EUTRA height report triggering condition for event H1 and H2. The value for h1-ThresholdOffset and h2-ThresholdOffset is expressed in meters such that granularity is 2meters. Value 0 corresponds to offset value 0m, value 1 corresponds to offset value 2m, value 3 correspond to offset value 4m, and so on. |  |
| includeMultiBandInfo If this field is present, the UE shall acquire and include multi band information in the measurement report. |  |
| maxReportCells Max number of cells, excluding the serving cell, to include in the measurement report concerning CRS, and max number of CSI-RS resources to include in the measurement report concerning CSI-RS. |
| measRSSI-ReportConfig If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy and ignore the triggerQuantity, reportQuantity and maxReportCells fields. E-UTRAN only sets this field to true when setting triggerType to periodical and purpose to reportStrongestCells. |
| numberOfTriggeringCells Indicates the number of cells detected that are required to fulfill an event for a measurement report to be triggered. This field is set only for the events concerning neighbor cells, i.e. eventA3, eventA4, eventA5. |
| report5GCInfo Indicates that the UE shall include the cgi-Info-5GC containing the entire CellAccessRelatedInfo-5GC for each PLMN in the measurement report as specified in sub clause 5.5.3.1 |
| reportAmount Number of measurement reports applicable for triggerType event as well as for triggerType periodical. In case purpose is set to reportCGI or reportSSTD-Meas is set to true, only value 1 applies. |
| reportCRS-Meas Inidicates that UE shall include rsrp, rsrq together with csi-rsrp in the measurement report, if possible. |
| reportOnLeave/ a6-ReportOnLeave/ a4-a5-ReportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1. |
| reportQuantity The quantities to be included in the measurement report. The value both means that both the rsrp and rsrq quantities are to be included in the measurement report. The value rsrpANDsinr and rsrqANDsinr mean that both rsrp and rs-sinr quantities, and both rsrq and rs-sinr quantities are to be included respectively in the measurement report. The value all means that rsrp, rsrq and rs-sinr are to be included in the measurement report. In case triggerQuantityCSI-RS is included, only value sameAsTriggerQuantity applies. If reportQuantity-v1310 is configured, the UE only considers this extension (and ignores reportQuantity i.e. without suffix). |
| reportSSTD-Meas If this field is set to true, the UE shall measure SSTD between the PCell and the PSCell as specified in TS 36.214 [48] and ignore the triggerQuantity, reportQuantity and maxReportCells fields. E-UTRAN only sets this field to true when setting triggerType to periodical and purpose to reportStrongestCells. |
| reportStrongestCSI-RSs Indicates that periodical CSI-RS measurement report is performed. EUTRAN configures value TRUE only if measDS-Config is configured in the associated measObject with one or more CSI-RS resources. |
| si-RequestForHO The field applies to the reportCGI functionality, and when the field is included, the UE is allowed to use autonomous gaps in acquiring system information from the neighbour cell, applies a different value for T321, and includes different fields in the measurement report. EUTRAN does not configure the field if reportConfig is linked to a measObject set to measObjectNR. |
| ThresholdEUTRA For RSRP: RSRP based threshold for event evaluation. The actual value is field value – 140 dBm. For RSRQ: RSRQ based threshold for event evaluation. The actual value is (field value – 40)/2 dB. For RS-SINR: RS-SINR based threshold for event evaluation. The actual value is (field value -46)/2 dB. For CSI-RSRP: CSI-RSRP based threshold for event evaluation. The actual value is field value – 140 dBm. EUTRAN configures the same threshold quantity for all the thresholds of an event. |
| timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report. |
| triggerQuantity The quantity used to evaluate the triggering condition for the event concerning CRS. EUTRAN sets the value according to the quantity of the ThresholdEUTRA for this event. The values rsrp, rsrq and sinr correspond to Reference Signal Received Power (RSRP), Reference Signal Received Quality (RSRQ) and Reference Signal Signal to Noise and Interference Ratio (RS-SINR), see TS 36.214 [48]. If triggerQuantity-v1310 is configured, the UE only considers this extension (and ignores triggerQuantity i.e. without suffix). |
| triggerQuantityCSI-RS The quantity used to evaluate the triggering condition for the event concerning CSI-RS. The value TRUE corresponds to CSI Reference Signal Received Power (CSI-RSRP), see TS 36.214 [48]. E-UTRAN configures value TRUE if and only if the measurement reporting event concerns CSI-RS. |
| ue-RxTxTimeDiffPeriodical If this field is present, the UE shall perform UE Rx-Tx time difference measurement reporting and ignore the fields triggerQuantity, reportQuantity and maxReportCells. If the field is present, the only applicable values for the corresponding triggerType and purpose are periodical and reportStrongestCells respectively. |
| ue-RxTxTimeDiffPeriodicalTDD If this field is set to TRUE, the UE shall perform UE Rx-Tx time difference measurement reporting according to EUTRAN TDD UE Rx-Tx time difference report mapping in TS 36.133 [16]. If the field is configured, the ue-RxTxTimeDiffPeriodical shall be configured. The field is applicable for TDD only. |
| usePSCell If this field is set to TRUE the UE shall use the PSCell instead of the PCell. E-UTRAN configures value TRUE only for events A3 and A5, see 5.5.4.4 and 5.5.4.6. |
| useT312 If value TRUE is configured, the UE shall use the timer T312 with the value t312 as specified in the corresponding measObject. If the corresponding measObject does not include the timer T312 then the timer T312 is considered as not configured. E-UTRAN configures value TRUE only if triggerType is set to event. |
| useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1. E-UTRAN does not configure the field for events A1, A2, C1 and C2. |
| ul-DelayConfig If the field is present, E-UTRAN configures UL PDCP Packet Delay per QCI measurement and the UE shall ignore the fields triggerQuantity and maxReportCells. The applicable values for the corresponding triggerType and reportInterval are periodical and (one of the) ms1024, ms2048, ms5120 or ms10240 respectively.The reportInterval indicates the periodicity for performing and reporting of UL PDCP Delay per QCI measurement as specified in TS 36.314 [71]. |
| wlan-MeasCconfig The value true indicates that the UE shall include WLAN measurements, if available. |
| wlan-MeasConfigNameList If configured, the UE only performs WLAN measurements according to names identified in this list. For each name, it refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 [67]. |

| Conditional presence | Explanation |
| --- | --- |
| a3a4a5 | This field is optional, need OR, in case eventId is set to eventA3 or eventA4 or eventA5; otherwise, this field is not present and the UE shall delete any existing value of this field. |
| a4a5 | This field is optional, need OR, in case eventId is set to eventA4 or eventA5; otherwise, this field is not present and the UE shall delete any existing value of this field. |
| reportCGI | The field is optional, need OR, in case purpose is included and set to reportCGI; otherwise the field is not present and the UE shall delete any existing value for this field. |