3GPP TSG-RAN WG2 Meeting #103bis	R2-1813890

Chengdu, China, 8 – 12 October 2018

Agenda item:	11.6

Source: 	Huawei, HiSilicon

Title: 	Considerations on architecture and deployment scenarios of NTN

Document for:	Discussion and Decision

Introduction

In the RAN plenary #80 meeting, a new RAN3 SI on “solutions evaluation for NR to support Non Terrestrial Network” was approved [1]. The objectives of the SI for Layer 2 and above, and RAN architecture are reported as follows:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2]

- Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1]

- Architecture: Identify needs for the 5G’s Radio Access Network architecture to support non-terrestrial networks (e.g. handling of network identities) [RAN3]

  - Paging: procedure adaptations in case of moving satellite foot prints or cells

In this contribution, we provide our consideration on architecture and deployment scenarios of NTN.

Discussion

As defined in [1], the possible NTN architectures are shown below:

Table 1: 5G system elements mapping in NTN architecture

| 5G elements - NTN elements mapping |  |  |  |
| --- | --- | --- | --- |
| NTN architecture options | NTN Terminal | Space or HAPS | NTN Gateway |
| A1: access network serving UEs via bentpipe satellite/aerial | UE | Remote Radio Head (Bent pipe relay of Uu radio interface signals) | gNB |
| A2: access network serving UEs with gNB on board satellite/aerial | UE | gNB or Relay Node functions | Router interfacing to Core network |
| A3: access network serving Relay Nodes via bent pipe satellite/aerial | Relay Node | Remote Radio Head (Bent pipe relay of Uu radio interface signals) | gNB |
| A4: access network serving Relay Nodes with gNB on board satellite/aerial | Relay Node | gNB or Relay Node functions | Router interfacing to Core network |

In A1 architecture, the satellite/HAPS implements radio frequency conversion, analogue filtering and amplification, and sends back to Earth what goes into the conduit with only amplification and a shift from uplink to downlink frequency. In A2 architecture, the satellite/HAPS implements gNB functions or gNB-DU functions. In A3 architecture, the satellite or the aerial will relay a "Satellite friendly" NR signal between the gNB and the Relay Nodes in a transparent manner. In A4 architecture, the satellite or the aerial includes full or part of a gNB to generate/receive a "Satellite friendly" NR signal to/from the Relay Nodes.

For A3 and A4 architecture, the link from UE to CU/gNB will need at least two relay links, which is more complex than the single access link in A1 and A2. Furthermore, the high mobility and high propagation delay character of NTN increase the difficulty to achieve multiple-hop access. Hence, compared to A1 and A2, the deployment of A3 and A4 are more complex and the architecture A3 and A4 have more specification impact. So, it is proposed that A3 and A4 architecture should have low priority in R16.

Proposal 1: A3 and A4 architecture should have low priority in R16.

The following table shows the deployment scenarios of NTN as defined in [1]. The deployment scenario for A1 architecture is D2. D2 is based on GEO at 35786km. The deployment scenario for A2 architecture is D3 and D5. D3 is based on Non-GEO down to 600km and D5 is based on UAS between 8km and 50km including HAPS. For Non-GEO, its motion is very rapid and will result in paging and handover issues as analysed in [2]. This will bring large specification impact. However, GEO and UAS does not move usually or the mobility speed is low, and then the NR paging and handover mechanism can work directly. What’s more, due to the lower altitude, the propagation delay of UAS will lower than GEO, and this reduce the specification impact further. The only problem of D5 may be how to solve the backhaul from UAS to 5GC. Therefore, D5 should have higher priority in R16.

Proposal 2: D5 should have higher priority in R16.

Table 2: 5G system elements mapping in NTN architecture

| Main attributes | Deployment-D1 | Deployment-D2 | Deployment-D3 | Deployment-D4 | Deployment-D5 |
| --- | --- | --- | --- | --- | --- |
| Platform orbit and altitude | GEO at 35 786 km | GEO at 35 786 km | Non-GEO down to 600 km | Non-GEO down to 600 km | UAS between 8 km and 50 km including HAPS |
| Carrier Frequency on the link between Air / space-borne platform and UE | Around 20 GHz for DL Around 30 GHz for UL (Ka band) | Around 2 GHz for both DL and UL (S band) | Around 2 GHz for both DL and UL (S band) | Around 20 GHz for DL Around 30 GHz for UL (Ka band) | Below and above 6 GHz |
| Beam pattern | Earth fixed beams | Earth fixed beams | Moving beams | Earth fixed beams | Earth fixed beams |
| Duplexing | FDD | FDD | FDD | FDD | FDD |
| Channel Bandwidth (DL + UL) | Up to 2 * 800 MHz | Up to 2 * 20 MHz | Up to 2 * 20MHz | Up to 2 * 800 MHz | Up to 2 * 80 MHz in mobile use and 2 * 1800 MHz in fixed use |
| NTN architecture options (See clause 4) | A3 | A1 | A2 | A4 | A2 |
| NTN Terminal type | Very Small Aperture Terminal (fixed or mounted on Moving Platforms) implementing a relay node | Up to 3GPP class 3 UE [2] | Up to 3GPP class 3 UE [2] | Very Small Aperture Terminal (fixed or mounted on Moving Platforms) implementing a Relay node | Up to 3GPP class 3 UE [2] Also Very Small Aperture Terminal |
| NTN terminal Distribution | 100% Outdoors | 100% Outdoors | 100% Outdoors | 100% Outdoors | Indoor and Outdoor |
| NTN terminal Speed | up to 1000 km/h (e.g. aircraft) | up to 1000 km/h (e.g. aircraft) | up to 1000 km/h (e.g. aircraft) | up to 1000 km/h (e.g. aircraft) | up to 500 km/h (e.g. high speed trains) |
| Main rationales | GEO based indirect access via relay node | GEO based direct access | Non-GEO based direct access | Non-GEO based indirect access via relay node | Support of low latency services for 3GPP mobile UEs, both indoors and outdoors |
| Supported Uses cases, see clause 4 | 1/ eMBB: multi-connectivity, fixed cell connectivity, mobile cell connectivity, network resilience, Trunking, edge network delivery, Mobile cell hybrid connectivity, Direct To Node multicast/ broadcast | 1/eMBB: Regional area public safety, Wide area public safety, Direct to mobile broadcast, Wide area IoT service | 1/eMBB: Regional area public safety, Wide area public safety, Wide area IoT service | 1/ eMBB: multi-homing, fixed cell connectivity, mobile cell connectivity, network resilience, Trunking, Mobile cell hybrid connectivity | 1/ eMBB: Hot spot on demand |

Conclusion

In this contribution, we considered the architecture and deployment scenarios of NTN, and we get the following observation and proposals:

Proposal 1: A3 and A4 architecture should have low priority in R16.

Proposal 2: D5 should have higher priority in R16.

References

[1] TR 38.811, Study on New Radio (NR) to support non terrestrial networks.

[2]	RP-181370, Consideration on the impact of rapid motion of satellite in NTN, Huawei, HiSilicon, RAN2 #103bis, 8-12 October 2018, Chengdu, China