3GPP TSG-RAN WG2 Meeting #103bis	R2-1814292

Chengdu, China, 08-12 October 2018	(revision of R2-1812323)

Agenda item:	11.2.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On Channel Access Priority Classes in NR-U

WID/SID:	FS_NR_unlic - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Channel Access Priority Classes for NR-U were discussed in RAN2#AH-1807. Some discussions were related to determination of the channel access priority class for UL transmissions using configured grant, and to mapping of channel access priority classes to standardized 5QI-values. Also, there were discussions related to UE behavior when data with different channel access priorities are mapped to the same MAC PDU, as well as the channel access priority class to be used during the random access procedure [1] [2]. NR-U channel access priority was further discussed in RAN2#103 based on [3]. However, no progress was possible on this topic as some companies expressed concerns that the concept of channel access priority class has not yet been specified for NR-U, while other companies thinks definition of channel access priority classes is a RAN1 issue.

In this contribution, we further discuss the concept of channel access priority class, provide some background information on RAN and LAA discussions, and propose a baseline definition of channel access priority class to be adopted for future discussions in RAN WG2.

It should be noted that how to configure the Channel Access Priority Class that a UE should use for NR-U configured grants is for RAN2 to discuss (and decide). Also, it is up to RAN2 to discuss which Channel Access priority Class the UE should use when multiplexing multiple logical channels in a MAC PDU transmitted using NR-U configured grants.

# 2	Discussion

Based on recent discussions, 3GPP RAN has decided to create a new RAN1 TS in 37 series (37.213), based on section 15 of TS 36.213, and specifying the physical layer procedures for shared spectrum channel access [4].  This also include the definition of channel access priority classes. The main motivation to introduce a different set of specifications is to exclude the use of unlicensed spectrum in the IMT-2020 self-evaluations from 3GPP. Basically, the set of specifications capturing standard impact specific to 3GPP-based access to unlicensed spectrum will not be submitted to ITU-R as part of the IMT-2020 submission.

However, one additional motivation to have a separate set of specifications is that many aspects of physical layer procedures for shared spectrum channel access are dictated by global regulation in e.g. the 5 GHz unlicensed spectrum. Therefore, it is somehow expected that several parts of those specifications will eventually be applicable also to 5G/NR, hence a single specification series handling physical layer procedures for shared spectrum channel access.

Observation 1: one of the reasons to introduce a separate RAN1 TS in 37 series (37.213) to specify the physical layer procedures for shared spectrum channel access, is that those procedures are determined by global regulations and as such are expected to present several similarities between LTE (LAA) and NR-U.

Specific to channel access priority classes, it should be further noted that the channel access priority classes standardized for LAA in 37.213 are essentially based on the priority classes specified in ETSI standards [5]. This is to guarantee fair co-existence with WiFi radio access technology which is also making use of priority classes defined in [5]. It can therefore be expected that to guarantee fair co-existence with other Radio Access Technologies operating in the 5 GHz unlicensed band, including WiFi, NR-U will need to adopt the same channel access priority classes definition as introduced for LAA in TS 37.213.

Observation 2: the channel access priority classes introduced for LAA in TS 37.213 were defined based on the priority classes specified in ETSI standards, with the scope of guaranteeing fair co-existence with other RATs operating in the 5 GHz unlicensed band (including WiFi).

As some companies are proposing to introduce more channel access priority classes into NR-U, we also see its benefit especially in a stand-alone (SA) deployment. However, the real use cases are not clear at the moment and the impact to coexistence fairness is not evaluated. We think it could be studied in later releases.

Proposal 1: the channel access priority classes specified in TS 37.213 for LAA are also adopted for NR-U SI as a baseline. RAN WG2 can assume these channel access priority classes for future discussions. Further enhancements (e.g., introducing additional channel access priority classes) can be studied in later releases.

If RAN WG2 cannot agree on Proposal 1 without direct confirmation from RAN WG1, it is proposed to send an LS to RAN WG1 asking if RAN WG2 can assume for NR-U the same channel access priority classes as defined for LAA in TS 37.213.

# 3	Summary

In this paper we discussed the channel access priority classes for NR-U and made the following observations and proposal:

Observation 1: one of the reasons to introduce a separate RAN1 TS in 37 series (37.213) to specify the physical layer procedures for shared spectrum channel access, is that those procedures are determined by global regulations and as such are expected to present several similarities between LTE (LAA) and NR-U.

Observation 2: the channel access priority classes introduced for LAA in TS 37.213 were defined based on the priority classes specified in ETSI standards, with the scope of guaranteeing fair co-existence with other RATs operating in the 5 GHz unlicensed band (including WiFi).

Proposal 1: the channel access priority classes specified in TS 37.213 for LAA are also adopted for NR-U SI as a baseline. RAN WG2 can assume these channel access priority classes for future discussions. Further enhancements (e.g., introducing additional channel access priority classes) can be studied in later releases.

If RAN WG2 cannot agree on Proposal 1 without direct confirmation from RAN WG1, it is proposed to send an LS to RAN WG1 asking if RAN WG2 can assume for NR-U the same channel access priority classes as defined for LAA in TS 37.213.

# References

- R2-1810212, “MAC impacts due to NR-based access to unlicensed spectrum”, Nokia, Nokia Shanghai Bell

- R2-1810402, “NR-U Channel access priority”, Ericsson

- R2-1812373, “NR-U Channel access priority”, Ericsson

- RP-172809,  “Details of specification handling of 3GPP-based access to unlicensed spectrum in Rel-15”, Huawei, Ericsson, HiSilicon, Intel, Nokia, Nokia Shanghai Bell, Qualcomm, Samsung

- ETSI EN 301 893, 5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU