3GPP TSG-RAN WG2#103bis	R2-1814549

Chengdu, China, 8th – 12th October 2018

Agenda Item:	10.5.4

Source:	Ericsson

Title:	Inter-node RRC messages for NR-DC

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss the introduction of inter-node RRC messages for NR-DC that are not been discussed yet.

# Discussion

One of the remaining issues that has not been addressed so far regarding NR-DC, is how and where inter-node RRC messages (i.e., sent either across the Xn- or the NG-interface) will be specified and handled in order to perform the procedures described in 3GPP TS 37.340.

As a first point, is good to highlight that in the RAN2 AH1801 meeting, the outcome from the email discussion [100#31][NR] [1] has been that inter-node RRC messages for EN-DC will be specified in the NR RRC (i.e., TS 38.331).

Inter-node RRC messages for EN-DC are specified in NR RRC (38.331).

Along this line, in order to have a sort of harmonization between EN-DC and the other MR-DC options, in [4] we propose to take the EN-DC inter-node RRC message as baseline for NGEN-DC and NE-DC without introduction specific messages for each of the MR-DC architecture options.

Therefore, following the principle to reduce the standardization effort and have a part of the specification that is general/harmonize for all the MR-DC options (i.e., that now include also NR-DC), it is straightforward to extend the inter-node RRC already present in 3GPP TS 38.331 in order to support also NR-DC.

Existing inter-node RRC message in NR RRC (38.331) are extended to support NR-DC.

RAN2 should consider the CR in [2] to be included in TS 37.340.

In order to start also stage-3 work for late drop MR-DC options, we have prepared also a draft CR for 38.331 to be considered for inclusion in to running CR for late drop.

RAN2 should consider the draft CR in [3] to be included in a running CR for TS 38.331 to introduce late drop functionality.

# Conclusion

In section 2 we made the following observations:

Observation 1	Inter-node RRC messages for EN-DC are specified in NR RRC (38.331).

Based on the discussion in section 2 we propose the following:

Proposal 1	Existing inter-node RRC message in NR RRC (38.331) are extended to support NR-DC.

Proposal 2	RAN2 should consider the CR in [2] to be included in TS 37.340.

Proposal 3	RAN2 should consider the draft CR in [3] to be included in a running CR for TS 38.331 to introduce late drop functionality.

# References

Chairman notes, RAN2 #AH1801, January 2018

R2-1814551, Inter-node RRC messages for MR-DC (37.340), RAN2 #103bis, October 2018.

R2-1814550, Inter-node RRC messages for NR-DC (38.331), RAN2 #103bis, October 2018.

R2-1814547, Inter-node RRC messages for NE-DC, RAN2 #103bis, October 2018