3GPP TSG-RAN2 Meeting #103bis 	R2-1814560

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0062 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCG failure handling in case of NE-DC and NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In case of NE-DC and NR-DC, there is need for further improvements compared to EN-DC in minimizing the service interruption time and maximizing the availability for URLLC services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE suspends MCG transmissions and sends MCG Failure Information if MCG RLF is detected when NE-DC or NR-DC is active and split SRB is configured. In addition, UE falls back to the RRC connection re-establishment procedure and does not try to report MCG Failure Information to SCG in case of simultaneous or consecutive RLF detections on MCG and SCG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequence would be that UE has to do re-establishment and experience service interruption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 SCG/MCG failure handling |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 7.7	SCG/MCG failure handling

RLF is declared separately for the MCG and for the SCG.

In EN-DC and NGEN-DC, if radio link failure is detected for MCG, the UE initiates the RRC connection re-establishment procedure.

In EN-DC and NGEN-DC, the following SCG failure cases are supported:

-	SCG RLF;

-	SN change failure;

-	SCG configuration failure (only for messages on SRB3);

-	SCG RRC integrity check failure (on SRB3).

In NE-DC and NR-DC, MCG failure due to MCG radio link failure is supported when split SRB is configured. The following MCG failure cases are supported:

- MCG RLF;

- MCG configuration with sync failure;

- MCG RRC integrity check failure (on SRB1/2);

- Mobility from NR/E-UTRA failure.

In EN-DC and NGEN-DC, upon SCG failure the UE suspends SCG transmissions for all radio bearers and reports the SCG Failure Information to the MN, instead of triggering re-establishment.

In all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN and the UE continues measurements based on configuration from the MN and the SN if possible. The SN measurements configured to be routed via the MN will continue to be reported after the SCG failure.

NOTE:	UE may not continue measurements based on configuration from the SN after SCG failure in certain cases (e.g. UE cannot maintain the timing of PSCell).

The UE includes in the SCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN.	The MN handles the SCG Failure Information message and may decide to keep, change, or release the SN/SCG. In all the cases, the measurement results according to the SN configuration and the SCG failure type may be forwarded to the old SN and/or to the new SN.

In NE-DC and NN-DC, upon detecting MCG failure when split SRB is configured and when SCG transmissions are not suspended, the UE suspends MCG transmissions and reports the MCG Failure Information to the SN, instead of triggering re-establishment if MCG transmission is not already suspended. If MCG transmissions are suspended prior to the MCG failure detection, the UE does nothing.

END OF CHANGES