3GPP TSG-RAN Meeting #103bis	Tdoc R2-1814577

Chengdu, China, 2018-10-08 to 2018-10-12

Agenda Item:	10.5.3

Source:	Ericsson

Title:	UE capabilities and band combinations for NE-DC

Document for:	Discussion, Decision

# Introduction

In the e-mail discussion [103#49] [1], it was discussed whether MR-DC capability container concept could be also applicable for NGEN-DC and NE-DC. This contribution further discusses the changes needed in UE-MRDC-Capabilities IE to accommodate NGEN-DC and NE-DC.

# Discussion

Although the same UE-MRDC-Capabilities IE would be used for EN-DC, NGEN-DC and NE-DC, it is still unclear how the NW would request different MR-DC options with the UE capability enquiry procedure, and how the UE would respond to it.

To define the NW request of capabilities for MR-DC options (except NR-DC), it should be taken into consideration that the capabilities related to band combinations, for each MR-DC option, may constitute the biggest portion of UE capabilities in the UE-MRDC-Capabilities container.

When the UE responds to an UECapabilityEnquiry message, most of the size of the UE-MRDC-Capabilities container, if included, may come from information related to band combinations clarify that it would be bandcombinationlist.

Moreover, since NGEN-DC does not imply physical layer changes, compared to EN-DC, it can be assumed that a report of band combinations and associated features (e.g. FeatureSets, FeatureSetCombinations, etc.) applicable to EN-DC is also applicable for NGEN-DC. This same assumption could also be extended to NE-DC, while an LS should be sent to RAN4 to confirm both assumptions.

Send an LS to RAN4 to confirm the assumption that band combinations, together with their corresponding features (e.g. FeatureSets and FeatureSetCombinations), applicable to EN-DC are also applicable to NGEN-DC and NE-DC.

If the aforementioned assumption can be confirmed, and considering the impact of the size of band combination information conveyed in the UE-MRDC-Capabilities container, as discussed above, to reduce the size of the capabilities reported by the UE related to MR-DC, a sole UE-MRDC-Capabilities container should be conveyed in UECapabilityInformation message to provide capabilities for EN-DC, NGEN-DC and NE-DC.

When responding to an UECapabilityEnquiry message that includes rat-Type set to eutra-nr and the UE supports EN-DC, NGEN-DC or NE-DC, the UE includes a sole UE-MRDC-Capabilities container in the UECapabilityInformation message to provide capabilities for EN-DC, NGEN-DC and NE-DC, whichever are supported.

The questions that follow from Proposal 2 are: whether any further capability indication should be included to signal the support of NGEN-DC and NE-DC; and how the procedures for the request of UE capabilities will account for NGEN-DC and NE-DC in TS 38.331 and TS 36.331. This contribution details the former, while the latter can be addressed after details for UE capability transfer for EN-DC are progressed [2].

For EN-DC support, the field en-DC conveyed in UE-EUTRA-Capabilities is used to indicate it.

IRAT-ParametersNR-r15 ::=		SEQUENCE {

en-DC-r15						ENUMERATED {supported}						OPTIONAL,

eventB2-r15						ENUMERATED {supported}						OPTIONAL,

supportedBandListNR-r15			SupportedBandListNR-r15						OPTIONAL

}

Moreover, the e-mail discussion [103#44] [3] have as proposal the following parameter for UE-EUTRA-Capabilities:

| eutra-5GC Indicates whether the UE supports E-UTRA/5GC. | No |
| --- | --- |

Therefore, if 5GC is supported (with eutra-5GC field) and EN-DC is supported (with en-DC field), these together can be used to indicate the support of NGEN-DC. Thus there is no need for an separate explicit capability bit for the NGEN-DC support.

The support of NGEN-DC is implicitly signalled in UE-EUTRA-Capabilities when both 5GC (indicated with eutra-5GC field) and EN-DC (indicated with en-DC field) are supported.

For NE-DC, since the MN is NR, an indication of its support should be included in NR-UE-capabilities.

NE-DC support is signalled in NR-UE-capabilities.

# Conclusion

In section 2 we made the following observations:

Observation 1	When the UE responds to an UECapabilityEnquiry message, most of the size of the UE-MRDC-Capabilities container, if included, may come from information related to band combinations clarify that it would be bandcombinationlist.

Based on the discussion in section 2 we propose the following:

Proposal 1	Send an LS to RAN4 to confirm the assumption that band combinations, together with their corresponding features (e.g. FeatureSets and FeatureSetCombinations), applicable to EN-DC are also applicable to NGEN-DC and NE-DC.

Proposal 2	When responding to an UECapabilityEnquiry message that includes rat-Type set to eutra-nr and the UE supports EN-DC, NGEN-DC or NE-DC, the UE includes a sole UE-MRDC-Capabilities container in the UECapabilityInformation message to provide capabilities for EN-DC, NGEN-DC and NE-DC, whichever are supported.

Proposal 3	The support of NGEN-DC is implicitly signalled in UE-EUTRA-Capabilities when both 5GC (indicated with eutra-5GC field) and EN-DC (indicated with en-DC field) are supported.

Proposal 4	NE-DC support is signalled in NR-UE-capabilities.

# References

E-mail discussion [103#49] [NR late drop] Capability coordination for NR-DC, Nokia, 3GPP TSG-RAN WG2#103 Gothenburg, Sweden, 20th – 24th August 2018.

R2-18xxxxx, UE capability enquiry for EN-DC, Ericsson, 3GPP TSG-RAN WG2#103bis Chengdu, China,  2018-10-08 to 2018-10-12.

E-mail discussion [103#44][NR] LTE/NR mobility capability and eLTE capability, Intel, 3GPP TSG-RAN WG2#103 Gothenburg, Sweden, 20th – 24th August 2018.