3GPP TSG-RAN WG2 Meeting #103bis	R2-1815524

# Chengdu, China 8–12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0494 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on moreThanOneRLC. |
| --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In field description of moreThanOneRLC in PDCP-config, there is "FFS/TODO" about the case of more than 2 “secondary cell groups”. This field is about the case that more than 2 configured RLC entities (i.e. 2 logical channels), and not limited to more than 2 SCGs. Rel-15 NR supports CA duplication where two RLCs for a radio bearer are configured on the same CG. Thus, this description should be fixed. Also, Rel-15 NR does not support two SCGs. NR RRC clearly mentioned that SCG can have only CellGroupID=1, i.e. two SCGs cannot be configured in this release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Field description of moreThanOneRLC was added. - Configuration of a radio bearer with more than one associated logical channel. - In this version of specification, at most two logical channels for a radio bearer are configured. FFS/TODO is removed. Impact analysis Impacted functionality: There is no impact to existing functionalities. Inter-operability: There is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Field description of moreThanOneRLC is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification >

–	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=			SEQUENCE {

drb						SEQUENCE {

discardTimer			ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity} 													OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL			ENUMERATED {len12bits, len18bits}				OPTIONAL, -- Cond Setup2

pdcp-SN-SizeDL			ENUMERATED {len12bits, len18bits}				OPTIONAL, -- Cond Setup2

headerCompression		CHOICE {

notUsed					NULL,

rohc					SEQUENCE {

maxCID					INTEGER (1..16383)				DEFAULT 15,

profiles				SEQUENCE {

profile0x0001			BOOLEAN,

profile0x0002			BOOLEAN,

profile0x0003			BOOLEAN,

profile0x0004			BOOLEAN,

profile0x0006			BOOLEAN,

profile0x0101			BOOLEAN,

profile0x0102			BOOLEAN,

profile0x0103			BOOLEAN,

profile0x0104			BOOLEAN

},

drb-ContinueROHC			ENUMERATED { true }												OPTIONAL	-- Need N

},

uplinkOnlyROHC			SEQUENCE {

maxCID					INTEGER (1..16383)				DEFAULT 15,

profiles				SEQUENCE {

profile0x0006			BOOLEAN

},

drb-ContinueROHC			ENUMERATED { true }												OPTIONAL	-- Need N

},

...

},

integrityProtection		ENUMERATED { enabled }														OPTIONAL,	-- Cond ConnectedTo5GC

statusReportRequired	ENUMERATED { true }															OPTIONAL,	-- Cond Rlc-AM

outOfOrderDelivery		ENUMERATED { true }													OPTIONAL	-- Need R

}																											OPTIONAL,	-- Cond DRB

moreThanOneRLC			SEQUENCE {

primaryPath				SEQUENCE {

cellGroup				CellGroupId											OPTIONAL,	-- Need R

logicalChannel			LogicalChannelIdentity								OPTIONAL	-- Need R

},

ul-DataSplitThreshold	UL-DataSplitThreshold 									OPTIONAL, -- Cond SplitBearer

pdcp-Duplication			BOOLEAN													OPTIONAL	-- Need R

}																										OPTIONAL, -- Cond MoreThanOneRLC

t-Reordering				ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300,	ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24, spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14, spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03, spare02, spare01 }		OPTIONAL, -- Need S

...,

[[

cipheringDisabled		ENUMERATED {true} 														OPTIONAL 	-- Cond ConnectedTo5GC

]]

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, cipherng is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms50 corresponds to 50 ms, ms100 corresponds to 100 ms and so on.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of reconfiguration with sync where the PDCP termination point is not changed and the fullConfig is not indicated..

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configure ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. The network reconfiguresheaderCompression only upon reconfiguration involving PDCP re-establishment. Network configures headerCompression to notUsed when outOfOrderDelivery is configured.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The value of integrityProtection for a DRB can only be changed using reconfiguration with sync. FFS: text to indicate where to find the key.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5] FFS: need to specify something with respect to UE capabilities.

moreThanOneRLC Configuration of a radio bearer with more than one associated logical channel. In this version of specification, at most two logical channels for a radio bearer are configured.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. Out-of-order delivery is configured only when the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates whether duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the initial state of the duplicateion. If set to TRUE, duplication is activated. The value of this field is always TRUE, when configured for a SRB.

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value 12 is applicable.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value 12 is applicable.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 clause 5.2.1 for UL data tranmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs. The NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW indicates logicalChannel for CA based PDCP duplication, i.e., if both logical channels terminate in the same cell group.

statusReportRequired For AM DRBs, indicates whether the DRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For UL DRBs, the value shall be ignored by the UE.

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0ms, value ms20 corresponds to 20ms, value ms40 corresponds to 40ms, and so on. When the field is absent the UE applies the value infinity.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 byte, value b100 corresponds to 100 byte, value b200 corresponds to 200 byte, and so on. The network sets this field to 'infinity' for UEs not supporting splitDRB-withUL-Both-MCG-SCG.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB is being set up, not present for SRBs. Otherwise this field is optionally present, need M. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than one associated logical channel and upon RRC reconfiguration with the association of an additional logical channel to the PDCP entity. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent and all its included parameters are released. |
| Rlc-AM | For RLC AM, the field is optionally present, need R. Otherwise, the field is not present. |
| Setup | The field is mandatory present in case of radio bearer setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC. Otherwise the field is absent. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM.. Otherwise, ths field is not present. |

<End of modification >