3GPP TSG-RAN WG2 Meeting #104		                                                                    R2-1816501

Spokane, USA, 12th – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0536 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on choice of 16-bit TAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-31 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In latest TS 38.331, there is one FFS whether CHOICE of 16-bit TAC is also needed for trackingareacode: Editor's Note: FFS whether CHOICE of 16-bit TAC is also needed. In our understanding this IE is used to designate the 5GC broadcast trackingareacode of bit length. For 5GC node the broadcast tracking area code consists of 3 octets [TS 24.501 section 9.11.3.49]. So there is no ambiguity on whether a gNB or ng-eNB can broadcast a tracking area code consisted with 2 octets However ANR CGI report a UE can also report CGI foran EUTRA cell indicated by cellForWhichToReportCGI as follow [TS 38.331]: 4> if the cell indicated by cellForWhichToReportCGI is an EUTRA cell: 5> if all mandatory fields of the cgi-Info-EPC for the concerned cell have been obtained: 6> include in the cgi-Info-EPC the fields broadcasted in EUTRA SystemInformationBlockType1 associated to EPC; The EPC trackingareacode of the EUTRA cell is a 2 octets. And this TAC is captured in TS 36.331. In TS 38.331, the field description of cgi-info-EPC indicates that this field includes the cellAccessRelatedInfo of TS 36.331. So there no need to capture EUTRA/EPC 2 bytes TAC in TS38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Editor's Note: FFS whether CHOICE of 16-bit TAC is also needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-operability If either UE or Network is not implemented according to this CR, then there may be misalignment on supported EUTRA/EPC CGI Impact Analysis: Specification of CGI report (including TAC) of EUTRA/EPC cell is not complete in TS38.331 If network instruct UE to report EUTRA/EPC cell CGI, the report may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------Change start----------------------------------------------------------

–

–	TrackingAreaCode

The IE TrackingAreaCode is used to identify a tracking area within the scope of a PLMN, see TS 24.501 [FFS_Ref].

.

TrackingAreaCodeinformation element

-- ASN1START

-- TAG-TRACKINGAREACODE-START

TrackingAreaCode ::= BIT STRING (SIZE (24))

-- TAG-TRACKINGAREACODE-STOP

-- ASN1STOP

-----------------------------------------------------Change end----------------------------------------------------------