3GPP TSG RAN WG2 Meeting #104 	R2-1816538

Spokane/USA, 12-16 November 2018

Title:	Report of email discussion [103bis#33] [NR - NTN]

Source: 	Thales

Type:	Discussion

Document for:	Agreement

Agenda Item:	11.6.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This email discussion is to discuss terminology and definitions for NR NTN SI. The scope of this email discussion is listed below.

- [103bis#33][NR – NTN]  (Thales)

Intended outcome:  capture agreements from this meeting into a TP

Deadline:  Thursday 2018-02-11

This document collects companies’ views and provides a Text proposal for the TR 38.821 v0.2.0 (see R3-186269).

# Discussion

The following have been agreed during RAN2#103bis:

As per requirements/scenarios

- The rapporteur will create a table with the worst case scenarios to be considered for requirements

- The study of TDD is down prioritised in the study. Note however that TDD can be considered for LEO based scenarios

- There is no TDD-specific timing requirements and solutions on layer 2 due to propagation delay

As per user plane, the following impacts will be studied

- DRX

- HARQ

- Random access response

- RLC/PDCP reordering (e.g. timers and SN space)

A priori there are no impact on SDAP to support non-terrestrial networks

As per Control Plane, the following impacts will be studied

- Mobility

- TA management and update

# Text Proposal for TR 38.821 v0.2.0

START OF CHANGES

# 7	Radio protocol issues and related solutions

Editor’s note: to be drafted by RAN2

## 7.1	Requirements and key issues

## 7.1.1 	Delay

In order to reduce the standardization work, the table here below identifies the worst case NTN scenarios to be considered for the delay constraint.

Table 8.1-1: NTN scenarios versus delay constraints, Source [2]

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35 786 km |  | 600 km |  |  |  |
| Relative speed of Satellite wrt earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 5° for feeder |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 1000 km |  | 50 km / 500 km |  |  |  |
| Maximum Round Trip Delay on the radio interface between the gNB and the UE | 541.75 ms (Worst case) | 270.57 ms | 28.41 ms |  | 12.88 ms |  |
| Minimum Round Trip Delay on the radio interface between the gNB and the UE | 477.14 ms | 238.57 ms | 8 ms |  | 4 ms |  |
| Maximum Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  | Up to +/- 20 µs/sec |  |
| Maximum delay difference within a NTN beam as seen by the UE (note 3) | 16 ms (Worst case) |  | 4.44 ms |  |  |  |
| Max rate of hand-over (FFS) |  |  |  |  |  |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario NOTE 3: The delay difference compares the delay (function of UE-satellite-NTN gateway distance) experienced by two different UEs served by the same beam at a given time |  |  |  |  |  |  |

When several non-terrestrial network scenarios features a maximum in terms of delay constraints, it is sufficient to study only one of these scenarios.

- NTN Scenario based on GEO with transparent payload for RTD and delay difference constraints

- NTN Scenario based on LEO with transparent payload and moving beams for the delay variation related constraint

As per the duplex mode:

- Down-prioritize TDD in this study item

- There is no TDD-specific timing requirements and solutions on layer 2 due to propagation delay.

## 7.2 		User plane enhancements

## 7.2.1 	MAC

Editor’s note: RAN2 will study impacts and possible enhancements to the following MAC functions including DRX, HARQ, Random Access procedure

## 7.2.2 	RLC

Editor’s note: RAN2 will study impacts and possible enhancements at least to RLC reordering (e.g. timers and SN space)

## 7.2.3 	PDCP

Editor’s note: RAN2 will study impacts and possible enhancements at least to PDCP reordering (e.g. timers and SN space)

## 7.2.4		SDAP

Editor’s note: A priori no impact are foreseen to support non-terrestrial networks

## 7.3 	Control plane enhancements

## 7.3.1 	Idle mode mobility enhancements

Editor’s note: RAN2 will study impacts and possible enhancements to Mobility (cell reselection), TA management and update

## 7.3.2 	Connected mode mobility enhancements

Editor’s note: RAN2 will study impacts and possible enhancements to Mobility (hand-over)

## 7.3.2 	Radio Link Monitoring

## 7.3.3 	Paging issue

Editor’s note: RAN2 will study impacts and possible enhancements to TA management and update and report to RAN3

## 7.3.4 	TBD

Editor’s note, section can be added based on enhancements identified to be studied

END OF CHANGES