3GPP TSG-RAN WG2 Meeting #104	R2-1816597

Spokane, USA, 12-16 November 2018

Agenda Item:	11.2.2.1

Source:	Charter Communications Inc.

Title:	Issue with NR-U Paging in Unlicensed Spectrum

Document for: Discussion and Agreement

# Introduction

This contribution highlights an issue wrt NR paging mechanism which requires enhancements due to operations in unlicensed spectrum, and correspondingly proposes a TP for inclusion in TR 38.889.

# Discussion

Today the CN-based paging operation uses Class 2 operation on NG-AP, and a timer-controller operation @ NAS-level. At a high-level:

- For RRC_IDLE UEs, AMF informs NG-RAN node (either gNB, or ng-eNB) to initiate paging for UE identified via 5G-S-TMSI in certain cells/TAs via NG-AP:PAGING message

- NG-RAN node pages target UEs via RRC:PAGING message

- If target UEs do not respond to the page in a certain time duration, then AMF can re-initiate paging. At this juncture, paging escalation strategies may be applied e.g. previous page msg was restricted to few cells in 1 TA, but re-paging may involve increasing the paging area to e.g. all cells in that TA, or all cells in multiple TAs etc – paging escalation strategies aren’t standardized in 3GPP.

The NG-AP paging (a class 2) procedure from TS 38.413 [1] clause 8.5.2 is shown below:

The NAS procedure for Paging from TS 24.501 [2] clause 5.6.2.2.1 is shown below:

When introducing paging over unlicensed carriers, access to the medium can’t be guaranteed. For such cases, when the AMF starts T3513 [2] it will not be able to distinguish between:

- T3513 expiring due to UE not responding v/s

- T3513 expiring due to NG-RAN not being able to send the paging message on physical carrier (due to LBT not clearing)

Table 10.2.2 of [2] shows the following default values for T3513:

| T3513 | NOTE 4 | 5GMM-REGISTERED | Paging procedure initiated | Paging procedure completed as specified in subclause 5.6.2.2.1 | Network dependent |
| --- | --- | --- | --- | --- | --- |

Case a) should result in paging escalation at 5GC.

Case b) should allow the possibility to re-transmit the same paging message at the same granularity at least once more prior to escalating. This is to avoid unnecessary load on paging channels across multiple gNBs.

Given the variance for default value of T3513 (of being network dependent), and that multiple consecutive LBT failures on loaded (>~60% load conditions) channel can be very likely, discerning between a) and b) becomes important.

When discussed in the context of NR-U SA, to give a flavor of what the possible solutions could look like:

- Introduce an NR-U specific paging message of type Class 1 e.g. NG-AP:PAGING_U message and  NG-AP:PAGING_U RESPONSE message

- Convert existing Class 2 NG-AP:PAGING message to Class 1 message; this is a non-backward compatible change

Note: Possible solutions are mentioned in order to help understand the nature of resulting impact. The authors are not suggesting adoption of one of the above.

Proposal 1: NG-RAN Paging mechanism (including possible NG-AP impacts) should account for LBT.

# Conclusion

In this contribution, we put forth the following proposals:

Proposal 1: NG-RAN Paging mechanism (including possible NG-AP impacts) should account for LBT.

# References

- TS 38.413 v15.1.0

- TS 24.501 v15.1.0

# Text Proposal

##### 7.2.2.3.A	Paging

<< insert TP from RAN2#103bis wrt support for CN- and RAN- paging >>

For 5GC- initiated paging, it is possible that NG-RAN may be unable to send the paging message on the physical medium due to systematic downlink LBT failures. In such cases, it is beneficial to have a mechanism to allow NG-RAN to inform 5GC of the same, such that 5GC is able to apply appropriate paging escalation strategy.

##### 7.2.2.3.X	Other