3GPP TSG-RAN WG2 Meeting 104	R2-1816701

Spokane, Washington, USA, 12-16 November 2018	(Revision of R2-1814055)

Agenda item:	11.4.2.1

Source:		Intel Corporation

Title:		On overall sidelink design principles for NR V2X

Document for:	Discussion

Introduction

One of the key objectives in the new SI on NR based V2X revolves around overall sidelink design for supporting advanced V2X use cases as described in [1].

1: Sidelink design [RAN1, RAN2]: - Identify technical solutions for a NR sidelink design to meet the requirements of advanced V2X services, including - Study the support of sidelink unicast, sidelink groupcast and sidelink broadcast - Study NR sidelink physical layer structures and procedure(s) - Study sidelink synchronization mechanism - Study sidelink resource allocation mechanism (also including objective 3) - Study sidelink L2/L3 protocols

In this regard, while there was some discussion on the NR vs LTE cross RAT control operation, there was no consideration of the operational scenario from the perspective of usage of a particular interface (Uu vs PC5) for the advanced V2X use cases. So, in this paper, we focus on this aspect and present our view and a corresponding TP in annex.

Discussion

  - NR V2X operation scenarios

During Rel-14, different V2X operation scenarios were considered as part of the SI to cater to the V2X use cases and their associated requirements. In this regard, different scenarios based on the usage of PC5 and Uu were considered and different design aspects for each of these scenarios were discussed. The general description and operational aspects for these scenarios are discussed in Rel-14 TR [2]. For NR, while the use cases are different and varied, from an operational standpoint, we do not expect the discussion on operation scenarios to be fundamentally different. So, while the operational aspects to consider for NR V2X scenarios might (and indeed will) be different, the scenarios themselves should be similar to LTE V2X, i.e. whether the V2X operation is based only on PC5, only Uu or a combination of both (for V2V, V2I and V2P operation). Before considering any enhancements to support these scenarios, we think it worthwhile to discuss and agree in RAN2 that the broad operational scenarios for NR V2X should be defined, based on LTE V2X and this can be captured in the TR for this SI (refer to the appendix for illustration of the LTE scenarios captured in TR 36.885). Additionally, the 3rd scenario is similar to relay based operation and does not appear to be the focus of any particular SI objective. It was deprioritized from study in Rel-14 and we think it can similarly be deprioritized from RAN2 point of view.

Proposal 1:	V2X operation scenarios for this SI should be defined based on LTE Rel-14 baseline, i.e. operation over only PC5 and only Uu shall be considered for this SI.

  - Operational design aspects

Once the operational scenarios to consider have been determined, the next step is to consider various factors and operational aspects for each of these scenarios. In our view, at least the following need to be considered for NR V2X, some of which are discussed in more detail below:

- Advanced V2X use case

- Applicability of unicast, broadcast and groupcast communication

- gNB deployment consideration and network control

- multi-carrier operation (similar to LTE)

- Operational bands/frequency range for deployment (similar to LTE)

Advanced V2X use cases:

Compared to LTE, the addition of advanced use cases as a potential factor to consider for supporting the operational scenarios is important, since the SID explicitly mentions that the focus and scope of NR V2X SI is to target advanced V2X use cases. Since the broad set of NR V2X use cases have very different set of requirements, it is worthwhile to consider these use cases individually when designing NR sidelink and whether a particular operational scenario should be prioritized/deprioritized. In this regard, it could be useful to analyse individual use cases and the applicability to these use cases as below:

Table 1 Analysis of advanced V2X use cases and applicability to Uu or PC5 based operation

| Advanced use case | Sub-division/scenario | Applicability to operational scenario |
| --- | --- | --- |
| Platooning | Cooperative driving | Primarily PC5 based communication between platoon members. Uu based operation might be useful for platoon leader operations. |
|  | Reporting to RSU entity |  |
|  | Information sharing between UEs |  |
| Advanced Driving | Emergency driving related | Primarily based on PC5. For information sharing related to automated driving, both PC5 and Uu can be applicable. Video sharing primarily based on Uu |
|  | Driving related information sharing to RSU entity |  |
|  | Video sharing to V2X application server |  |
| Extended sensor support | Sensor information sharing between UEs | PC5 based communication seems to be focus for both cases |
|  | Video sharing between UEs |  |
| Remote driving |  | Predominantly based on Uu |

From the preliminary analysis in Table 1, it can be shown that PC5 only based operational scenario seems ideally suited to support extended sensor support while Uu based operation is almost exclusively relevant for the remote driving use case. So, both PC5 only and Uu only modes of operation are valid. Additionally, as briefly discussed above, the need for the 3rd scenario from LTE V2X (i.e. Uu and PC5 based relay operation) needs to be sufficiently motivated, given that proposed NR sidelink enhancements in RAN1 already seek to enhance NR sidelink in substantial ways (e.g. HARQ feedback, resource selection mechanism enhancements, etc. [3]) to support the NR use cases. So, the mixed scenario itself needs further discussion before it is agreed to be studied.

Proposal 2a:	The set of NR advanced V2X use cases as in TS 22.186 shall be explicitly considered as a fundamental aspect for RAN2 design work.

Proposal 2b:	The following association between the advanced V2X use cases to realistic operational scenarios shall be considered:

- Both Uu and PC5 scenarios: platooning and advanced driving

- PC5 scenario: extended sensor support

- Uu scenario: remote driving

Conclusion

This contributions discusses aspects related to overall design of NR sidelink for supporting advanced V2X use cases and makes the following proposals:

Proposal 1:	V2X operation scenarios for this SI should be defined based on LTE Rel-14 baseline, i.e. operation over only PC5 and only Uu shall be considered for this SI.

Proposal 2a:	The set of NR advanced V2X use cases as in TS 22.186 shall be explicitly considered as a fundamental aspect for RAN2 design work.

Proposal 2b:	The following association between the advanced V2X use cases to realistic operational scenarios shall be considered:

- Both Uu and PC5 scenarios: platooning and advanced driving

- PC5 scenario: extended sensor support

- Uu scenario: remote driving

References

| [1] | "TR 22.186, Enhancement of 3GPP support for V2X scenarios; Stage 1". |
| --- | --- |
| [2] | "TR 36.885, "Study on LTE-based V2X Services"". |
| [3] | "RAN1#94, "Chairman Notes"". |

Annex (LTE V2X operation scenarios)

(a) V2V operation

(b) V2I operation

(c) V2P operation

Figure 5-1: Scenario 1

(a) V2V operation

(b) V2I operation

(c) V2P operation

(d) V2N operation

Figure 5-2: Scenario 2