3GPP TSG-RAN2 Meeting #104 	R2-1816806

Spokane, USA, 2018-11-12 to 2018-11-16

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Release of bearers at MR-DC during full config |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In case of full configuration during MR-DC, the SRB3 should be released unless it is reconfigured. In addition, for NE-DC, the SCG will be configured with E-UTRA RLC and DTCH which need to be released |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added condition to SRB release to be triggered also from full configuration. Introduced procedures for full configuration to release E-UTRA RLC bearer in case of NE-DC. Impact analysis Impacted functionality: - Full configuration Inter-operability: If the network is implemented according to this CR while the UE is not, the UE will not be able to correctly perform full configuration. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.6.2 SRB release 5.3.5.1 Full Configuration |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | This CR is subject to changes by IPA CRs. In this CR we have assumed IPA CRs are approved. |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 5.3.5.6	Radio Bearer configuration

##### 5.3.5.6.2	SRB release

The UE shall:

1>	if SRB3 is to be released as the result of full configuration according to 5.3.5.11; or

1> if the srb3-ToRelease is present:

2>	release the PDCP entity of the SRB3.

END OF FIRST CHANGE

START OF SECOND CHANGE

#### 5.3.5.11	Full configuration

WI Rapporteur's Note; This section is subject to changes by IPA CRs. In this section, we have assumed IPA CRs are approved.

The UE shall:

1>	release/ clear all current dedicated radio configurations except the MCG C-RNTI and the security configurations associated with the master key;

NOTE 1:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig. Radio configuration does not include SRB configurations and DRB configurations as configured by radioBearerConfig.

1>	if the spCellConfig in the masterCellGroup includes the reconfigurationWithSync (handover):

2>	release/ clear all current common radio configurations;

2>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311;

1>	else (full configuration after re-establishment or during RRC resume):

2>	if ue-TimersAndConstants are included in the SIB1:

3>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1

2>	else:

3>		use the default values specified in 9.2.3 for timer T310, T311 and constant N310, N311;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	for each srb-Identity value that is part of the current UE configuration:

2>	if the SRB is associated with an E-UTRA RLC bearer:

3>	release the SRB as specified in TS 36.331 [10] section xxx;

2>	if the srb-Identity value is included in the srb-ToAddModList (SRB reconfiguration):

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 2:	This is to get the SRBs (SRB1 and SRB2 for handover and SRB2 for reconfiguration after re-establishment) to a known state from which the reconfiguration message can do further configuration.

2>	else (SRB3 release):

3>	release the SRB3 as specified in 5.3.5.6.2;

3>	release the RLC entity or entities for SRB3 as specified in 5.3.5.5.3;

3>	release the DCCH logical channel for SRB3;

3>	release the srb-Identity for SRB3;

1> for each pdu-Session that is part of the current UE configuration:

2> release the SDAP entity (section 5.1.2 in TS 37.324 [24]);

2>	release the PDCP entity for each DRB associated to the pdu-Session;

2>	if the DRB is associated to an NR RLC bearer:

3>	release the RLC entity or entities for each DRB associated to the pdu-Session;

3>	release the DTCH logical channel for each DRB associated to the pdu-Session;

3>	release the drb-identity for each DRB associated to the pdu-Session;

NOTE 3:	This will retain the pdu-Session but remove the DRBs including drb-identity of these bearers from the current UE configuration and trigger the setup of the DRBs within the AS in Section 5.3.10.3 using the new configuration. The pdu-Session acts as the anchor for associating the released and re-setup DRB. In the AS the DRB re-setup is equivalent with a new DRB setup (including new PDCP and logical channel configurations).

2>	if the DRB is associated to an E-UTRA RLC bearer:

3>	release the DRB as specified in TS 36.331 [10] section 5.3.10.2;

1>	for each pdu-Session that is part of the current UE configuration but not added with same pdu-Session in:

2>	indicate the release of the user plane resources for the pdu-Session to upper layers;