3GPP TSG-RAN2 Meeting #104 	R2-1817592

Spokane, USA, 12th – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0075 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCG measurement results and reporting for MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The encoding and reporting of the measurement results for the SCG measurement results for MR-DC is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 7.2 Measurements - Clarified that SCG measurements are encoded in NR format in case of NGEN-DC and NR-DC, whereas are encoded in LTE formated in case of NE-DC. 10.10 RRC Transfer - Clarified that the procedure is not only used to transfer NR measurement report for the SCG, but it can be used also for NR measurement report for the other MR-DC options. Impact analysis If the network implements the CR and the UE does not, the measurement report in case of NE-DC will not work and the network will never receive measurements. Also, it will not be clear how to encode the measurements that need to be sent to the SN. If the UE implements the CR and the network does not, there are no particular inter-operability impacts on existing features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not implemented the UE does not know where and how to send the SCG measurement results in case is configured with MR-DC (i.e., NGEN-DC, NE-DC, and NR-DC). |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 7.2	Measurements

If the measurement is configured to the UE in preparation for the Secondary Node Addition procedure described in sub-clause 10.2, the Master node should configure the measurement to the UE.

In case of the intra-secondary node mobility described in sub-clause 10.3, the SN should configure the measurement to the UE in coordination with the MN, if required.

The Secondary Node Change procedure described in sub-clause 10.5 can be triggered by both the MN (only for inter-frequency secondary node change) and the SN. For secondary node changes triggered by the SN, the RRM measurement configuration is maintained by the SN which also processes the measurement reporting, without providing the measurement results to the MN.

Measurements can be configured independently by the MN (for inter-RAT measurement) and by the SN (intra-RAT measurements on serving and non-serving frequencies). The MN indicates the maximum number of frequency layers and measurement identities that can be used in the SN to ensure that UE capabilities are not exceeded. If MN and SN both configure measurements on the same carrier frequency, then the configurations need to be consistent (if the network wants to ensure these are considered as a single measurement layer). Each node (MN and SN) can configure independently a threshold for the SpCell quality: when the PCell quality is above the threshold configured by the MN, the UE is still required to perform inter-RAT measurements configured by the MN on the SN RAT (while it's not required to perform intra-RAT measurements); when the PSCell quality is above the threshold configured by the SN, the UE is not required to perform measurements configured by the SN.

NOTE:	The SN cannot renegotiate the number of frequency layers allocated by the MN in this version of the protocol.

When SRB3 is not configured, reports for measurements configured by the SN are sent on SRB1. When SRB3 is configured, reports for measurements configured by the SN are sent on SRB3.

Measurement results related to the target SN can be provided by MN to target SN at MN initiated SN change procedure. Measurement results of target SN can be forwarded from source SN to target SN via MN at SN initiated SN change procedure. Measurement results related to the target SN can be provided by source MN to target MN at Inter-MN handover with/without SN change procedure.

In EN-DC, NGEN-DC and NR-DC, measurement results according to measurement configuration from the MN are encoded according to NR RRC when they are provided by MN to SN in SgNB Addition Request message. During SN initiated SN change procedure, measurement results according to measurement configuration from SN are encoded according to NR RRC when they are provided by MN to SN in SgNB Addition Request message.

In NE-DC, measurement results according to measurement configuration from the MN are encoded according to LTE RRC when they are provided by MN to SN in SeNB Addition Request message. During SN initiated SN change procedure, measurement results according to measurement configuration from SN are encoded according to LTE RRC when they are provided by MN to SN in SeNB Addition Request message.

Per-UE or per-FR measurement gaps can be configured, depending on UE capability and network preference. Per-UE gap applies to both FR1 (LTE and NR) and FR2 (NR) frequencies. For per-FR gap, two independent gap patterns (i.e. FR1 gap and FR2 gap) are configured for FR1 and FR2 respectively. The UE may also be configured with a per-UE gap sharing configuration (applying to per-UE gap) or with two separate gap sharing configurations (applying to FR1 and FR2 measurement gaps respectively) [8].

In EN-DC, if per-UE gap is used, the MN decides the gap pattern and the related gap sharing configuration. If per-FR gap is used, the MN decides the FR1 gap pattern and the related gap sharing configuration for FR1, while the SN decides the FR2 gap pattern and the related gap sharing configuration for FR2. The measurement gap configuration from the MN to the UE indicates if the configuration from the MN is a per-UE gap or an FR1 gap configuration. The MN also indicates the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR1) to the SN. Measurement gap configuration assistance information can be exchanged between the MN and the SN. For the case of per-UE gap, the SN indicates to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE. For the per-FR gap case, the SN indicates to the MN the list of SN configured frequencies in FR1 measured by the UE and the MN indicates to the SN the list of MN configured frequencies in FR2 measured by the UE.

END OF CHANGES

START OF CHANGES

## 10.10	RRC Transfer

### 10.10.2	MR-DC with 5GC

Editor's note: MR-DC with the 5GC is not complete and is targeted for completion in December 2018.

The RRC Transfer procedure is used to exchange RRC messages between the MN and the UE via the SN (split SRB) and to provide measurement reports from the UE to the SN. In NGEN-DC and NR-DC, the measurement report is NR whereas in NE-DC is LTE.

Split SRB:

Figure 10.10.2-1: RRC Transfer procedure for split SRB (DL operation)

Figure 10.10.2-1 shows an example signaling flow for DL RRC Transfer in case of the split SRB:

1.	The MN, when it decides to use the split SRBs, starts the procedure by initiating the RRC Transfer procedure. The MN encapsulates the RRC message in a PDCP-C PDU and ciphers with own keys.

NOTE:	The usage of the split SRBs shall be indicated in the Secondary Node Addition procedure or Modification procedure.

2.	The SN forwards the RRC message to the UE.

3.	The SN may send PDCP delivery acknowledgement of the RRC message forwarded in step 2.

Figure 10.10.2-2: RRC Transfer procedure for split SRB (UL operation)

Figure 10.10.2-2 shows an example signaling flow for UL RRC Transfer in case of the split SRB:

1.	When the UE provides response to the RRC message, it sends it to the SN.

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received PDCP-C PDU with encapsulated RRC message.

Measurement report:

Figure 10.10.2-3: RRC Transfer procedure for measurement report

Figure 10.10.2-3 shows an example signaling flow for RRC Transfer in case of the forwarding of the measurement report from the UE:

1.	When the UE sends a measurement report, it sends it to the MN in the GenericContainer.

2.	The MN initiates the RRC Transfer procedure, in which it transfers the received measurement report as an octet string.

END OF CHANGES