3GPP TSG-RAN2 Meeting #104 	R2-1817915

Spokane, USA, 2018-11-12 to 2018-11-16

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0720 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security codepoints for NGEN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During email discussion 103bis#25, the leading alternatives to configure the security algorithms for bearers terminating in an ng-eNB was Option 1, where UEs supporting NGEN-DC use NR code points for all bearers (even when NGEN-DC) is not configured. This CR implements this option. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Since the network knows the UE capability, under option, if the UE supports NGEN-DC, then the network will configure it with NR codepoints (i.e securityConfig inside the radiobearerconfig). Otherwise (if the UE doesn’t support NGEN-DC), the network will not configure the securityConfig inside the radioBearerConfig and the UE will use the LTE codepoints provide in the SMC or in securityConfigHO during HO. Since both the LTE codepoints provided in SMC or securityConfigHO and the NR codepoints provided in radioBearerConfig are maintained (if not included) the UE will keep using the currently saved configurations if no security information is provided in the reconfiguration message. If the UE doesn’t support NGEN-DC and handover eLTE to NR or from NR to eLTE the UE shall release the stored security configurations from the source RAT. 5.3.5.6.3 SRB addition/modification - Added a condition to check if the securityAlgorithmConfig is received from E-UTRA or if the securityConfig is received from NR. If so, the UE uses the received algorithms and releases the stored security configurations from the source RAT if it has any stored. If the UE doesn’t receive any security configurations from either NR or E-UTRA, the UE keep using the ones it is configured with. - Modified procedure for modifying E-UTRA SRBs with NR PDCP connected to 5GC in same way as the addition of SRBs 5.3.5.6.5 DRB addition/modification - Updated procedures in same way as SRB addition/modification 6.3.2 RadioBearerConfig - Change field descriptions condition of securityAlgortihmConfig to to clarify that network only configures it if the UE supports NGEN-DC. Impact analysis Impacted functionality: - Security algorithms configuration Inter-operability: If the network is implemented according to this CR while the UE is not, the UE and network may assume wrong security code points which could result in a breakdown of the conditions. If the UE is implemented according to this CR while the network is not, the UE and network may assume wrong security code points which could result in a breakdown of the conditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of security algorithms for ng-eNB may be incorrect leading to connection failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.6.3 SRB addition/modification 5.3.5.6.5 DRB addition/modification 6.3.2 RadioBearerConfig |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGES

##### 5.3.5.6.3	SRB addition/modification

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity;

2>	if the securityAlgorithmConfig is included in the RRCConnectionReconfiguration message as specified in TS 36.331 [10]:

3>	configure the PDCP entity with the security algorithms and apply the keys (KRRCenc and KRRCint) configured/derived as specified in E-UTRA in TS 36.331 [10];

3>	release securityConfig, if configured;

2>	else if securityConfig is included in the received radioBearerConfig:

3>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB/ KgNB) or secondary key (S-KeNB/S-KgNB) as indicated in keyToUse, if applicable;

3>	release the securityAlgorithmConfig, if configured by E-UTRA as specified in TS 36.331 [10];

2>	else (i.e. the UE receives neither the securityAlgorithmConfig from E-UTRA nor the securityConfig from NR):

3>	if the UE is configured with securityAlgorithmConfig by E-UTRA as specified in TS 36.331 [10]:

4>	configure the PDCP entity with the security algorithms and apply the keys (KRRCenc and KRRCint) configured/derived as specified in E-UTRA in TS 36.331 [10];

3>	else (if the UE is configured with securityConfig by NR):

4>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB/ KgNB) or secondary key (S-KeNB/S-KgNB) as indicated in keyToUse, if applicable;

2>	if the current UE configuration as configured by E-UTRA in TS 36.331 [10] includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC entity and DCCH of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config;

2>	else:

3>	configure the PDCP entity in accordance with the default configuration defined in 9.2.1 for the corresponding SRB;

1>	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration:

2>	if the reestablishPDCP is set:

3> if the securityAlgorithmConfig is included in the RRCConnectionReconfiguration message as specified in TS 36.331 [10]:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key configured/derived as specified in TS 36.331 [10], i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key configured/derived as specified in TS 36.331 [10], i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	release securityConfig, if configured;

3>	else if the securityConfig is included in the received radioBearerConfig:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB/KgNB) or secondary key (S-KeNB/S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB/KgNB) or secondary key (S-KeNB/S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	release the securityAlgorithmConfig, if configured by E-UTRA as specified in TS 36.331 [10];

3>	else (i.e. the UE receives neither securityAlgorithmConfig from E-UTRA nor securityConfig from NR):

4>	if the UE is configured with securityAlgorithmConfig by E-UTRA as specified in 36.331 [10]:

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key configured/derived as specified in TS 36.331 [10], i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key configured/derived as specified in TS 36.331 [10], i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	else (i.e. the UE is configured with securityConfig by NR):

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB/KgNB) or secondary key (S-KeNB/S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB/KgNB) or secondary key (S-KeNB/S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	re-establish the PDCP entity of this SRB as specified in 38.323 [5];

2>	else, if the discardOnPDCP is set:

3>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

END OF FIRST CHANGES

START OF SECOND CHANGES

##### 5.3.5.6.5	DRB addition/modification

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	if an sdap-Config is included:

3> if an SDAP entity with the received pdu-Session does not exist:

4> establish an SDAP entity as specified in TS 37.324 [24] section 5.1.1;

3> configure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [24] and associate the DRB with the SDAP entity;

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	if the securityAlgorithmConfig is included in the RRCConnectionReconfiguration message as specified in TS 36.331 [10]:

3>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

4>configure the PDCP entity with the ciphering algorithms and apply the KUPenc key configured/derived as specified in TS 36.331 [10];

3>	release securityConfig, if configured;

2>	else if securityConfig is included in the received radioBearerConfig:

3>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

4>configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the KUPenc key associated with the master key (KeNB/KgNB) or the secondary key (S-KeNB/S-KgNB) as indicated in keyToUse;

3> if the PDCP entity of this DRB is configured with integrityProtection:

4> configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KeNB/S-KgNB) as indicated in keyToUse;

3>	release the securityAlgorithmConfig, if configured by E-UTRA as specified in TS 36.331 [10];

2>	else (i.e. the UE receives neither securityConfig from NR nor securityAlgorithmConfig from E-UTRA):

3>	if the UE is configured with securityAlgorithmConfig by E-UTRA as specified in TS 36.331 [10]:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>configure the PDCP entity with the ciphering algorithms and apply the KUPenc key configured/derived as specified in TS 36.331 [10];

3>	else (i.e. the UE is configured with securityConfig):

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the KUPenc key associated with the master key (KeNB/KgNB) or the secondary key (S-KeNB/S-KgNB) as indicated in keyToUse;

4> if the PDCP entity of this DRB is configured with integrityProtection:

5> configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KeNB/S-KgNB) as indicated in keyToUse;

2>	if the UE is operating in EN-DC:

3>if the DRB was configured with the same eps-BearerIdentity either by NR or E-UTRA prior to receiving this reconfiguration:

4> associate the established DRB with the corresponding eps-BearerIdentity;

3> else:

4>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration:

2> if an sdap-Config is included, reconfigure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [24];

2>	if the reestablishPDCP is set:

3>	if the securityAlgorithmConfig is included in the RRCConnectionReconfiguration message as specified in TS 36.331 [10]:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10], i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE on this DRB;

4>	release securityConfig, if configured;

3>	else if the securityConfig is included in the received radioBearerConfig:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/KgNB) or secondary key (S-KeNB/S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE on this DRB;

4> if the PDCP entity of this DRB is configured with integrityProtection:

5> configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KeNB/S-KgNB) as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE on this DRB;

4>	release the securityAlgorithmConfig, if configured by E-UTRA as specified in TS 36.331 [10];

3>	else (i.e. the UE receives neither securityConfig from NR nor securityAlgorithmConfig from E-UTRA):

4>	if the UE is configured with securityAlgorithmConfig by E-UTRA as specified in TS 36.331 [10]:

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithms and apply the KRRCenc key configured/derived as specified in E-UTRA in TS 36.331 [10], i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE on this DRB;

4>	else (i.e. the UE is configured with securityConfig by NR):

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/KgNB) or secondary key (S-KeNB/S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE on this DRB;

5>	if the PDCP entity of this DRB is configured with integrityProtection:

6>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KeNB/S-KgNB) as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE on this DRB;

3>	re-establish the PDCP entity of this DRB as specified in 38.323 [5], section 5.1.2;

2>	else, if the recoverPDCP is set:

3>	trigger the PDCP entity of this DRB to perform data recovery as specified in 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

NOTE 1:	Removal and addition of the same drb-Identity in a single radioResourceConfig is not supported. In case drb-Identity is removed and added due to reconfiguration with sync or re-establishment with the full configuration option, the network can use the same value of drb-Identity.

NOTE 2:	When determining whether a drb-Identity value is part of the current UE configuration, the UE does not distinguish which RadioBearerConfig and DRB-ToAddModList that DRB was originally configured in.  To re-associate a DRB with a different key (KeNB to S-KeNB or vice versa), the network provides the drb-Identity value in the (target) drb-ToAddModList and sets the reestablishPDCP flag. The network does not list the drb-Identity in the (source) drb-ToReleaseList.

NOTE 3:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 4: 	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

NOTE 5: Ciphering and integrity protection can be enabled or disabled for a DRB. The enabling/disabling of ciphering or integrity protection can be changed only by releasing and adding the DRB.

END OF SECOND CHANGES

START OF THIRD CHANGES

### 6.3.2	Radio resource control information elements

#### –	RadioBearerConfig

The IE RadioBearerConfig is used to add, modify and release signalling and/or data radio bearers. Specifically, this IE carries the parameters for PDCP and, if applicable, SDAP entities for the radio bearers.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIO-BEARER-CONFIG-START

RadioBearerConfig ::=                   SEQUENCE {

srb-ToAddModList                        SRB-ToAddModList                                        OPTIONAL,   -- Cond HO-Conn

srb3-ToRelease                          ENUMERATED{true}                                        OPTIONAL,   -- Need N

drb-ToAddModList                        DRB-ToAddModList                                        OPTIONAL,   -- Cond HO-toNR

drb-ToReleaseList                       DRB-ToReleaseList                                       OPTIONAL,   -- Need N

securityConfig                          SecurityConfig                                          OPTIONAL,   -- Need M

...

}

SRB-ToAddModList ::=                    SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=                        SEQUENCE {

srb-Identity                            SRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

discardOnPDCP                           ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

DRB-ToAddModList ::=                    SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=                        SEQUENCE {

cnAssociation                           CHOICE {

eps-BearerIdentity                      INTEGER (0..15),                                    -- EPS-DRB-Setup

sdap-Config                             SDAP-Config                                         -- 5GC

}                                       OPTIONAL, -- Cond DRBSetup

drb-Identity                            DRB-Identity,

reestablishPDCP                         ENUMERATED{true}                                        OPTIONAL,   -- Need N

recoverPDCP                             ENUMERATED{true}                                        OPTIONAL,   -- Need N

pdcp-Config                             PDCP-Config                                             OPTIONAL,   -- Cond PDCP

...

}

DRB-ToReleaseList ::=                   SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=                      SEQUENCE {

securityAlgorithmConfig                 SecurityAlgorithmConfig                                 OPTIONAL,   -- Cond RBTermChange2

keyToUse                                ENUMERATED{master, secondary}                           OPTIONAL,   -- Cond RBTermChange

...

}

-- TAG-RADIO-BEARER-CONFIG-STOP

-- ASN1STOP

DRB-ToAddMod field descriptions

cnAssociation Indicates if the bearer is associated with the eps-bearerIdentity (when connected to EPC) or sdap-Config (when connected to 5GC).

drb-Identity In case of DC, the DRB identity is unique within the scope of the UE, i.e. an MCG DRB cannot use the same value as a split DRB. For a split DRB the same identity is used for the MCG and SCG parts of the configuration.

eps-BearerIdentity The EPS bearer ID determines the EPS bearer when NR connects to EPC using EN-DC

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to TRUE whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured.

recoverPDCP Indicates that PDCP should perform recovery according to TS38.323.

sdap-Config The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC and presence/absence of UL/DL SDAP headers.

RadioBearerConfig field descriptions

securityConfig Indicates the security algorithm and key to use for the signalling and data radio bearers configured with the list in this radioBearerConfig When the field is not included, the UE shall continue to use the currently configured keyToUse and security algorithm for the radio bearers reconfigured with the lists in this radioBearerConfig. If the UE is connected to EPC, this field is included for all bearers using NR PDCP. If the UE is connected to 5GC, the network only includes this field if the UE supports NGEN-DC.

srb3-ToRelease Release SRB3. SRB3 release can only be done at SCG release and reconfiguration with sync.

SecurityConfig field descriptions

keyToUse Indicates if the bearers configured with the list in this radioBearerConfig is using the master key or the secondary key for deriving ciphering and/or integrity protection keys. For EN-DC, network should not configure SRB1 and SRB2 with secondary key and SRB3 with the master key. When the field is not included, the UE shall continue to use the currently configured keyToUse for the radio bearers reconfigured with the lists in this radioBearerConfig..

securityAlgorithmConfig Indicates the security algorithm for the signalling and data radio bearers configured with the list in this radioBearerConfig. When the field is not included, the UE shall continue to use the currently configured security algorithm for the radio bearers reconfigured with the lists in this radioBearerConfig except if the UE is configured with security algorithms by E-UTRA as specified in TS 36.331 [10].

SRB-ToAddMod field descriptions

discardOnPDCP Indicates that PDCP should discard stored SDU and PDU according to TS38.323.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to TRUE whenever the security key used for this radio bearer changes. Key change could for example be due to reconfig with sync, and for SRB2 when resuming an RRC connection the first reconfiguration after RRC connection reestablishment in NR. For LTE SRBs using NR PDCP, it could be for handover, RRC connection reestablishment or resume.

srb-Identity Value 1 is applicable for SRB1 only. Value 2 is applicable for SRB2 only. Value 3 is applicable for SRB3 only.

| Conditional Presence | Explanation |
| --- | --- |
| RBTermChange | The field is mandatory present in case of set up of signalling and data radio bearer and change of termination point for the radio bearer between MN and SN. It is optionally present otherwise, Need S. |
| PDCP | The field is mandatory present if the corresponding DRB is being setup or corresponding RB is reconfigured with NR PDCP; otherwise the field is optionally present, need M. |
| DRBSetup | The field is mandatory present if the corresponding DRB is being setup; otherwise the field is optionally present, need M. |
| HO-Conn | The field is mandatory present in case of inter-system handover from E-UTRA (connected to EPC) to E-UTRA (connected to 5GC) or NR, or when the fullConfig is included in the RRCReconfiguration message, or in case of RRCSetup; otherwise the field is optionally present, need N. Upon RRCSetup, only SRB1 can be present. |
| HO-toNR | The field is mandatory present in case of inter-system handover from E-UTRA (connected to EPC) to E-UTRA (connected to 5GC) or NR, or when the fullConfig is included in the RRCReconfiguration message. In case of RRCSetup and RRCReestablishment, the field is not present; otherwise the field is optionally present, need N. |

END OF FOURTH CHANGES