3GPP TSG-RAN WG2 Meeting #104	R2-1818113

Spokane, USA, 12th – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0571 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on scaling RSRP threshold for beam failure recovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current TS38.321 states that, for beam failure recovery, the RSRP threshold for CSI-RS, i.e. rsrp-ThresholdCSI-RS, shall be set to a value calculated by multiplying the rsrp-ThresholdSSB by powerControlOffsetSS. However, considering that TS38.331 defines the powerControlOffsetSS as a value in dB, the scaling operation in TS38.321 should rather be “addition” than “multiplication”. We suggest to use the more general expression. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In S5.1.1, the text for scaling rsrp-ThresholdCSI-RS for beam failure recovery should be revised as “If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS shall be set to a value obtained by scaling rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE with powerControlOffsetSS as specified in TS 38.213 [6];”. Impact analysis Impacted functionality: The change only impacts the RA procedure. Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE would incorrectly scale the rsrp-ThresholdCSI-RS, which degrades the BFR performance. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability problem foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE incorrectly scales the rsrp-ThresholdCSI-RS for beam failure recovery and this results in performance degradation of beam failure recovery procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

RRC configures the following parameters for the Random Access procedure:

-	prach-ConfigIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble;

-	preambleReceivedTargetPower: initial Random Access Preamble power;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB and corresponding Random Access Preamble and/or PRACH occasion. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS and corresponding Random Access Preamble and/or PRACH occasion. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS shall be set to a value obtained by scaling rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE with powerControlOffsetSS as specified in TS 38.213 [6];

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	powerControlOffsetSS: a power offset between rsrp-ThresholdSSB and rsrp-ThresholdCSI-RS to be used when the Random Access procedure is initiated for beam failure recovery;

-	powerRampingStep: the power-ramping factor;

-	powerRampingStepHighPriority: the power-ramping factor in case of differentiated Random Access procedure;

-	scalingFactorBI: a scaling factor for differentiated Random Access procedure;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: defines PRACH occasion(s) associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see subclause 7.4);

-	ra-OccasionList: defines PRACH occasion(s) associated with a CSI-RS in which the MAC entity may transmit a Random Access Preamble;

-	preambleTransMax: the maximum number of Random Access Preamble transmission;

-	ssb-perRACH-Occasion (SpCell Only): defines the number of SSBs mapped to each PRACH occasion.

-	ssb-perRACH-OccasionAndCB-PreamblesPerSSB (SpCell only): defines the number of SSBs mapped to each PRACH occasion and the number of contention-based Random Access Preambles mapped to each SSB;

-	if groupBconfigured is configured, then Random Access Preambles group B is configured.

-	The Random Access Preambles in Random Access Preamble group A are the Random Access Preambles 0 to numberOfRA-PreamblesGroupA – 1, if Random Access Preambles group B is configured; Otherwise, the Random Access Preambles in Random Access Preamble group A are the Random Access Preambles 0 to the number of Random Access Preamles per SSB configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB;

-	The Random Access Preambles in Random Access Preamble group B, if configured, are the Random Access Preambles numberOfRA-PreamblesGroupA to the number of Random Access Preamles per SSB configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

NOTE 2:	If Random Access Preambles group B is supported by the cell and SSBs are mapped to Random Access Preambles, Random Access Preambles group B is included in each SSB.

-	if Random Access Preambles group B is configured:

-	ra-Msg3SizeGroupA (per cell): the threshold to determine the groups of Random Access Preambles;

-	msg3-DeltaPreamble: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection;

-	numberOfRA-PreamblesGroupA (SpCell only): defines the number of Random Access Preambles in Random Access Preamble group A for each SSB.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B is configured:

-	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink, and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101 [10].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101 [10].

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_POWER_RAMPING_STEP;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	SCALING_FACTOR_BI;

-	TEMPORARY_C-RNTI.

When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	if the carrier to use for the Random Access procedure is explicitly signalled:

2>	select the signalled carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:

2>	select the NUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

1>	set PREAMBLE_POWER_RAMPING_STEP to preamblePowerRampingStep;

1>	if powerRampingStepHighPriority is configured:

2>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.1.7); or

2>	if the Random Access procedure was initiated for handover:

3>	set the PREAMBLE_POWER_RAMPING_STEP to powerRampingStepHighPriority;

1>	set SCALING_FACTOR_BI to 1;

1>	if scalingFactorBI is configured:

2>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.1.7); or

2>	if the Random Access procedure was initiated for handover:

3>	set the SCALING_FACTOR_BI to scalingFactorBI;

1>	perform the Random Access Resource selection procedure (see subclause 5.1.2).

End of the first change