3GPP TSG-RAN WG2 Meeting 104	R2-1818209

# Spokane, USA, 12 – 16 November 2018

Agenda Item:	11.1.2

Source: 	ITL

Title: 	Discussion on flow control

Document for:	Discussion and decision

Introduction

In RAN2#103 meeting [1], RAN2 agreed the followings about flow control.

- 1.  FFS if Flow control mechanism is not considered for the uplink data congestion problem (as the current transmission/scheduling mechanisms provide per hop “flow control”).

- 2.  Flow control mechanism should be considered for the downlink data congestion problem.

- 3.  Study further both end-to-end flow control (CU – Access DU or CU - Congested Node FFS) and hop-by-hop flow control for the downlink data congestion problem.

- 4.  Downlink data congestion problem could be handled by a parent IAB node or the IAB donor with feedback reporting from the congested IAB nodes

In this contribution, we will discuss the IAB behaviour to support both end-to-end and hop-by-hop flow control.

Triggering events

Both end-to-end flow control and hop-by-hop flow control have in common that they are a way to solve the downlink congestion problem, but the mechanism is different as follows. In end-to-end flow control, the UE’s access IAB node can provide a downlink delivery status to the IAB donor. On the other hand, in hop-by-hop flow control, the congested IAB node can provide a downlink delivery status to the parent’s IAB node.

Hop-by-hop flow control is necessary for fast responding to congestions, and end-to-end is necessary for overall picture and statistics. Based on the advantage, the IAB node can determine whether to perform hop-by-hop flow control or end-to-end flow control. For example, when the IAB node requests a fast response for congestions, the IAB node can determine to use hop-by-hop flow control. The situation of requesting fast response can be that the IAB node detects a DL congestion problem. Therefore, we suggest that the IAB node can use hop-by-hop flow control mechanism based on event-triggering.

Proposal 1: IAB node can use hop-by-hop flow control mechanism when the IAB node detect the DL congestion.

On the other hand, since the end-to-end flow control can be slower than hop-by-hop flow control, the IAB node can use end-to-end flow control mechanism to prevent congestion before it determines that congestion has occurred. Thus, unlike hop-by-hop flow control, the IAB node can use end-to-end flow control by reporting periodically the downlink delivery status.

Proposal 2: IAB node can use end-to-end flow control mechanism by reporting periodically the DL delivery status.

Reporting mechanism

As we agreed to support both hop-by-hop and end-to-end flow control, it is preferred to simplify IAB node behaviour by aligning IAB node behaviour as much as possible. The one of the aligning behaviour can be a reporting message. The IAB node could use a MAC CE, adaptation header, or STATUS PDU for reporting message.

The IAB node can report the DL delivery status by defining a new MAC CE format. However, the MAC CE can only be used in hop-by-hop flow control because the MAC CE is not relayed. Alternatively, the IAB node can report the DL delivery status in adaptation header or a STATUS PDU in the adaptation layer. These can be used for hop-by-hop and end-to-end flow control.

We need to discuss the appropriate reporting message for supporting both hop-by-hop and end-to-end flow control mechanism.

Proposal 3: We need to discuss the reporting message, and the reporting message could be used for both hop-by-hop and end-to-end flow control.

Conclusion

In this contribution, we provide our view on the IAB node behavior for flow control. Based on the discussion, the following proposal is drawn:

Proposal 1: IAB node can use hop-by-hop flow control mechanism when the IAB node detect the DL congestion.

Proposal 2: IAB node can use end-to-end flow control mechanism by reporting periodically the DL delivery status.

Proposal 3: We need to discuss the reporting message, and the reporting message could be used for both hop-by-hop and end-to-end flow control.

Reference

RAN2#103 meeting reports