3GPP TSG-RAN WG2 Meeting #104                                	R2-1818251

Spokane, USA, 12th - 16th November 2018

Agenda item:	11.6.3.2

Source: 	Huawei, HiSilicon

Title: 	Further discussion on RLC/PDCP impacts in NTN

Document for:	Discussion and Decision

Introduction

In the RAN plenary #80 meeting, a new RAN3 SI on “solutions evaluation for NR to support Non Terrestrial Network” was approved [1]. One of the objectives is related to the propagation delay which is stated as follows:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2]

During RAN2#130bis meeting, the impacts of propagation delay on user plane are discussed and some general agreements are achieved as follows.

UP Impacts to study 1. DRX 2. HARQ 3. Random access response 4. RLC/PDCP reordering (e.g. timers and SN space) 5. SDAP => no impact

In this contribution, we will further discuss the impact of high propagation delay in Non Terrestrial Network (NTN) based on the above agreements.

Discussion

The relation between the altitude of satellite and propagation delay is showed in the following tables [2].

Table 1 Propagation delays for different NGEO satellites (altitude and payload types)

|  |  | LEO at 600 km |  | LEO at 1500 km |  | MEO at 10000 km |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Elevation angle | Path | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) |
| UE: 10° | satellite - UE | 1932.24 | 6,440 | 3647.5 | 12,158 | 14018.16 | 46.727 |
| GW: 5° | satellite - gateway | 2329.01 | 7.763 | 4101.6 | 13.672 | 14539.4 | 48.464 |
| 90° | satellite - UE | 600 | 2 | 1500 | 5 | 10000 | 33.333 |
| Bent pipe satellite |  |  |  |  |  |  |  |
| One way delay | Gateway-satellite_UE | 4261.2 | 14.204 | 7749.2 | 25.83 | 28557.6 | 95.192 |
| Round Trip Delay | Twice | 8522.5 | 28.408 | 15498.4 | 51.661 | 57115.2 | 190.38 |
| Regenerative satellite |  |  |  |  |  |  |  |
| One way delay | Satellite -UE | 1932.24 | 6.44 | 3647.5 | 12.16 | 14018.16 | 46.73 |
| Round Trip Delay | Satellite-UE-Satellite | 3864.48 | 12.88 | 7295 | 24.32 | 28036.32 | 93.45 |

Table 2 Propagation delays for GEO satellite at 35786 km

|  | GEO at 35786 km |  |  |
| --- | --- | --- | --- |
| Elevation angle | Path | D (km) | Time (ms) |
| UE :10° | satellite - UE | 40586 | 135.286 |
| GW : 5° | satellite - gateway | 41126.6 | 137.088 |
| 90° | satellite - UE | 35786 | 119.286 |
| Bent Pipe satellite |  |  |  |
| One way delay | Gateway-satellite_UE | 81712.6 | 272.375 |
| Round trip Time | Twice | 163425.3 | 544.751 |
| Regenerative Satellite |  |  |  |
| One way delay | Satellite -UE | 40586 | 135.286 |
| Round Trip Time | Satellite-UE-Satellite | 81172 | 270.572 |

As shown in above tables, the round trip delay (RTD) of MEO at 10000 km is up to 93.45ms while the round trip delay for GEO satellite arrives at 544.751ms. The overly large RTD may have impacts to RLC/PDCP layer.

  - Impact on L2 buffer size

L2 buffer size is calculated as MaxDLDataRate * RLC RTT + MaxULDataRate * RLC RTT. In NTN, the RTT will be hundreds of times larger. However, in NTN the maximum UL and DL data rate may not be very large because the untimely CQI feedback. Therefore, the impacts on L2 buffer size needs more RAN1 input, i.e., whether the maximum DL and UL data rate will be decreased. After RAN1 conclusion, RAN2 can begin to evaluate the impacts to L2 buffer size.

Proposal 1: The impacts of large RTD on L2 buffer size calculation require more RAN1 input.

  - Impact on RLC/PDCP SN

The length of PDCP SN will impact on the number of PDCP SDUs within one PDCP RTT, i.e. different peak data rate may require different length of PDCP SN.

In simplicity, we can obtain the needed PDCP SN length by the following calculation:

Similar to the calculation of L2 buffer size, the SN size is related to both RTD and maximum data rate. Therefore, whether PDCP/RLC SN needs to be extended requires further RAN1 input on the data rate.

Proposal 2: Whether PDCP/RLC SN needs to be extended requires further RAN1 input.

Conclusion

In this contribution, we discussed the RLC/PDCP impact of high propagation delay in Non Terrestrial Network (NTN), and we get the following observation and proposals:

Proposal 1: The impacts of large RTD on L2 buffer size calculation require more RAN1 input.

Proposal 2: Whether PDCP/RLC SN needs to be extended requires further RAN1 input.

References

[1]		RP-181370, Study on solutions evaluation for NR to support Non Terrestrial Network.

[2]		TR 38.811, Study on New Radio (NR) to support non terrestrial networks.