3GPP TSG-RAN WG2 Meeting #104       							      R2-1818270

Spokane, USA, 12th - 16th November 2018            (Resubmission of R2-1814267)

Source: 	vivo

Title:  	Evaluation of the RLM in NR-U

Agenda Item:	11.2.2.2

Document for:	Discussion and Decision

Introduction

The gNB operating on unlicensed spectrum needs to apply LBT procedure before transmitting RLM-RS resources. There is high probability that the physical layer in UE indicates out-of-sync to RRC layer when RLM-RS resources cannot be transmitted due to LBT failure. As a result, the UE will probably declare RLF.

In this contribution, we will focus on the evaluation of RLM for NR unlicensed, attempt to give some quantitative analysis on the relationship between LBT failure probability and the total number of RLF occurrences of both cell center users and cell edge users, according on the system-level simulation results.

Discussion

RLM and RLF

In NR, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell [1]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP.

The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate. If SSB is configured as RLM-RS, Qout shall correspond to 10% block error rate of a hypothetical PDCCH. The threshold Qin is defined as the level at which the downlink radio link quality can be significantly more reliably received than at Qout and shall correspond to the in-sync block error rate. If SSB is configured as RLM-RS, Qin shall correspond to 2% block error rate of a hypothetical PDCCH.

The physical layer in the UE shall in frames where the radio link quality is assessed indicate out-of-sync (OOS) to higher layers when the radio link quality is worse than the threshold Qout for all resources in the set of resources for radio link monitoring [2]. When the radio link quality is better than the threshold Qin for any resource in the set of resources for radio link monitoring, the physical layer in the UE shall in frames where the radio link quality is assessed indicate in-sync (IS) to higher layers.

If higher layers in the UE receive N310 consecutive OOS indications, T310 timer will start. Before expiry of T310, if higher layers receive N311 consecutive IS indications, T310 stops. Otherwise, UE declares RLF if T310 expires [3].

An example of RLM procedure in NR is shown in the following figure.

Figure 1. An example of RLM procedure in NR

Simulation Scenarios and Evaluation Metrics

This simulation focuses on the dense-urban (macro-layer) deployment scenario, in which all macro cells are on the same frequency layer. The number of unique RLM-RS resource configurations per BWP is set to X. For example, when SSB is configured as RLM-RS resources, a case for X=4 is illustrated in Figure 2 below.

Figure 2. A case for X=4

As for quantitative analysis, four metrics below are used:

- : The probability of indicating OOS to higher layers per indication period, which can be calculated according to the following equation,

: The probability of not indicating any indication to higher layers per indication period, which can be calculated according to the following equation,

- : The probability of indicating IS to higher layers per indication period, which can be calculated according to the following equation,

- : The total number of RLF occurrences among all the cell users over evaluation period.

Analysis of Simulation Results

In RLM procedure, the threshold Qout and Qin are correspond to 10% block error rate of a hypothetical PDCCH and 2% block error rate of a hypothetical PDCCH. In order to simplify the quantitative analysis on the relationship between LBT failure probability and the number of RLF occurrences, we just assess the CDF curve of cell users’ SINR under the assumption that UEs always access to the beam with maximum RSRP. Note that this simplification would not change the overall trend of simulation results. More details of the simulation assumptions can be found in annex A and annex B.

The simulation results are shown in the figure below.

Figure 3. LBT failure probability vs. RLF occurrences

The following table summarizes the simulation results.

Table 1. Summary of simulation results

| The total number of RLF occurrences |  | LBT failure probability |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | =0 | =0.1 | =0.2 | =0.3 | =0.4 | =0.5 | =0.6 |
| X=1 | Cell center users | 95 | 387 | 1024 | 1884 | 2834 | 3581 | 4153 |
|  | Cell edge users | 2235 | 3069 | 3532 | 3929 | 4226 | 4425 | 4516 |
| X=2 | Cell center users | 0 | 0 | 8 | 139 | 738 | 1907 | 3167 |
|  | Cell edge users | 64 | 300 | 881 | 1721 | 2695 | 3487 | 4088 |
| X=3 | Cell center users | 0 | 0 | 0 | 17 | 265 | 1194 | 2619 |
|  | Cell edge users | 0 | 5 | 80 | 459 | 1361 | 2531 | 3539 |
| X=4 | Cell center users | 0 | 0 | 0 | 6 | 155 | 954 | 2384 |
|  | Cell edge users | 0 | 0 | 6 | 111 | 666 | 1806 | 3085 |
| X=5 | Cell center users | 0 | 0 | 0 | 5 | 127 | 858 | 2294 |
|  | Cell edge users | 0 | 0 | 1 | 35 | 367 | 1402 | 2761 |
| X=6 | Cell center users | 0 | 0 | 0 | 4 | 110 | 822 | 2254 |
|  | Cell edge users | 0 | 0 | 0 | 16 | 232 | 1149 | 2577 |
| X=7 | Cell center users | 0 | 0 | 0 | 4 | 107 | 812 | 2243 |
|  | Cell edge users | 0 | 0 | 0 | 8 | 186 | 1003 | 2442 |
| X=8 | Cell center users | 0 | 0 | 0 | 3 | 107 | 804 | 2223 |
|  | Cell edge users | 0 | 0 | 0 | 7 | 144 | 926 | 2371 |

As shown in Figure 3 and Table 1, there is a positive correlation between the LBT failure probability and the total number of RLF occurrences among all the cell users, regardless of cell centre users or cell edge users. For instance, the number of RLF occurrences increase largely from 387 to 1024 (164.6%) for the cell centre users and the number of RLF occurrences will increase slightly from 3069 to 3532 (15.1%) for the cell edge users, when the number of RLM-RS resource configuration is set to 1 and LBT failure probability increases from 0.1 to 0.2. This is because LBT failure will cause the RLM-RS resources cannot be successfully transmitted within an indication period. The physical layer in UE indicates OOS to high layers, which helps to accelerate the pace of RLF declaration.

Observation 1: The total number of RLF occurrences has positive correlation with LBT failure probability.

Observation 2: LBT failure has greater impact on cell center users than cell edge users.

Give that the LBT failure probability is 10%. If the maximum number of RLM-RS resource configurations is set from 1 to 2, the number of RLF occurrences will decrease largely from 387 to 0 (100%) for the cell centre users and the number of RLF occurrences will decrease largely from 3069 to 300 (90.2%) for the cell edge users. This means that the more the number of RLM-RS resource configurations, the users are less likely to declare RLF. The reason is that the probability of indicating OOS will decrease and the probability of indicating IS will increase when the number of RLM-RS resource configurations increases.

Observation 3: Configuring more number of RLM-RS resource configurations within the indication period helps to reduce the total number of RLF occurrences due to LBT failure.

Regarding the number of RLM-RS resource configurations, although more configured RLM-RS resources can reduce the number of RLF occurrences due to LBT failure, the extra benefit against radio resource overhead, sophisticated design, and unnecessary power consumption needs to be carefully considered. For example, given that the LBT failure probability is 40%, when the maximum number of RLM-RS resource configurations is set from 5 to 8, for the cell center users, the total number of RLF occurrences will only decrease by 15.8%.

Observation 4: When the maximum number of RLM-RS resource configurations is larger than 5, additional RLM-RS transmission opportunities to mitigate the impact of missed RLM-RS transmissions due to LBT failure may be trivial and high-cost.

The corresponding text proposal capturing the simulation results can be found in [3].

Proposal 1: To capture the simulation results in TR 38.889.

Based on the observations given above, we think that RAN2 should discuss further on the RLM procedure in NR-U in order to make progresses on mobility aspects of NR unlicensed. Some potential solutions to reduce the number RLF occurrences due to LBT failure should be studied.

Potential solutions

According to the Observation 3 mentioned above, a potential solution to reduce the false RLF detection due to LBT failure is to increase the transmission opportunities for RLM-RS resources during the indication period. In our understanding, we can increase the RLM reference signaling configured for the UE in both time domain and frequency domain, as illustrated in the Figure 4. In time domain, we can increase the number of configured RLM-RS resources during the RLM indication period. In frequency domain, RLM-RS(s) in each indication period can be configured in different frequencies (e.g. different frequency channels within a BWP and different BWP(s)).

The drawbacks of increasing the number of RLM-RS resource configurations are the increased resource cost, the increased complexity, and the increased power consumption of RLM-RS monitoring.

Figure 4. More transmission opportunities for RLM-RS(s) in both time domain and frequency domain

Another potential solution is to allow the UE to detect that the configured RLM-RS resource is missed due to LBT failure via the UE measurement. For example, if the receive signal quality metric (e.g. block error rate of a hypothetical PDCCH) of the RLM-RS is below a threshold (e.g. Threshold_1 given in Figure 2), the physical layer in the UE will indicate LBT failure to the higher layer, rather than OOS. Therefore, the false RLF due to the LBT failure can be avoided. This solution can also reduce the number of RLM-RS resource configurations to save the network resource and the UE power. Last but not least, we consider that if higher layers receive N312 consecutive LBT failure indications, UE will declare RLF. The value of N321 needs to be discussed further in RAN2.

However, a quite obvious defect of this solution is that the complexity of the UE to detect the RLM-RS is missed due to LBT failure. How to differentiate the temporary air-interface blockage between the UE and the gNB and the LBT failure should be further studied.

Figure 5. A potential criterion for indicating IS/OOS/LBT failure

Proposal 2: Increase the transmission opportunities for RLM-RS resources during the indication period via following solutions:

- More RLM-RS resources can be configured within the indication period.

- More RLM RS resources can be configured in different frequency domains (e.g. different frequency channels within a BWP and different BWP(s)).

Proposal 3: If the measurement result of the RLM-RS is below a threshold, the physical layer in the UE would be able to indicate LBT failure.

Proposal 4: If higher layers receive N312 consecutive LBT failure indications, UE will declare RLF.

Proposal 5: To capture the solutions above in TR 38.889.

Conclusions

Based on the evaluations given above, we have the following Observations and Proposals：

Observation 1: The total number of RLF occurrences has positive correlation with LBT failure probability.

Observation 2: LBT failure has greater impact on cell center users than cell edge users.

Observation 3: Configuring more number of RLM-RS resource configurations within the indication period helps to reduce the total number of RLF occurrences due to LBT failure.

Observation 4: When the maximum number of RLM-RS resource configurations is larger than 5, additional RLM-RS transmission opportunities to mitigate the impact of missed RLM-RS transmissions due to LBT failure may be trivial and high-cost.

Proposal 1: To capture the simulation results in TR 38.889.

Proposal 2: Increase the transmission opportunities for RLM-RS resources during the indication period via following solutions:

- More RLM-RS resources can be configured within the indication period.

- More RLM RS resources can be configured in different frequency domains (e.g. different frequency channels within a BWP and different BWP(s)).

Proposal 3: If the measurement result of the RLM-RS is below a threshold, the physical layer in the UE would be able to indicate LBT failure.

Proposal 4: If higher layers receive N312 consecutive LBT failure indications, UE will declare RLF.

Proposal 5: To capture the solutions above in TR 38.889.

Reference

[1] 3GPP TS 38.133, “Requirements for support of radio resource management”.

[2] 3GPP TS 38.213, “Physical layer procedures for control”.

[3] 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”.

[4] R2-18014268, “Text proposals for the evaluation results of the RLM for NR-U”, vivo.

Annex

- Annex A: Simulation assumptions for RLM procedure

Table 2. Initial parameters for RLM

| Attributes | Values or assumptions |
| --- | --- |
| N310 | 4 |
| N311 | 4 |
| T310 | 500 ms |
| The number of RLM-RSs (X) | Up to 8 |
| RLM-RS resource | SSB |
| SSB periodicity | 10 ms |
| Indication period | 10 ms |
| Evaluation period | 5 s |
| The number of gNBs | 19 |
| The number of users in the network | 570 (19*3*10) |
| Qout mapping to SINR | -7.3 dB |
| Qin mapping to SINR | -4.3 dB |

- Annex B: Simulation assumptions for system-level simulation

Table 3. System parameters of system-level simulation

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Mode | DL only |
| Bandwidth | 20 MHz |
| Subcarrier Spacing | 15 kHz |
| Channel Model | UMa in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT. |
| Beam selection | Based on the criteria of maximizing receive power after beamforming. |
| ISD | 200 m |
| BS Tx power | 33 dBm |
| Minimum distance between gNB and UE | 35m |
| BS Antenna Configuration | (Mg, Ng, M, N, P) = (1,1,2,1,2) (dH, dV) = (0.5, 0.5)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (Mg, Ng, M, N, P,) = (1, 1, 1, 1, 2); (dV,dH) = (0, 0)λ. The polarization angles are 0 and 90 |
| UE array orientation | ΩUT,uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | Omni |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |

Based on the simulation assumptions listed above, system-level simulation can be carried out to derive the CDF of cell center users and cell edge users, respectively. Note that the distance between cell center user and gNB is uniform distribution between 35 and 0.9*2/3*ISD, while the distance between cell edge user and gNB is uniform distribution between 0.9*2/3*ISD and 2/3*ISD in this contribution.

The CDF curves of cell center users and cell edge users are shown in the following figure.

Figure 4. CDF of cell users’ SINR