3GPP TSG-RAN WG2 Meeting #104	R2-1818376

Spokane, USA, 12 - 16 November 2018

Agenda item:	11.6.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Further considerations on Mobility impacts for LEO based networks

WID/SID:	FS_NR_NTN_solutions - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN agreed the SI – “Study on solutions evaluation for NR to support Non Terrestrial Network” in [1]. This contribution captures some of the issues relevant to NTN mobility scenarios for LEO.

As mentioned in [3] we propose RAN2 to prioritize the LEO satellite scenarios, as it is most challenging from a mobility point of view. Further in [4], we have captured mobility issues related to moving satellite beam footprint. This contribution further lists additional mobility related issues related to LEO satellite scenarios that should be considered by RAN2.

# 2	Considerations on Mobility impacts

## 2.1 NG Interface

In case of full gNB on the satellite, shown in Figure 1, which is one of the options considered for NTN SI in [3], while the CN is on earth, the consequence of the satellites moving is that the NG interface changes frequently. Every time a satellite moves away, the NG interface is teared down and when a satellite appears at the horizon it may be setup again. As the satellite paths and the timing are highly predictable there maybe methods to lower the signalling load for setting up and tearing down the NG interface, which may have an impact on potential connection/handover failures and interruptions.

CN

CN

gNB

-

NG

Figure 1: Regenerative architecture with full gNB on satellite

Proposal 1: RAN2 to study potential modifications on the NG tear down and setup and their impact on connection failures and interruptions.

## 2.2 F1 interface

In case of a split architecture with DU on the satellite and CU on the ground, the movement of the satellite leads to frequent tear down and setups of the F1 interface. Utilising the predictability of the satellite movements, these methods can be optimised.

Figure 2: Regenerative architecture with DU on satellite and CU on the ground

Proposal 2: RAN2 to study potential impacts of F1 interface tear down.

## 2.3 Neighbour cell list

As the satellites and the cells move continuously, neighbouring cells may also change, depending on the satellites’ constellations.

This has several implications, some are listed below:

- HO candidate cells change, when evaluating potential HOs it needs to be taken into account whether the candidate is still valid and potentially for how long.

- In case of full gNB on the satellite, the X2 interface changes frequently.

- In case intercell mechanisms, e.g. when interference coordination schemes are deployed, they require frequent adjustments.

Proposal 3: RAN2 to study potential modifications to the above issues and potential optimisations.

## 2.4 Radio measurements

The considerations in [4] for moving satellite beam footprint also apply for radio measurement configurations. The RSRP, RSRQ, CSI-RS measurement reports need to be configured for each UE by the RAN. The optimal measurement configuration might be different in the Scenarios C2/D2 compared to Scenarios C1/D1, especially for the measurements needed for inter-satellite beam footprint cases.

In case of the measurements needed for intra-satellite beam footprint HO or Radio Link quality estimation, the radio measurements are performed at higher S(I)NR compared to the measurements for inter-satellite beam footprint. The associated measurement uncertainties in these two cases need to be considered when setting the parameters to achieve good reliability with minimum feedback (from UE) overhead and signalling latency (to/from UE).

Radio Link Failure conditions might impact differently the performance at the inter-satellite beam footprint areas in Scenario A compared to Scenario B.

Proposal 4: In the LEO scenarios with moving or fixed beam footprint on Earth, RAN2 to study the suitability of the existing 5G NR network controlled mobility measurements options for optimal configuration in inter-satellite beam footprint mobility cases.

## 2.5 Cell search, cell selection and cell reselection

The 5G NR cell search procedure (synchronisation to PSS and SSS and decoding of PBCH to get MIB) shall be used as baseline also in NTN scenarios. The cell search can be further optimized in case the UE has a priori configuration of allowed/barred radio cells (PCIs) and carrier frequencies priorities.

The cell (re)selection mobility is specified for NR-RRC IDLE and NR-RRC INACTIVE modes. The cell (re)selection mobility can also be further optimized in case the UE has been configured with the allowed/barred radio cells (PCIs) and carrier frequency priorities for a given geographical area. The parameters controlling the cell selection criteria need to be optimized for NTN scenarios. In case of LEO satellites, for both fixed and moving satellite beam footprint cases, the optimal (re)selection parameters depend on the expected (deterministic) signal variations due to the movement of the satellites and the location of the UE on Earth.  The ‘best’ and ‘worst’ UE locations on Earth with respect to the LEO satellite orbits might need to be estimated for a general (any UE) parameter configuration. When UE location information is available it can greatly help to fine tune the (re)selection parameters.

Proposal 5: In the LEO scenarios with moving or fixed beam footprint on Earth, RAN2 to study the suitability of the existing 5G NR cell (re)selection mechanisms for best UE parameter configuration regardless of its location on Earth with respect to the LEO satellite orbits.

# 3	Conclusion

Proposal 1: RAN2 to study potential modifications on the NG tear down and setup and their impact on connection failures and interruptions.

Proposal 2: RAN2 to study potential impacts of F1 interface tear down.

Proposal 3: RAN2 to study potential modifications to the above issues and potential optimisations.

Proposal 4: In the LEO scenarios with moving or fixed beam footprint on Earth, RAN2 to study the suitability of the existing 5G NR network controlled mobility measurements options for optimal configuration in inter-satellite beam footprint mobility cases.

Proposal 5: In the LEO scenarios with moving or fixed beam footprint on Earth, RAN2 to study the suitability of the existing 5G NR cell (re)selection mechanisms for best UE parameter configuration regardless of its location on Earth with respect to the LEO satellite orbits.

# References

- RP-181370 Study on solutions evaluation for NR to support Non Terrestrial Network, Thales.

- R3-186257 RAN3 101bis Email: # 1_NTN_Mob_mgmt_principles, Thales

- R2-1818369 NTN architecture and scenarios to be prioritized, Nokia, Nokia Shanghai Bell.

- R2-1818372 NTN Mobility aspects for LEO based networks, Nokia, Nokia Shanghai Bell.