3GPP TSG-RAN WG2 Meeting #105	R2-1900616

Athens, Greece, 25th of February – 1st of March 2019	revision of R2-1817688

Agenda item:	12.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on failure recovery in LTE

WID/SID:	LTE_feMob-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN#80 plenary approved a Work Item (WI) entitled “Even further Mobility enhancement in E-UTRAN” to address mobility optimizations that were not included in Release 15 [1]. One of the objectives of the WI is to:

- Specify further enhancements to achieve following targets, [RAN2/3]

  - reduce user data interruption during handover, which targets as close as possible to 0ms, i.e. relaxed requirements could be considered.

  - improve the robustness during handover

Having optimizations that minimize the number of handover and radio link failures should be given the highest priority. However, such failures cannot be avoided for all UE speeds and use cases. As such, it is also important to optimize failure recovery to minimize user data interruption time.

# 2	Baseline failure recovery mechanisms

Handover could fail for many reasons. One of the main scenarios is a Radio Link Failure (RLF) while Time to Trigger (TTT) is running. The RLF may occur before Measurement Report (MR) is triggered, or after the MR is sent but due to poor radio link the Handover Command (HO CMD) is not successfully received.

Considering the envisioned dense deployment and use cases with high UE speed, it will be difficult to achieve zero handover and radio link failures. Thus, it is important to have as well efficient mechanisms for failure recovery. The baseline failure recovery mechanisms, including the enhancements in NR that can be applied for LTE, are discussed below.

## 2.1	Failure recovery through NAS signalling

Failure recovery using NAS signalling is usually applied as a last resort if recovery through AS signalling fails, e.g. if RRC Reestablishment procedure fails. This is done by discarding the AS context in the UE and initiating RRC Setup procedure to create a new AS context. To reduce interruption time, for example in NR, the network can immediately send RRC Connection Setup when it realizes RRC Reestablishment procedure has failed as shown in Figure 1 [2].

## 2.2	Failure recovery through RRC signalling

As the objective of failure recovery is to re-establish the RRC connection between the UE and the RAN, it will be natural to achieve this using RRC signalling. RRC Reestablishment procedure is used to this end, which is shown in Figure 2 with possible enhancement for early activation of SRB1 so that RRCReestablishment can be transmitted using SRB1 [3]. With the early activation of SRB1, RRCReconfiguration can already be sent in RA msg 4, either multiplexed with RRCReestablishment or as a separate message after RRCReestablishment is transmitted, thus reducing the data transmission interruption by a round trip latency. When UE decides the radio link to the source eNB fails, or the MCG link fails in dual-connectivity, it initiates RRC Reestablishment procedure to the best available cell. RRC Reestablishment failure occurs only if valid UE AS context cannot be found to verify the UE.

Observation 1: It is beneficial to adopt NR recovery enhancements in LTE.

Observation 2: Enhancement for early activation of SRB1 is beneficial to speed up failure recovery.

Figure 1: RRC re-establishment, fallback to RRC establishment, successful

Figure 2: RRC connection re-establishment, with potential enhancement for early activation of SRB1 [3], successful

# 3	Factors affecting interruption time during failure recovery

Minimizing the interruption time during HO or failure recovery is important due to its impact on UE service quality. We will discuss factors that affect the interruption time during failure recovery.

## 3.1	The type of signalling used for recovery

The signalling type used for failure recovery has a significant impact on the interruption time. Using NAS signalling leads to the largest interruption time as it involves Core Network (CN) entities and it is accompanied by creating new AS context. Therefore, it is important to use RRC signalling to recover failure whenever it is possible.

Proposal 1: Failure recovery using NAS signalling should be avoided if the recovery can be achieved using RRC signalling.

## 3.2	System information availability

System Information (SI) acquisition may have considerable impact on failure recovery latency. After RLF/HOF, the UE needs SI of a potential target cell for recovery to check if the cell is suitable for cell reselection and obtain the necessary radio resource configurations for performing RACH access to initiate RRC Connection Reestablishment procedure. If the failure occurs after the UE has received Handover command, which contains SI of the target cell, the UE can immediately start the RACH procedure to the target cell. Otherwise, the UE need to read SI of potential target cells. However, system information acquisition usually takes time since MIB, SIB 1 and SIB 2 are transmitted periodically, increasing the failure recovery latency.

Proactive provisioning of system information of the potential target cells to UE can reduce the failure recovery latency due to system information acquisition. The simplest approach to do this would be for the serving cell to provide the SI of all neighbouring cells to all the UEs it is serving. However, this blind approach wastes network resources and most of the provided SI of neighbour cells are left unused as the majority of UEs won’t encounter RLF. Therefore, it would be important to optimize provisioning of SI to reduce its signalling overhead impact. For example, this can be done with the assistance of MRO to provide SI of a subset of the neighbouring cells and UEs that are prone for failure.

Observation 3: System information acquisition have an impact on failure recovery if the UE does not have SI of the potential target cell for recovery.

Proposal 2: The network may provide SI of potential target cells for failure recovery to reduce failure recovery latency. However, optimizations to minimize its impact on the network, for example with the assistance of MRO, should be considered.

## 3.3	Declaration of RLF/HOF

The UE does not declare RLF immediately as link quality may improve or to avoid a false alarm due to unstable measurement. Due to this, when a bad link is detected, the UE waits for a specific time, i.e. T310 duration, before declaring an RLF. However, the link may not be recovered. In this case, the UE must wait for the duration of T310 before it even declares RLF and thus before initiating the recovery procedure. Therefore, it may be beneficial to expedite the declaration of RLF if it is done with minimal false alarm. However, if the early declaration only happens after a measurement report has been sent out by the UE and T310 was already running, we can assume that it won’t lead to false alarm.

The timer T312, and its extensions such as the one discussed in [4], are used to expedited failure declaration to speed up the recovery process. The timer T312 is initiated when measurement report has been triggered while T310 is running. An enhancement of T312 is discussed in [4] that allows the initiation of the timer even if the measurement report is triggered before the start of T310, leading to effectively initiating T312 timer at the same time as T310 [4]. It should be noted, however, the time elapsed between the triggering of the measurement report and the initiation of T310 might be too long that the condition for triggering the measurement report may not hold anymore. To avoid such mismatch, it is beneficial to check if the condition for triggering the measurement report still holds before starting the T312 timer, for example, if the time elapsed between the triggering of the measurement report and the start of T310 is larger than a given threshold.

Proposal 3: Timer T312 is started as early as when T310 starts running as long as measurement report has already been triggered and the condition for triggering the measurement report still holds at the start of T310.

## 3.4	Expedited conditional handover

When the link between UE and source eNB is poor and there is a prepared cell for handover, expediting the handover process might be useful to avoid RLF and recover from a potential handover failure. In conditional handover, there would be at least one cell prepared for handover. However, there is a risk that the link between the UE and the source cell fails while the UE is waiting for the condition to execute handover is met. Therefore, it is beneficial to initiate handover towards a prepared cell as early as when the condition for initiating T310 is met, even if the condition for initiating handover execution is not met yet, to avoid RLF and minimize the service interruption time. How early the expedited handover is executed needs to be optimized to balance the trade-off between avoiding too early handover execution and reducing handover failure.

Proposal 4: UE may initiate handover execution towards a prepared cell as early as when the condition to initiate T310 is met even if the condition for handover execution is not met.

## 3.5	Suitable cell selection for RRC recovery

Measurements need to be averaged to filter out the effect of fading and measurement errors before they can be reliably used for mobility decisions. However, if the measurement periodicity is large, e.g. due to large DRX cycles, it takes some time to get a reliable measurement. Thus, the selection of a suitable cell for failure recovery might delay the recovery if reliable measurement cannot be obtained on time.

Observation 4: The selection of a suitable cell for failure recovery contributes to recovery latency, and its severity depends on how fast a reliable measurement can be obtained.

## 3.6	UE AS context availability

One of the factors that affects recovery latency is whether UE AS context is available in the selected cell for recovery. When UE initiates failure recovery, the target eNB for recovery needs to have UE’s AS context for the recovery to succeed. If the context has been proactively provided to the target eNB, for example, as part of HO preparation, the recovery procedure can be successfully completed solely using signalling exchange between the UE and the new eNB. However, if the new eNB does not have UE context, it needs to fetch UE context from the source cell for successful recovery. This incurs additional interruption especially if a non-ideal interface is used between the target eNB and the source eNB.

To minimize failure recovery latency, cells with UE context that the UE is aware of should be prioritized for failure recovery over cells that do not have UE context. For example, if a UE has been prepared for conditional handover with multiple target cells, it is already aware of the cells with UE context. Similarly, if the network proactively provides UE context to a subset of neighbouring cells to assist fast recovery, it may also indicate to the UE the list of prepared cells. Failure recovery towards such a cell, although it might not have the strongest signal strength, leads to faster failure recovery than a recovery to a cell with no UE AS context.

One approach to prioritize cells with UE context for recovery is to apply different T312 timer values, with lower value used for recovery towards cells with UE context. Therefore, the UE can be configured with two T312 values, let us call them T1 and T2, with T1 < T2 to be used when recovering towards a cell with UE context and T2 is used when there is no UE context in the target cell. This implies, if an aggressive value of T1 = 0 is used, the UE can immediately start failure recovery towards a cell with UE context as soon as the condition for T310 triggering is met and measurement report was triggered.

Proposal 5: UE should prioritise re-establishment towards cells with proactively prepared UE AS context. Different T312 timer values can be used for different cells to prioritise and speed up re-establishment towards cells with UE context.

# 4	Conclusion

The contribution discussed failure recovery for LTE. Several factors that might impact the recovery process are discussed, and the following observations and proposal are made:

Observation 1: It is beneficial to adopt NR recovery enhancements in LTE.

Observation 2: Enhancement for early activation of SRB1 is beneficial to speed up failure recovery.

Observation 3: System information acquisition have an impact on failure recovery if the UE does not have SI of the potential target cell for recovery.

Observation 4: The selection of a suitable cell for failure recovery contributes to recovery latency, and its severity depends on how fast a reliable measurement can be obtained.

Proposal 1:  Failure recovery using NAS signalling should be avoided when it is possible to recover from the failure using RRC signalling.

Proposal 2: The network may provide SI of potential target cells for failure recovery smartly to reduce failure recovery latency. This should be done in a smart way, for example with the assistance of MRO, to minimize its impact on the network.

Proposal 3: Timer T312 is started as early as when T310 starts running as long as measurement report has already been triggered and the condition for triggering the measurement report holds at the start of T310.

Proposal 4: UE may initiate handover execution towards a prepared cell as early as when the condition to initiate T310 is met even if the condition for handover execution is not met.

Proposal 5: UE should prioritise re-establishment towards cells with proactively prepared UE AS context. Different T312 timer values can be used for different cells to prioritise and speed up re-establishment towards cells with UE context.

# References

- RP-181544, Revised WID: Even further mobility enhancement in E-UTRAN, 3GPP TSG-RAN Meeting #81, Gold Coast, Australia, September 10-13, 2018

- 3GPP TS 38.331, NR; Radio Resource Control (RRC); Protocol specification, December 2018

- R2-1814322, Enhancements to re-establishment procedure in LTE, 3GPP TSG-RAN WG2#103bis, Chengdu, China, 8th – 12th October 2018

- R2-1814323, Enhancing the handling of timer T312, 3GPP TSG-RAN WG2#103bis, Chengdu, China, 8th – 12th October 2018

- R2-1805012, Reestablishment procedure for NR, 3GPP TSG RAN WG2 Meeting #101bis, Sanya, China, 16th - 20th April 2018