3GPP TSG-RAN WG2 Meeting #105	R2-1901075

Athens, Greece, 25th February - 1st March 2019

Agenda Item:	11.4.3

Source: 	MediaTek Inc.

Title:  	Inter-RAT control of sidelink communication

Document for:	Discussion and decision

Introduction

During RAN1#94bis and Ad-Hoc Meeting 1901, the group reached some agreements related to the inter-RAT control of NR and LTE sidelink:

Agreements:

It is supported that LTE Uu provides at least necessary semi-static configuration for NR mode-2 SL communications

FFS details

Further study impact and benefits of LTE Uu managing NR mode-1 SL communications

Agreements:

It is supported that NR Uu provides necessary semi-static configuration for mode-4 LTE SL communications

From RAN1 perspective, signalling should be similar to LTE in terms of UE-specific or cell-specific

Signalling details up to RAN2

Further study feasibility, benefits (others than ones already identified for LTE) and impact of NR Uu managing LTE mode-3 SL communications.

Conclusion:

- Send LS to RAN2 informing them the above two agreements:

  - For the 2nd agreements, ask RAN2 to work on signalling

  - For the 1st agreements, RAN1 is continuing working on identifying necessary details, including parameters

- Draft LS in R1-1812005 (Phillippe, Huawei), which is approved and final LS in R1-1812057

Agreements:

- LTE Uu to schedule NR sidelink mode 1 is supported:

  - The support is done based on type 1 configured grant with configuration restricted to time/frequency resources & periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu

  - Both DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1

- Send an LS to RAN2 – Phillipe (HW), R1-1901410, which is approved with the final LS in R1-1901445

This document considers these agreements and their consequences from a RAN2 perspective.

Discussion

Mode 2 and mode 4

Based on the RAN1 agreements, RAN2 will need to define signalling for the cross-RAT support of mode 2 (NR) and mode 4 (LTE) communication.  While the necessary parameters for control of NR sidelink are still under discussion, RAN1 indicated that the parameters for LTE sidelink control in NR should be similar to those in LTE, and RAN2 can start to consider the related signalling.

RAN1 requested both UE-specific and cell-specific support, similar to the inclusion in LTE of sl-V2X-ConfigDedicated in the RRCConnectionReconfiguration message and the cell-specific information in SIB21.  This should be straightforward to match in the NR environment with signalling in the RRCReconfiguration message and an LTE-V2X SIB.

The dedicated signalling for the requested mode 4 support does not fully duplicate what is there in LTE; rather, the needed signalling would be similar to the existing ue-Selected branch and the general configurations in SL-V2X-ConfigDedicated.  The actual description of a resource pool need not change; probably SL-CommResourcePoolV2X can be imported into the NR ASN.1 as it is, although the details would need to be determined in the WI phase.

Proposal 1: RAN2 confirms the feasibility of signalling UE-specific LTE-V2X configuration for mode 4 in dedicated NR RRC signalling (e.g. RRCReconfiguration).

Similarly, the signalling for cell-specific V2X support can closely follow the LTE model, nearly or entirely duplicating the content of LTE SIB21.

Proposal 2: RAN2 confirms the feasibility of signalling cell-specific LTE-V2X configuration for mode 4 in broadcast NR RRC signalling (e.g. a new LTE-V2X SIB).

Note that these proposals suggest a significant migration of LTE ASN.1 code into the NR RRC specification.  However, it would also be possible to avoid code duplication by incorporating the LTE IEs as OCTET STRINGs rather than copying their definitions and all the other IEs on which they depend.

Proposal 3: Unless issues are identified, plan to capture the LTE V2X IEs as OCTET STRINGs in the NR ASN.1.

Mode 1

For mode 1 and mode 3, RAN1 concludes that LTE Uu to schedule NR sidelink mode 1 is supported with type 1 configured grant only. DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1, the reason is to reduce the spec complexity.

From the RAN2 perspective, LTE Uu to schedule NR sidelink mode 1 can be support the same manner as mode 2 and mode 4. Due to the detail NR V2X IE is not finalized yet, the detail can left to WI phase.

Proposal 4: The NR V2X IEs can be included as OCTET STRINGs in the LTE ASN.1 as the same mechanism as in mode 2, the detail can left to WI phase.

Conclusion

This document provides the following proposals:

Proposal 1: RAN2 confirms the feasibility of signalling UE-specific LTE-V2X configuration for mode 4 in dedicated NR RRC signalling (e.g. RRCReconfiguration).

Proposal 2: RAN2 confirms the feasibility of signalling cell-specific LTE-V2X configuration for mode 4 in broadcast NR RRC signalling (e.g. a new LTE-V2X SIB).

Proposal 3: Unless issues are identified, plan to capture the LTE V2X IEs as OCTET STRINGs in the NR ASN.1.

Proposal 4: The NR V2X IEs can be included as OCTET STRINGs in the LTE ASN.1 as the same mechanism as in mode 2, the detail can left to WI phase.

References

- R1-1901445, “LS on Uu-based sidelink resource allocation for NR V2X”, LS from RAN1 to RAN2, RAN1 Ad-Hoc Meeting 1901.