3GPP TSG-RAN WG2 Meeting #105	R2-1901404

Athens, Greece, 25th February-1st March 2019

Agenda Item:		11.6.1

Source: 		Vodafone

Title: 			IoT Device Density Models for Various Environments

Document for:		Discussion and Decision

# Introduction

Following the previous discussion on NR Non Terrestrial Networks (NTN) held in Spokane (RAN2 #104) we present a realistic and practical Device Density Models for IoT Deployment.

The application of the presented model is not limited to the NTN Technology and it can also be applied to conventional Terrestrial Network coverage scenarios.

# Discussions

In RAN2#104 a number of documents were presented on the projected device or user density per cell in urban and rural environments for NTN applications which were unrealistic.

In this contribution we present a model which has been based on statistical data and realistic projections.

# The Model

In order to arrive at a realistic model for device density per area we used statistical data from the UK’s Office of National Statistics (ONS) [1] where we have combined the data from the following geographical data on distribution of population

- ONS rural / urban population split for 2014 [2]

- ONS data on rural and urban areas dated 2009

Combining the data in the attached ONS report from 2009 and some other data (e.g. average household size of 2. 4 people etc., that can be found at [2,3] we arrive at an estimated device density calculation for various areas in England which is illustrated  in the following  Table 1:

Table 1 IoT Device Density Model for England

| Area type | Devices per square Kilometres |
| --- | --- |
| Urban - Less Sparse (Dense Urban) | 26,255 |
| Urban – Sparse | 3,646 |
| Rural - Town & Fringe - Less Sparse (Semi-Urban) | 3,515 |
| Rural - Town & Fringe - Sparse | 1,340 |
| Rural - VHID - Less Sparse* | 1,226 |
| Rural - VHID - Sparse* | 394 |
| England | 6,628 |

* Definition of VHID: Village, hamlet & isolated dwellings

The figures in Table 1 illustrate a more a realistic number of IoT devices in built up areas (Dense Urban to Semi-Urban) which is in line with our expectations.

Furthermore, ~400 device density per square kilometre of rural area is more reasonable considering the use scenarios and uptake of IoT in rural areas.

Device Density in a Satellite Footprint

In this section we give examples of projected device density in a satellite footprint in various environments, the assumption is that the satellite’s beam has a ground foot print of  approximately 40km diameter as shown in the following Figure 1

Figure 1 Satellite footprint on the ground

The projected Device Density for a typical Satellite footprint is illustrated in the following Table 2

Table 2 Device Density in a Satellite Footprint

| Area type | Devices per individual satellite coverage area** |
| --- | --- |
| Urban - Less Sparse (Dense Urban) | 32,992,569 |
| Urban - Sparse | 4,581,757 |
| Rural - Town & Fringe - Less Sparse (Semi-Urban) | 4,417,498 |
| Rural - Town & Fringe - Sparse | 1,684,188 |
| Rural - VHID - Less Sparse* | 1,541,239 |
| Rural - VHID - Sparse* | 495,687 |
| England | 8,328,991 |

* Definition of VHID: Village, hamlet & isolated dwellings

** Estimated satellite coverage area: 1,257 square km (based on coverage of 40 km in diameter)

Taking the sparsely populated areas as an example, we expect to connect to a maximum of ~ 500,000 IoT devices with a satellite coverage. This figure could be much less than we have estimated here based on the application and use cases in a particular territory.

Proposal and Conclusions

In this contribution we have presented a realistic IoT Device Density model based on geographical and statistical data obtained from the ONS for Dense-Urban to Sparsely Rural environments.

Furthermore, we have presented a projected IoT Device Density model under a satellite footprint with a 40Km cell diameter.

Although the models are based on the UK/England statistics, never the less the model is valid for various dense/developed countries.

We propose to use the figures shown in Table 1 and Table 2 to estimate IoT Device Densities for various environments.

# References

- https://www.ons.gov.uk/

- https://www.gov.uk/government/publications/rural-population-and-migration/rural-population-201415

- https://www.ons.gov.uk/peoplepopulationandcommunity/birthsdeathsandmarriages/families/bulletins/familiesandhouseholds/2016

- https://www.ons.gov.uk/peoplepopulationandcommunity/populationandmigration/populationestimates/bulletins/populationandhouseholdestimatesfortheunitedkingdom/2011-03-21#population-density

# Annex: Calculations behind the IoT Device Density Model

This calculation is also valid for other regions