3GPP TSG-RAN WG2 #105	Tdoc R2-1901560

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.6.3.1

Source:	Ericsson

Title:	On HARQ for NTN

Document for:	Discussion, Decision

# Introduction

In RAN#80, a new SI “Solutions for NR to support Non-Terrestrial Network” was agreed [1]. It is a continuation of the preceding SI “NR to support Non-Terrestrial Networks” (RP-171450), where the objective was to study the channel model for the non-terrestrial networks, to define deployment scenarios, parameters and identify the key potential impacts on NR. The results are summarized in [2]. The new study item has the objective at evaluating potential solutions addressing the minimum necessary identified key impact areas from the previous activity and to study impact on RAN protocols/architecture.

In RAN2#103bis, it is agreed to study the following UP and CP aspects:

UP Impacts to study

- DRX

- HARQ

- Random access response

- RLC/PDCP reordering (e.g. timers and SN space)

- SDAP => no impact

Impacts to study for CP

- Mobility

- TA management and update

In RAN2#104, the following agreements on UP were reached:

RACH Agreements:

-	Discussion on 2-step RACH will be postponed until the procedures are more stable.

HARQ Agreements:

-	Both options (enhancing HARQ and disabling HARQ) will be studied.

RLC Agreements:

-	All RLC modes are supported.

-	Study the need to extend the RLC/PDCP SN and window sizes based on throughput requirements.

In this paper, we discuss HARQ operation.

# Discussion

An important part of modern wireless communication standards is to ensure end-to-end error free packet delivery through retransmissions. NR ensures delivery through several steps of retransmissions on HARQ, RLC and PDCP layers which all have different characteristics and purposes. It has been possible to turn off retransmissions in all or several layers in LTE for important use-cases such as MBMS, SC-PTM and VoLTE. While these are all important use-cases where turning off retransmissions is crucial for the specific use case, we believe that retransmissions should be supported for the satellite use case as well.

- Retransmissions at one or several layers shall be supported for NTN and configurable by the network.

In this contribution we will discuss for what circumstances that HARQ can be useful and when HARQ will see problems due to the large propagation delay associated with non-terrestrial networks.

## HARQ useful scenarios in NTN

### RRC configuration

At initial attach, the UE will use different SRBs to acquire RRC configurations. When the UE sends the RRCSetupRequest (Msg3), SRB0 is used and the RLC mode is transparent mode, TM. The same is valid when the gNB sends the RRCSetup (Msg4) message which is used to setup the SRB1 bearer. Not until RRCSetupComplete is sent from the UE, SRB1 is established and the RLC mode is switched to acknowledge mode, AM. Msg3 and Msg4 make use of HARQ in both UL and DL to help with reliable communication during the RRC configuration. If HARQ is disabled for NTN, it would increase the probability of mismatch between the UE and gNB if the RRC configuration messages are not received and could delay the setup significantly.

HARQ enables reliable RRC configuration at initial attach

### HARQ for small packets and infrequent data

The rate reduction due to not enough HARQ-processes can be quite significant. This is however only the case when a sustained rate is required. For smaller packets arriving from higher layers, resulting in that the required amount of HARQ processes is smaller than the maximum amount of HARQ-processes, then HARQ may still be used to great effect.

The maximum PDCP packet size before HARQ could stall the packet is roughly (#HARQ-processes)*(TBS), which depends on the specific radio environment such as bandwidth, SNR and UE class. Given that the maximum PDCP PDU size is 9kB, a TBS of the size (9kB)/#HARQ-processes = ~4500 bits TBS would be required to be able to deliver one single PDCP packet without stalling the number of HARQ-processes.

The benefit of HARQ for DRBs is two-fold. The first benefit is the HARQ soft-combining gain which in some instance may be very large. Due to the satellite link budget often being a lot more restrained compared to the terrestrial case, the soft-combining gain of HARQ should not be disregarded.

The second benefit is however subtler and ties to the scenario of small and infrequent data. The benefit is due to the dynamic scheduling of HARQ feedback and HARQ retransmissions which can be realized by seeing how HARQ and RLC schedule feedback and retransmissions. This can be seen in an example in figure 2 where the retransmission delay for HARQ would not be more than 1 RTT, but for RLC, in a scenario of no UL resources, the delay would be at least 2 RTT.

Figure 2: a) showing the procedure of retransmission for HARQ, b) showing the procedure for retransmission in RLC AM when HARQ is turned off and UE has no uplink resources.

Relying on RLC for retransmission instead of HARQ may increase the delay compared to using HARQ.

The benefits of HARQ include HARQ soft-combining gain and dynamic fast scheduling of feedback and retransmissions.

## Switching off HARQ

Any system that has a propagation delay larger than the number of available HARQ processes, may suffer from HARQ stalling. In NTN, propagation delays of tens of milliseconds up to hundreds of milliseconds will be seen. Any scenario in NTN that requires piecewise continuous, continuous or even just periodic transmissions where the number of required transmissions is greater than the number of available HARQ processes per RTT, will only be able to use a limited number of scheduling resources. Any MBB or streaming scenario using continuous transmission in NR will be limited to 16/RTT scheduling opportunities. Even for Voice, with a need of periodic resources every ~20ms, will not be handled satisfactory in a GEO system.

Using HARQ for all scenarios in NTN systems will limit the number of use-cases that can be offered

## HARQ enabled or disabled

As discussed in previous meetings, HARQ may be limited due to the amount of HARQ-processes. When the rates needed are much larger than (#HARQ-processes)*(TBS)/RTT, then there could be a need to rely on RLC instead of HARQ.

In a previous contribution [4], it was discussed how HARQ-feedback could be turned off through network implementation. If the goal is to only turn off HARQ-feedback, then the only specification-impact that would be needed would be to turn off the HARQ-feedback in the downlink. However, several features such as DRX, configured scheduling, random access all have procedures to accommodate for HARQ. There is thus a need signal to the network that HARQ-procedures should not be used so that these processes may be able to adjust.

Given that HARQ may be very beneficial in some cases and might have large drawbacks in other cases, it would seem as if a selective way of turning off HARQ could be beneficial.

- RAN2 to consider how to selectively turn off HARQ.

# Conclusion

In section 2 we made the following observations:

Observation 1	HARQ enables reliable RRC configuration at initial attach

Observation 2	Relying on RLC for retransmission may increase the delay compared to using HARQ.

Observation 3	The benefits of HARQ include HARQ-processing gain and dynamic scheduling of feedback and retransmissions.

Observation 4	Using HARQ for all scenarios in NTN systems will limit the number of use-cases that can be offered

Based on the discussion in section 2 we propose the following:

Proposal 1	Retransmissions at one or several layers shall be supported for NTN and configurable by the network.

Proposal 2	RAN2 to consider how to selectively turn off HARQ.

# References

RP-181370, New SID on Solutions for NR to support Non Terrestrial Network, Thales, RAN#80, La Jolla, USA, June 2018

TR 38.811, Study on New Radio (NR) to support non terrestrial networks, Release 15

TR 38.821, Solutions for NR to support Non Terrestrial Network

R2-1817764, On HARQ for NTN, Ericsson, RAN#104, Spokane, November 2018