3GPP TSG-RAN WG2 #105	 TDoc R2-1901661

# Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.4.2.1

Source:	Ericsson

Title:	Discussions on NR V2X in DC Scenarios

Document for:	Discussion, Decision

# Introduction

In RAN2#103bis, the following agreements were made:

Agreements

1: 	RAN2 to study and prioritize the Uu control/configuration of NR SL and LTE SL in SA scenarios, i.e. gNB and ng-eNB, as proposed in Figure 1 and 2 (FFS on the support of mode1 for the cross-RAT control).

2:	Capture the figure 3 and 4 into TR but will be deprioritized compared to SA scenarios.

In this contribution, we further discuss the target scenarios of NR V2X SL communication.

# Discussion

In [1] four scenarios were discussed where NR/LTE SL is supported by NR and/or LTE Uu, as shown below. It was proposed that the support of NR/LTE SL under EN-DC (figure 3) and MR-DC (figure 4) is deprioritized, because each node (MN/SN) may only support the SL with the same RAT correspondingly (e.g. LTE Uu controls LTE SL, NR Uu controls NR SL), so that the SL communication may have to be controlled by the SN if only SN uses the same RAT as that of SL, which increases complexity and standadization efforts. However, cross RAT V2X sidelink configurations by LTE/NR Uu was already agreed in RAN1#94bis, which is beneficial to ensure e.g. good sidelink performance when a UE moves between areas served by diifferent Uu RATs. By this the sidelink configurations could always be provided by MN no matter DC is activated or not. Besides, many eV2X services are safety related, it is harmful if the safety servives have to be dropped when DC is activated over Uu. Therefore, we think the DC scenario should be studied in RAN2 with prioritizing the case where the sidelink configurations are always provided by MN.

The sidelink configurations could always be provided by MN no matter DC is activated or not.

It is harmful if the safety servives have to be dropped when DC is activated over Uu.

The DC scenario should be studied in RAN2 with prioritizing the case where the sidelink configurations are always provided by MN.

Figure 1: SA NR with gNB			Figure 2: SA NR with ng-eNB (connected to 5GC)

Figure 3: NSA with EN-DC				       Figure 4: MR-DC under 5GC

Besides, even giving that the configurations always come from MN, the SL transmission could be scheduled by SN if the RAT used by SL is the same as that of SN. This is in line with the principle of Uu DC where MN provides the RRC configurations while transmission in each node is scheduled by the corresponding node. By this mode 1/3 scheduling (via. PDCCH) can be enabled in DC scenario and the UE does not have to switch SL transmission mode when (de-)activates DC over Uu.

Mode 1/3 scheduling (via. PDCCH) is performed by the node using the same RAT as that of SL in DC scenarios.

A UE may not be able to perform SL transmissions when DC is activated for UL, due to e.g. limited number of UE Tx chains. In this case prioritization rules may need to be defined to determine how SL transmissions and UL transmissions over MN and SN should be handled. UL/SL prioritization rules introduced in LTE Rel.14 could be used as the baseline.

A UE may not be able to perform SL transmissions and UL transmissions over multiple UL tags simultaneously due to e.g. limited number of UE Tx chains.

Prioritization rules may need to be defined to handle simultaneous SL transmissions and UL transmissions over MN and SN.

Nevertheless, even if prioritization rules may need to be specified to tackle the above issue, in our understanding, it is important if some degree of coordination is enabled between the MN and the SN. That is because the MN and SN may use separate scheduling strategies, and it should be minimized the probability of collisions between UL/SL scheduling decisions from the MN and UL/SL scheduling decision from the SN. Even if prioritization rule and power split rule might need to be specified to ensure proper DC behaviour, it is important that the probability of discarding packets due to such rules is minimized. To achieve that RAN2 and RAN3 work might be needed. For example, the MN could inform the SN (or viceversa) about the SL resources which the MN (or the SN) intends to provide to the UE. In this way, the SN (or the MN) can make sure to do not allocate those resources for UL transmissions. We note that in legacy NR such level of coordination between MN and SN is already present for UL.

RAN2/RAN3 considers exchanging of SL resource allocation between MN and SN to minimize probability of collisions between UL/SL scheduling decisions at MN and SN.

# Conclusion

In section 2 we made the following observations:

Observation 1	The sidelink configurations could always be provided by MN no matter DC is activated or not.

Observation 2	It is harmful if the safety servives have to be dropped when DC is activated over Uu.

Observation 3	A UE may not be able to perform SL transmissions and UL transmissions over multiple UL tags simultaneously due to e.g. limited number of UE Tx chains.

Based on the discussion in section 2 we propose the following:

Proposal 1	The DC scenario should be studied in RAN2 with prioritizing the case where the sidelink configurations are always provided by MN.

Proposal 2	Mode 1/3 scheduling (via. PDCCH) is performed by the node using the same RAT as that of SL in DC scenarios.

Proposal 3	Prioritization rules may need to be defined to handle simultaneous SL transmissions and UL transmissions over MN and SN.

Proposal 4	RAN2/RAN3 considers exchanging of SL resource allocation between MN and SN to minimize probability of collisions between UL/SL scheduling decisions at MN and SN.

# References

R2-1813933, “Scenarios for NR V2X SL communication”, RAN2#103bis, Huawei, HiSilicon