3GPP TSG-RAN WG2 Meeting #105	R2-1902118

Athens, Greece, 25th February – 1st March 2019

Agenda Item	: 11.7.2.2 (FS_NR_IIOT)

Source	: LG Electronics Inc.

Title	: TP on multiple active configured grants

Document for	: Decision

1	Introduction

This contribution provides a text proposal to TR 38.825 to capture that introducing a new MAC CE for CG activation/deactivation command can be considered for either activating and deactivation multiple active CGs per a BWP at a time.

<START>

6.2	TSN use cases, scenarios and architectures

Editor’s note: RAN3 responsibility

6.3	TSN performance evaluation

6.3.1	Requirements

Editor’s note: RAN2 responsibility

6.3.2	Physical layer aspects

Editor’s note: RAN1 responsibility

6.3.3	Protocol aspects

Editor’s note: RAN2 responsibility

6.3.4	Radio access network aspects

Editor’s note: RAN3 responsibility

6.4	Accurate reference timing provisioning

Editor’s note: RAN2 responsibility with potential network interfaces impacts handled by RAN3

6.5	QoS and scheduling enhancements

Editor’s note: RAN2 responsibility with potential PHY impacts handled by RAN1

Configured grant is attractive scheduling methods for TSN traffic because TSN traffic is deterministic and periodical with known traffic patterns. To support multiple TSN traffic simultaneously according to TS 22.804, it is deemed necessary to enhance CG activation/deactivation command. Currently, CG activation/deactivation command is transmitted via PDCCH and there is no explicit indication to indicate which CG configuration among multiple CG configurations is to be activated or deactivated. Therefore, multiple CG configurations on one BWP can only be activated serially and cannot be activated at one time. In order to activate multiple CG configurations on one BWP at a time, CG configuration index should be included in the CG activation/deactivation command. Depending on the maximum number of CG configurations per BWP, introducing a new MAC CE for CG activation/deactivation command can be considered than enhancing PDCCH based CG activation/deactivation command because PDCCH overhead might not be small.

Therefore, the detailed format of CG activation/deactivation command needs to be discussed in WI phase by considering overhead due to CG configuration index and the maximum number of CG configurations per BWP.

<END>