3GPP TSG-RAN WG2 Meeting #105 	R2-1902350

# Athens, Greece, 25 February 2019 – 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0603 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | [Changes in revision 0] The text in the latest MAC specification (i.e. v15.4.0) can be improved by having the following changes: - the reference needs to be corrected; - a space can be added both before and after binary operators; - unnecessary spaces and double quotes (") can be removed; - the field descriptions for the following PHY-related MAC CEs need to be improved (including changing 'should' statement to 'shall' statement, which should be editorial): . SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE; . PUCCH spatial relation Activation/Deactivation MAC CE; . SP SRS Activation/Deactivation MAC CE; . SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE. [Changes in revision 1] - In subclause 6.1.3.9 (Multiple Entry PHR), P̃CMAX,f,c (i.e. with tilde) is not needed as P̃CMAX,c is needed only for the 'Type 1 PH with PUCCH' which is not supported in NR (but still needed for LTE); - The current TS 38.321 uses the terminology 'MAC PDU subheader' mixed with 'MAC subheader', and they can be unified into 'MAC subheader'; - In subclause 5.1.1, Random Access Preambles group B-related parameters are only applicalbe to SpCell, so unnecessary description '(per cell)' and (SpCell only) can be removed for the consistency; - In subclause 5.1.1, ssb-perRACH-OccasionAndCB-PreamblesPerSSB is applicable to SCell for the ssb-perRACH-Occasion, and thus incorrect '(SpCell only)' can be removed; - In subclause 5.7, the descriptions for the DRX ambigious period can be misinterpreted, and needs to add missing preposition 'until' before 4 ms; - In subclause 5.7, as the detailed PDCCH monitoring timing is specified in subclause 11.1.1 of TS 38.213, the reference can be added; - In subclauses 5.4.3.1.3, 5.4.5, and 5.4.6, the description for the configuration of boolean parameters (i.e. skipUplinkTxDynamic, logicalChannelSR-DelayTimerApplied, multiplePHR, and phr-Type2OtherCell) can be clarified that it is configured with value true; - In subclause 5.3.2.1, the word 'dynamic' should be removed to cover DL configured assignment as specified in subclause 5.1.2.1 of TS 38.214. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [Changes in revision 0] - In subclause 5.4.2.1, the reference is corrected; - a space is added both before and after binary operators; - Throughout subclauses 6.1.3, unnecessary spaces and double quotes (") are removed; - The field descriptions for the following PHY-related MAC CEs are improved: . SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE; . PUCCH spatial relation Activation/Deactivation MAC CE; . SP SRS Activation/Deactivation MAC CE; . SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE. [Changes in revision 1] - In subclause 6.1.3.9, (unused) P̃CMAX,f,c (i.e. with tilde) is removed; - The terminology 'MAC PDU subheader' is updated to 'MAC subheader'; - In subclause 5.1.1, '(per cell)' and '(SpCell only)' for the group B-related parameters are removed; - In subclause 5.1.1, '(SpCell only)' for ssb-perRACH-OccasionAndCB-PreamblesPerSSB is removed; - In subclause 5.7, missing 'until' are added; - In subclause 5.7, the reference for the PDCCH monitoring is added. - In subclauses 5.4.3.1.3, 5.4.5, and 5.4.6, it is clarified that the configuration of boolean parameter means the configuration with value true; - In subclause 5.3.2.1, the word 'dynamic' is removed. Impact analysis Impacted 5G architecture options: Standalone and EN-DC. Impacted functionality: General MAC (no functional changes). Inter-operability: If only the network is implemented according to the CR, no interoperability problems are foreseen. If only the UE is implemented according to the CR, no interoperability problems are foreseen. This is because the CR does not require any implemenation change for both UE and NW side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains with editorial problems, wrong references, inconsistency with other specification, and unused parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.3, 5.3.2.1, 5.4.1, 5.4.2.1, 5.4.3.1.3, 5.4.5, 5.4.6, 5.7, 5.8.2, 5.18.1, 5.18.2, 6.1.3.1, 6.1.3.2, 6.1.3.3, 6.1.3.4, 6.1.3.5, 6.1.3.6, 6.1.3.7, 6.1.3.8, 6.1.3.9, 6.1.3.10, 6.1.3.11, 6.1.3.12, 6.1.3.13, 6.1.3.14, 6.1.3.15, 6.1.3.16, 6.1.3.17, 6.1.3.18, 6.1.3.19, 6.1.3.20, and 6.2.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

RRC configures the following parameters for the Random Access procedure:

-	prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble;

-	preambleReceivedTargetPower: initial Random Access Preamble power;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	candidateBeamRSList: a list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated Random Access parameters;

-	recoverySearchSpaceId: the search space identity for monitoring the response of the beam failure recovery request;

-	powerRampingStep: the power-ramping factor;

-	powerRampingStepHighPriority: the power-ramping factor in case of prioritized Random Access procedure;

-	scalingFactorBI: a scaling factor for prioritized Random Access procedure;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: defines PRACH occasion(s) associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see subclause 7.4);

-	ra-OccasionList: defines PRACH occasion(s) associated with a CSI-RS in which the MAC entity may transmit a Random Access Preamble;

-	ra-PreambleStartIndex: the starting index of Random Access Preamble(s) for on-demand SI request;

-	preambleTransMax: the maximum number of Random Access Preamble transmission;

-	ssb-perRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion and the number of contention-based Random Access Preambles mapped to each SSB;

-	if groupBconfigured is configured, then Random Access Preambles group B is configured.

-	Amongst the contention-based Random Access Preambles associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

NOTE 2:	If Random Access Preambles group B is supported by the cell Random Access Preambles group B is included for each SSB.

-	if Random Access Preambles group B is configured:

-	ra-Msg3SizeGroupA: the threshold to determine the groups of Random Access Preambles;

-	msg3-DeltaPreamble: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for reconfiguration with sync, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B is configured:

-	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink, and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_POWER_RAMPING_STEP;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	SCALING_FACTOR_BI;

-	TEMPORARY_C-RNTI.

When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	if the carrier to use for the Random Access procedure is explicitly signalled:

2>	select the signalled carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:

2>	select the NUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

1>	perform the BWP operation as specified in subclause 5.15;

1>	set PREAMBLE_POWER_RAMPING_STEP to powerRampingStep;

1>	set SCALING_FACTOR_BI to 1;

1>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.17); and

1>	if beamFailureRecoveryConfig is configured for the active UL BWP of the selected carrier:

2>	start the beamFailureRecoveryTimer, if configured;

2>	apply the parameters powerRampingStep, preambleReceivedTargetPower, and preambleTransMax configured in the beamFailureRecoveryConfig;

2>	if powerRampingStepHighPriority is configured in the beamFailureRecoveryConfig:

3>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

2>	else:

3>	set PREAMBLE_POWER_RAMPING_STEP to powerRampingStep.

2>	if scalingFactorBI is configured in the beamFailureRecoveryConfig:

3>	set SCALING_FACTOR_BI to the scalingFactorBI.

1>	else if the Random Access procedure was initiated for handover; and

1>	if rach-ConfigDedicated is configured for the selected carrier:

2>	if powerRampingStepHighPriority is configured in the rach-ConfigDedicated:

3>	set PREAMBLE_POWER_RAMPING_STEP to the powerRampingStepHighPriority.

2>	if scalingFactorBI is configured in the rach-ConfigDedicated:

3>	set SCALING_FACTOR_BI to the scalingFactorBI.

1>	perform the Random Access Resource selection procedure (see subclause 5.1.2).

Next change

5.1.3	Random Access Preamble transmission

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if SSB or CSI-RS selected is not changed from the selection in the last Random Access Preamble transmission:

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to subclause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP;

1>	except for contention-free Random Access Preamble for beam failure recovery request, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the Random Access Preamble using the selected PRACH occasion, corresponding RA-RNTI (if available), PREAMBLE_INDEX and PREAMBLE_RECEIVED_TARGET_POWER.

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

Next change

5.3.2.1	HARQ Entity

The MAC entity includes a HARQ entity for each Serving Cell, which maintains a number of parallel HARQ processes. Each HARQ process is associated with a HARQ process identifier. The HARQ entity directs HARQ information and associated TBs received on the DL-SCH to the corresponding HARQ processes (see subclause 5.3.2.2).

The number of parallel DL HARQ processes per HARQ entity is specified in TS 38.214 [7]. The dedicated broadcast HARQ process is used for BCCH.

The HARQ process supports one TB when the physical layer is not configured for downlink spatial multiplexing. The HARQ process supports one or two TBs when the physical layer is configured for downlink spatial multiplexing.

When the MAC entity is configured with pdsch-AggregationFactor > 1, the parameter pdsch-AggregationFactor provides the number of transmissions of a TB within a bundle of the downlink assignment. Bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. After the initial transmission, pdsch-AggregationFactor – 1 HARQ retransmissions follow within a bundle.

The MAC entity shall:

1>	if a downlink assignment has been indicated:

2>	allocate the TB(s) received from the physical layer and the associated HARQ information to the HARQ process indicated by the associated HARQ information.

1>	if a downlink assignment has been indicated for the broadcast HARQ process:

2>	allocate the received TB to the broadcast HARQ process.

Next change

5.4.1	UL Grant reception

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, or configured semi-persistently by RRC. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the correponding HARQ process, if configured.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in subclause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if the configuredGrantTimer for the corresponding HARQ process is not running:

3>	consider the NDI bit for the corresponding HARQ process to have been toggled;

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

NOTE 1:	CURRENT_symbol refers to the symbol index of the first transmission occasion of a repetition bundle that takes place.

NOTE 2:	A HARQ process is configured for a configured uplink grant if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes.

NOTE 3:	If the MAC entity receives both a grant in a Random Access Response and an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI or the grant for its C-RNTI or CS-RNTI.

Next change

5.4.2.1	HARQ Entity

The MAC entity includes a HARQ entity for each Serving Cell with configured uplink (including the case when it is configured with supplementaryUplink), which maintains a number of parallel HARQ processes.

The number of parallel UL HARQ processes per HARQ entity is specified in TS 38.214 [7].

Each HARQ process supports one TB.

Each HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RA Response, HARQ process identifier 0 is used.

When the MAC entity is configured with pusch-AggregationFactor > 1, the parameter pusch-AggregationFactor provides the number of transmissions of a TB within a bundle of the dynamic grant. After the initial transmission, pusch-AggregationFactor – 1 HARQ retransmissions follow within a bundle. When the MAC entity is configured with repK > 1, the parameter repK provides the number of transmissions of a TB within a bundle of the configured uplink grant. After the initial transmission, HARQ retransmissions follow within a bundle. For both dynamic grant and configured uplink grant, bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle, HARQ retransmissions are triggered without waiting for feedback from previous transmission according to pusch-AggregationFactor for a dynamic grant and repK for a configured uplink grant, respectively. Each transmission within a bundle is a separate uplink grant after the initial uplink grant within a bundle is delivered to the HARQ entity.

For each transmission within a bundle of the dynamic grant, the sequence of redundancy versions is determined according to subclause 6.1.2.1 of TS 38.214 [7]. For each transmission within a bundle of the configured uplink grant, the sequence of redundancy versions is determined according to subclause 6.1.2.3 of TS 38.214 [7].

For each uplink grant, the HARQ entity shall:

1>	identify the HARQ process associated with this grant, and for each identified HARQ process:

2>	if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or

2>	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

2>	if the uplink grant was received in a Random Access Response; or

2>	if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or

2>	if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to subclause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle:

3>	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response; or:

3>	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery:

4>	obtain the MAC PDU to transmit from the Msg3 buffer.

4>	if the uplink grant size does not match with size of the obtained MAC PDU; and

4>	if the Random Access procedure was successfully completed upon receiving the uplink grant:

5>	indicate to the Multiplexing and assembly entity to include MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in the subsequent uplink transmission;

5>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity.

3>	else:

4>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a new transmission;

4>	if the uplink grant is addressed to CS-RNTI; or

4>	if the uplink grant is a configured uplink grant; or

4>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

5>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

3>	else:

4>	flush the HARQ buffer of the identified HARQ process.

2>	else (i.e. retransmission):

3>	if the uplink grant received on PDCCH was addressed to CS-RNTI and if the HARQ buffer of the identified process is empty; or

3>	if the uplink grant is part of a bundle and if no MAC PDU has been obtained for this bundle; or

3>	if the uplink grant is part of a bundle of the configured uplink grant, and the PUSCH of the uplink grant overlaps with a PUSCH of another uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell:

4>	ignore the uplink grant.

3>	else:

4>	deliver the uplink grant and the HARQ information (redundancy version) of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a retransmission;

4>	if the uplink grant is addressed to CS-RNTI; or

4>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

5>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

When determining if NDI has been toggled compared to the value in the previous transmission the MAC entity shall ignore NDI received in all uplink grants on PDCCH for its Temporary C-RNTI.

Next change

5.4.3.1.3	Allocation of resources

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in subclause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in subclause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes while having data available and allowed (according to subclause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied:

-	the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant; and

-	there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

-	the MAC PDU includes zero MAC SDUs; and

-	the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	C-RNTI MAC CE or data from UL-CCCH;

-	Configured Grant Confirmation MAC CE;

-	MAC CE for BSR, with exception of BSR included for padding;

-	Single Entry PHR MAC CE or Multiple Entry PHR MAC CE;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding.

Next change

5.4.5	Buffer Status Reporting

The Buffer Status reporting (BSR) procedure is used to provide the serving gNB with information about UL data volume in the MAC entity.

RRC configures the following parameters to control the BSR:

-	periodicBSR-Timer;

-	retxBSR-Timer;

-	logicalChannelSR-DelayTimerApplied;

-	logicalChannelSR-DelayTimer;

-	logicalChannelSR-Mask;

-	logicalChannelGroup.

Each logical channel may be allocated to an LCG using the logicalChannelGroup. The maximum number of LCGs is eight.

The MAC entity determines the amount of UL data available for a logical channel according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4].

A BSR shall be triggered if any of the following events occur:

-	UL data, for a logical channel which belongs to an LCG, becomes available to the MAC entity; and either

-	this UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or

-	none of the logical channels which belong to an LCG contains any available UL data.

in which case the BSR is referred below to as 'Regular BSR';

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR';

-	retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR';

-	periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'.

NOTE:	When Regular BSR triggering events occur for multiple logical channels simultaneously, each logical channel triggers one separate Regular BSR.

For Regular BSR, the MAC entity shall:

1>	if the BSR is triggered for a logical channel for which logicalChannelSR-DelayTimerApplied with value true is configured by upper layers:

2>	start or restart the logicalChannelSR-DelayTimer.

1>	else:

2>	if running, stop the logicalChannelSR-DelayTimer.

For Regular and Periodic BSR, the MAC entity shall:

1>	if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built:

2>	report Long BSR for all LCGs which have data available for transmission.

1>	else:

2>	report Short BSR.

For Padding BSR:

1>	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

2>	if more than one LCG has data available for transmission when the BSR is to be built:

3>	if the number of padding bits is equal to the size of the Short BSR plus its subheader:

4>	report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission.

3>	else:

4>	report Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of the highest priority logical channel (with or without data available for transmission) in each of these LCG(s), and in case of equal priority, in increasing order of LCGID.

2>	else:

3>	report Short BSR.

1>	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader:

2>	report Long BSR for all LCGs which have data available for transmission.

For BSR triggered by retxBSR-Timer expiry, the MAC entity considers that the logical channel that triggered the BSR is the highest priority logical channel that has data available for transmission at the time the BSR is triggered.

The MAC entity shall:

1>	if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the BSR MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the BSR MAC CE(s);

3>	start or restart periodicBSR-Timer except when all the generated BSRs are long or short Truncated BSRs;

3>	start or restart retxBSR-Timer.

2>	if a Regular BSR has been triggered and logicalChannelSR-DelayTimer is not running:

3>	if there is no UL-SCH resource available for a new transmission; or

3>	if the MAC entity is configured with configured uplink grant(s) and the Regular BSR was triggered for a logical channel for which logicalChannelSR-Mask is set to false; or

3>	if the UL-SCH resources available for a new transmission do not meet the LCP mapping restrictions (see subclause 5.4.3.1) configured for the logical channel that triggered the BSR:

4>	trigger a Scheduling Request.

NOTE:	UL-SCH resources are considered available if the MAC entity has an active configuration for either type of configured uplink grants, or if the MAC entity has received a dynamic uplink grant, or if both of these conditions are met. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time.

A MAC PDU shall contain at most one BSR MAC CE, even when multiple events have triggered a BSR. The Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

The MAC entity shall restart retxBSR-Timer upon reception of a grant for transmission of new data on any UL-SCH.

All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

NOTE:	MAC PDU assembly can happen at any point in time between uplink grant reception and actual transmission of the corresponding MAC PDU. BSR and SR can be triggered after the assembly of a MAC PDU which contains a BSR MAC CE, but before the transmission of this MAC PDU. In addition, BSR and SR can be triggered during MAC PDU assembly.

5.4.6	Power Headroom Reporting

The Power Headroom reporting procedure is used to provide the serving gNB with the following information:

-	Type 1 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell;

-	Type 2 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH and PUCCH transmission on SpCell of the other MAC entity (i.e. E-UTRA MAC entity in EN-DC case only);

-	Type 3 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for SRS transmission per activated Serving Cell.

RRC controls Power Headroom reporting by configuring the following parameters:

-	phr-PeriodicTimer;

-	phr-ProhibitTimer;

-	phr-Tx-PowerFactorChange;

-	phr-Type2OtherCell;

-	phr-ModeOtherCG;

-	multiplePHR.

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one activated Serving Cell of any MAC entity which is used as a pathloss reference since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

NOTE 1:	The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between.

-	phr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink;

-	addition of the PSCell (i.e. PSCell is newly added or changed);

-	phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

NOTE 2:	The MAC entity should avoid triggering a PHR when the required power backoff due to power management decreases only temporarily (e.g. for up to a few tens of milliseconds) and it should avoid reflecting such temporary decrease in the values of PCMAX,f,c/PH when a PHR is triggered by other triggering conditions.

If the MAC entity has UL resources allocated for a new transmission the MAC entity shall:

1>	if it is the first UL resource allocated for a new transmission since the last MAC reset:

2>	start phr-PeriodicTimer;

1>	if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled; and

1>	if the allocated UL resources can accommodate the MAC CE for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in subclause 5.4.3.1:

2>	if multiplePHR with value true is configured:

3>	for each activated Serving Cell with configured uplink associated with any MAC entity:

4>	obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in subclause 7.7 of TS 38.213 [6];

4>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

4>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

5>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

3>	if phr-Type2OtherCell with value true is configured:

4>	if the other MAC entity is E-UTRA MAC entity:

5>	obtain the value of the Type 2 power headroom for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity);

5>	if phr-ModeOtherCG is set to real by upper layers:

6>	obtain the value for the corresponding PCMAX,f,c field for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity) from the physical layer.

3>	instruct the Multiplexing and Assembly procedure to generate and transmit the Multiple Entry PHR MAC CE as defined in subclause 6.1.3.9 based on the values reported by the physical layer.

2>	else (i.e. Single Entry PHR format is used):

3>	obtain the value of the Type 1 power headroom from the physical layer for the corresponding uplink carrier of the PCell;

3>	obtain the value for the corresponding PCMAX,f,c field from the physical layer;

3>	instruct the Multiplexing and Assembly procedure to generate and transmit the Single Entry PHR MAC CE as defined in subclause 6.1.3.8 based on the values reported by the physical layer.

2>	start or restart phr-PeriodicTimer;

2>	start or restart phr-ProhibitTimer;

2>	cancel all triggered PHR(s).

NOTE 3:	For a band combination in which the UE does not support dynamic power sharing, the UE may omit reporting power headroom information for Serving Cells in the other MAC entity.

Next change

5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer or in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception.

1>	in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

2>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7].

1>	if CSI masking (csi-Mask) is setup by upper layers:

2>	in current symbol n, if onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

3>	not report CSI on PUCCH.

1>	else:

2>	in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

3>	not report CSI on PUCCH and semi-persistent CSI on PUSCH.

Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

Next change

5.8.2	Uplink

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC per Serving Cell and per BWP. Multiple configurations can be active simultaneously only on different Serving Cells. For Type 2, activation and deactivation are independent among the Serving Cells. For the same Serving Cell, the MAC entity is configured with either Type 1 or Type 2.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = 0 in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant.

Upon configuration of a configured grant Type 1 for a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset and S (derived from SLIV as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
 (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

When a configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if the configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	instruct the Multiplexing and Assembly procedure to generate an Configured Grant Confirmation MAC CE as defined in subclause 6.1.3.7;

2>	cancel the triggered configured uplink grant confirmation.

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant immediately after first transmission of Configured Grant Confirmation MAC CE triggered by the configured uplink grant deactivation.

Retransmissions except for repetition of configured uplink grants use uplink grants addressed to CS-RNTI.

Next change

5.18.1	General

This subclause specifies the requirements upon reception of the following MAC CEs:

-	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE;

-	Aperiodic CSI Trigger State Subselection MAC CE;

-	TCI States Activation/Deactivation for UE-specific PDSCH MAC CE;

-	TCI State Indication for UE-specific PDCCH MAC CE;

-	SP CSI reporting on PUCCH Activation/Deactivation MAC CE;

-	SP SRS Activation/Deactivation MAC CE;

-	PUCCH spatial relation Activation/Deactivation MAC CE;

-	SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE;

-	Recommended Bit Rate MAC CE.

Next change

5.18.2	Activation/Deactivation of Semi-persistent CSI-RS/CSI-IM resource set

The network may activate and deactivate the configured Semi-persistent CSI-RS/CSI-IM resource sets of a Serving Cell by sending the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE described in subclause 6.1.3.12. The configured Semi-persistent CSI-RS/CSI-IM resource sets are initially deactivated upon configuration and after a handover.

The MAC entity shall:

1>	if the MAC entity receives an SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE.

Next change

6.1.3.1	Buffer Status Report MAC CEs

Buffer Status Report (BSR) MAC CEs consist of either:

-	Short BSR format (fixed size); or

-	Long BSR format (variable size); or

-	Short Truncated BSR format (fixed size); or

-	Long Truncated BSR format (variable size).

The BSR formats are identified by MAC subheaders with LCIDs as specified in Table 6.2.1-2.

The fields in the BSR MAC CE are defined as follows:

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) whose buffer status is being reported. The length of the field is 3 bits;

-	LCGi: For the Long BSR format, this field indicates the presence of the Buffer Size field for the logical channel group i. The LCGi field set to 1 indicates that the Buffer Size field for the logical channel group i is reported. The LCGi field set to 0 indicates that the Buffer Size field for the logical channel group i is not reported. For the Long Truncated BSR format, this field indicates whether logical channel group i has data available. The LCGi field set to 1 indicates that logical channel group i has data available. The LCGi field set to 0 indicates that logical channel group i does not have data available;

-	Buffer Size: The Buffer Size field identifies the total amount of data available according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4] across all logical channels of a logical channel group after the MAC PDU has been built (i.e. after the logical channel prioritization procedure, which may result the value of the Buffer Size field to zero). The amount of data is indicated in number of bytes. The size of the RLC and MAC headers are not considered in the buffer size computation. The length of this field for the Short BSR format and the Short Truncated BSR format is 5 bits. The length of this field for the Long BSR format and the Long Truncated BSR format is 8 bits. The values for the 5-bit and 8-bit Buffer Size fields are shown in Tables 6.1.3.1-1 and 6.1.3.1-2, respectively. For the Long BSR format and the Long Truncated BSR format, the Buffer Size fields are included in ascending order based on the LCGi. For the Long Truncated BSR format the number of Buffer Size fields included is maximised, while not exceeding the number of padding bits.

NOTE:	The number of the Buffer Size fields in the Long BSR and Long Truncated BSR format can be zero.

Figure 6.1.3.1-1: Short BSR and Short Truncated BSR MAC CE

Figure 6.1.3.1-2: Long BSR and Long Truncated BSR MAC CE

Table 6.1.3.1-1: Buffer size levels (in bytes) for 5-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 8 | ≤ 102 | 16 | ≤ 1446 | 24 | ≤ 20516 |
| 1 | ≤ 10 | 9 | ≤ 142 | 17 | ≤ 2014 | 25 | ≤ 28581 |
| 2 | ≤ 14 | 10 | ≤ 198 | 18 | ≤ 2806 | 26 | ≤ 39818 |
| 3 | ≤ 20 | 11 | ≤ 276 | 19 | ≤ 3909 | 27 | ≤ 55474 |
| 4 | ≤ 28 | 12 | ≤ 384 | 20 | ≤ 5446 | 28 | ≤ 77284 |
| 5 | ≤ 38 | 13 | ≤ 535 | 21 | ≤ 7587 | 29 | ≤ 107669 |
| 6 | ≤ 53 | 14 | ≤ 745 | 22 | ≤ 10570 | 30 | ≤ 150000 |
| 7 | ≤ 74 | 15 | ≤ 1038 | 23 | ≤ 14726 | 31 | > 150000 |

Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 560 | 128 | ≤ 31342 | 192 | ≤ 1754595 |
| 1 | ≤ 10 | 65 | ≤ 597 | 129 | ≤ 33376 | 193 | ≤ 1868488 |
| 2 | ≤ 11 | 66 | ≤ 635 | 130 | ≤ 35543 | 194 | ≤ 1989774 |
| 3 | ≤ 12 | 67 | ≤ 677 | 131 | ≤ 37850 | 195 | ≤ 2118933 |
| 4 | ≤ 13 | 68 | ≤ 720 | 132 | ≤ 40307 | 196 | ≤ 2256475 |
| 5 | ≤ 14 | 69 | ≤ 767 | 133 | ≤ 42923 | 197 | ≤ 2402946 |
| 6 | ≤ 15 | 70 | ≤ 817 | 134 | ≤ 45709 | 198 | ≤ 2558924 |
| 7 | ≤ 16 | 71 | ≤ 870 | 135 | ≤ 48676 | 199 | ≤ 2725027 |
| 8 | ≤ 17 | 72 | ≤ 926 | 136 | ≤ 51836 | 200 | ≤ 2901912 |
| 9 | ≤ 18 | 73 | ≤ 987 | 137 | ≤ 55200 | 201 | ≤ 3090279 |
| 10 | ≤ 19 | 74 | ≤ 1051 | 138 | ≤ 58784 | 202 | ≤ 3290873 |
| 11 | ≤ 20 | 75 | ≤ 1119 | 139 | ≤ 62599 | 203 | ≤ 3504487 |
| 12 | ≤ 22 | 76 | ≤ 1191 | 140 | ≤ 66663 | 204 | ≤ 3731968 |
| 13 | ≤ 23 | 77 | ≤ 1269 | 141 | ≤ 70990 | 205 | ≤ 3974215 |
| 14 | ≤ 25 | 78 | ≤ 1351 | 142 | ≤ 75598 | 206 | ≤ 4232186 |
| 15 | ≤ 26 | 79 | ≤ 1439 | 143 | ≤ 80505 | 207 | ≤ 4506902 |
| 16 | ≤ 28 | 80 | ≤ 1532 | 144 | ≤ 85730 | 208 | ≤ 4799451 |
| 17 | ≤ 30 | 81 | ≤ 1631 | 145 | ≤ 91295 | 209 | ≤ 5110989 |
| 18 | ≤ 32 | 82 | ≤ 1737 | 146 | ≤ 97221 | 210 | ≤ 5442750 |
| 19 | ≤ 34 | 83 | ≤ 1850 | 147 | ≤ 103532 | 211 | ≤ 5796046 |
| 20 | ≤ 36 | 84 | ≤ 1970 | 148 | ≤ 110252 | 212 | ≤ 6172275 |
| 21 | ≤ 38 | 85 | ≤ 2098 | 149 | ≤ 117409 | 213 | ≤ 6572925 |
| 22 | ≤ 40 | 86 | ≤ 2234 | 150 | ≤ 125030 | 214 | ≤ 6999582 |
| 23 | ≤ 43 | 87 | ≤ 2379 | 151 | ≤ 133146 | 215 | ≤ 7453933 |
| 24 | ≤ 46 | 88 | ≤ 2533 | 152 | ≤ 141789 | 216 | ≤ 7937777 |
| 25 | ≤ 49 | 89 | ≤ 2698 | 153 | ≤ 150992 | 217 | ≤ 8453028 |
| 26 | ≤ 52 | 90 | ≤ 2873 | 154 | ≤ 160793 | 218 | ≤ 9001725 |
| 27 | ≤ 55 | 91 | ≤ 3059 | 155 | ≤ 171231 | 219 | ≤ 9586039 |
| 28 | ≤ 59 | 92 | ≤ 3258 | 156 | ≤ 182345 | 220 | ≤ 10208280 |
| 29 | ≤ 62 | 93 | ≤ 3469 | 157 | ≤ 194182 | 221 | ≤ 10870913 |
| 30 | ≤ 66 | 94 | ≤ 3694 | 158 | ≤ 206786 | 222 | ≤ 11576557 |
| 31 | ≤ 71 | 95 | ≤ 3934 | 159 | ≤ 220209 | 223 | ≤ 12328006 |
| 32 | ≤ 75 | 96 | ≤ 4189 | 160 | ≤ 234503 | 224 | ≤ 13128233 |
| 33 | ≤ 80 | 97 | ≤ 4461 | 161 | ≤ 249725 | 225 | ≤ 13980403 |
| 34 | ≤ 85 | 98 | ≤ 4751 | 162 | ≤ 265935 | 226 | ≤ 14887889 |
| 35 | ≤ 91 | 99 | ≤ 5059 | 163 | ≤ 283197 | 227 | ≤ 15854280 |
| 36 | ≤ 97 | 100 | ≤ 5387 | 164 | ≤ 301579 | 228 | ≤ 16883401 |
| 37 | ≤ 103 | 101 | ≤ 5737 | 165 | ≤ 321155 | 229 | ≤ 17979324 |
| 38 | ≤ 110 | 102 | ≤ 6109 | 166 | ≤ 342002 | 230 | ≤ 19146385 |
| 39 | ≤ 117 | 103 | ≤ 6506 | 167 | ≤ 364202 | 231 | ≤ 20389201 |
| 40 | ≤ 124 | 104 | ≤ 6928 | 168 | ≤ 387842 | 232 | ≤ 21712690 |
| 41 | ≤ 132 | 105 | ≤ 7378 | 169 | ≤ 413018 | 233 | ≤ 23122088 |
| 42 | ≤ 141 | 106 | ≤ 7857 | 170 | ≤ 439827 | 234 | ≤ 24622972 |
| 43 | ≤ 150 | 107 | ≤ 8367 | 171 | ≤ 468377 | 235 | ≤ 26221280 |
| 44 | ≤ 160 | 108 | ≤ 8910 | 172 | ≤ 498780 | 236 | ≤ 27923336 |
| 45 | ≤ 170 | 109 | ≤ 9488 | 173 | ≤ 531156 | 237 | ≤ 29735875 |
| 46 | ≤ 181 | 110 | ≤ 10104 | 174 | ≤ 565634 | 238 | ≤ 31666069 |
| 47 | ≤ 193 | 111 | ≤ 10760 | 175 | ≤ 602350 | 239 | ≤ 33721553 |
| 48 | ≤ 205 | 112 | ≤ 11458 | 176 | ≤ 641449 | 240 | ≤ 35910462 |
| 49 | ≤ 218 | 113 | ≤ 12202 | 177 | ≤ 683087 | 241 | ≤ 38241455 |
| 50 | ≤ 233 | 114 | ≤ 12994 | 178 | ≤ 727427 | 242 | ≤ 40723756 |
| 51 | ≤ 248 | 115 | ≤ 13838 | 179 | ≤ 774645 | 243 | ≤ 43367187 |
| 52 | ≤ 264 | 116 | ≤ 14736 | 180 | ≤ 824928 | 244 | ≤ 46182206 |
| 53 | ≤ 281 | 117 | ≤ 15692 | 181 | ≤ 878475 | 245 | ≤ 49179951 |
| 54 | ≤ 299 | 118 | ≤ 16711 | 182 | ≤ 935498 | 246 | ≤ 52372284 |
| 55 | ≤ 318 | 119 | ≤ 17795 | 183 | ≤ 996222 | 247 | ≤ 55771835 |
| 56 | ≤ 339 | 120 | ≤ 18951 | 184 | ≤ 1060888 | 248 | ≤ 59392055 |
| 57 | ≤ 361 | 121 | ≤ 20181 | 185 | ≤ 1129752 | 249 | ≤ 63247269 |
| 58 | ≤ 384 | 122 | ≤ 21491 | 186 | ≤ 1203085 | 250 | ≤ 67352729 |
| 59 | ≤ 409 | 123 | ≤ 22885 | 187 | ≤ 1281179 | 251 | ≤ 71724679 |
| 60 | ≤ 436 | 124 | ≤ 24371 | 188 | ≤ 1364342 | 252 | ≤ 76380419 |
| 61 | ≤ 464 | 125 | ≤ 25953 | 189 | ≤ 1452903 | 253 | ≤ 81338368 |
| 62 | ≤ 494 | 126 | ≤ 27638 | 190 | ≤ 1547213 | 254 | > 81338368 |
| 63 | ≤ 526 | 127 | ≤ 29431 | 191 | ≤ 1647644 | 255 | Reserved |

6.1.3.2	C-RNTI MAC CE

The C-RNTI MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size and consists of a single field defined as follows (Figure 6.1.3.2-1):

-	C-RNTI: This field contains the C-RNTI of the MAC entity. The length of the field is 16 bits.

Figure 6.1.3.2-1: C-RNTI MAC CE

6.1.3.3	UE Contention Resolution Identity MAC CE

The UE Contention Resolution Identity MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1.

It has a fixed 48-bit size and consists of a single field defined as follows (Figure 6.1.3.3-1):

-	UE Contention Resolution Identity: This field contains the UL CCCH SDU. If the UL CCCH SDU is longer than 48 bits, this field contains the first 48 bits of the UL CCCH SDU.

Figure 6.1.3.3-1: UE Contention Resolution Identity MAC CE

6.1.3.4	Timing Advance Command MAC CE

The Timing Advance Command MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1.

It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.4-1):

-	TAG Identity (TAG ID): This field indicates the TAG Identity of the addressed TAG. The TAG containing the SpCell has the TAG Identity 0. The length of the field is 2 bits;

-	Timing Advance Command: This field indicates the index value TA (0, 1, 2… 63) used to control the amount of timing adjustment that MAC entity has to apply (as specified in TS 38.213 [6]). The length of the field is 6 bits.

Figure 6.1.3.4-1: Timing Advance Command MAC CE

6.1.3.5	DRX Command MAC CE

The DRX Command MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1.

It has a fixed size of zero bits.

6.1.3.6	Long DRX Command MAC CE

The Long DRX Command MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1.

It has a fixed size of zero bits.

6.1.3.7	Configured Grant Confirmation MAC CE

The Configured Grant Confirmation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size of zero bits.

6.1.3.8	Single Entry PHR MAC CE

The Single Entry PHR MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size and consists of two octets defined as follows (figure 6.1.3.8-1):

-	R: Reserved bit, set to 0;

-	Power Headroom (PH): This field indicates the power headroom level. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.8-1 below (the corresponding measured values in dB are specified in TS 38.133 [11]);

-	PCMAX,f,c: This field indicates the PCMAX,f,c (as specified in TS 38.213 [6]) used for calculation of the preceding PH field. The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in Table 6.1.3.8-2 (the corresponding measured values in dBm are specified in TS 38.133 [11]).

Figure 6.1.3.8-1: Single Entry PHR MAC CE

Table 6.1.3.8-1: Power Headroom levels for PHR

| PH | Power Headroom Level |
| --- | --- |
| 0 | POWER_HEADROOM_0 |
| 1 | POWER_HEADROOM_1 |
| 2 | POWER_HEADROOM_2 |
| 3 | POWER_HEADROOM_3 |
| … | … |
| 60 | POWER_HEADROOM_60 |
| 61 | POWER_HEADROOM_61 |
| 62 | POWER_HEADROOM_62 |
| 63 | POWER_HEADROOM_63 |

Table 6.1.3.8-2: Nominal UE transmit power level for PHR

| PCMAX,f,c | Nominal UE transmit power level |
| --- | --- |
| 0 | PCMAX_C_00 |
| 1 | PCMAX_C_01 |
| 2 | PCMAX_C_02 |
| … | … |
| 61 | PCMAX_C_61 |
| 62 | PCMAX_C_62 |
| 63 | PCMAX_C_63 |

6.1.3.9	Multiple Entry PHR MAC CE

The Multiple Entry PHR MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-2.

It has a variable size, and includes the bitmap, a Type 2 PH field and an octet containing the associated PCMAX,f,c field (if reported) for SpCell of the other MAC entity, a Type 1 PH field and an octet containing the associated PCMAX,f,c field (if reported) for the PCell. It further includes, in ascending order based on the ServCellIndex, one or multiple of Type X PH fields and octets containing the associated PCMAX,f,c fields (if reported) for Serving Cells other than PCell indicated in the bitmap. X is either 1 or 3 according to TS 38.213 [6] and TS 36.213 [17].

The presence of Type 2 PH field for SpCell of the other MAC entity is configured by phr-Type2OtherCell with value true.

A single octet bitmap is used for indicating the presence of PH per Serving Cell when the highest ServCellIndex of Serving Cell with configured uplink is less than 8, otherwise four octets are used.

The MAC entity determines whether PH value for an activated Serving Cell is based on real transmission or a reference format by considering the configured grant(s) and downlink control information which has been received until and including the PDCCH occasion in which the first UL grant for a new transmission is received since a PHR has been triggered if the PHR MAC CE is reported on an uplink grant received on the PDCCH or until the first uplink symbol of PUSCH transmission minus PUSCH preparation time as defined in subclause 6.4 of TS 38.214 [7] if the PHR MAC CE is reported on a configured grant.

The PHR MAC CEs are defined as follows:

-	Ci: This field indicates the presence of a PH field for the Serving Cell with ServCellIndex i as specified in TS 38.331 [5]. The Ci field set to 1 indicates that a PH field for the Serving Cell with ServCellIndex i is reported. The Ci field set to 0 indicates that a PH field for the Serving Cell with ServCellIndex i is not reported;

-	R: Reserved bit, set to 0;

-	V: This field indicates if the PH value is based on a real transmission or a reference format. For Type 1 PH, the V field set to 0 indicates real transmission on PUSCH and the V field set to 1 indicates that a PUSCH reference format is used. For Type 2 PH, the V field set to 0 indicates real transmission on PUCCH and the V field set to 1 indicates that a PUCCH reference format is used. For Type 3 PH, the V field set to 0 indicates real transmission on SRS and the V field set to 1 indicates that an SRS reference format is used. Furthermore, for Type 1, Type 2, and Type 3 PH, the V field set to 0 indicates the presence of the octet containing the associated PCMAX,f,c field, and the V field set to 1 indicates that the octet containing the associated PCMAX,f,c field is omitted;

-	Power Headroom (PH): This field indicates the power headroom level. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.8-1 (the corresponding measured values in dB for the NR Serving Cell are specified in TS 38.133 [11] while the corresponding measured values in dB for the E-UTRA Serving Cell are specified in TS 36.133 [12]);

-	P: This field indicates whether the MAC entity applies power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]). The MAC entity shall set the P field to 1 if the corresponding PCMAX,f,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,f,c: If present, this field indicates the PCMAX,f,c (as specified in TS 38.213 [6]) for the NR Serving Cell and the PCMAX,c or P̃CMAX,c (as specified in TS 36.213 [17]) for the E-UTRA Serving Cell used for calculation of the preceding PH field. The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in Table 6.1.3.8-2 (the corresponding measured values in dBm for the NR Serving Cell are specified in TS 38.133 [11] while the corresponding measured values in dBm for the E-UTRA Serving Cell are specified in TS 36.133 [12]).

Figure 6.1.3.9-1: Multiple Entry PHR MAC CE with the highest ServCellIndex of Serving Cell with configured uplink is less than 8

6.1.3.10	SCell Activation/Deactivation MAC CEs

The SCell Activation/Deactivation MAC CE of one octet is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing seven C-fields and one R-field. The SCell Activation/Deactivation MAC CE with one octet is defined as follows (Figure 6.1.3.10-1).

The SCell Activation/Deactivation MAC CE of four octets is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of four octets containing 31 C-fields and one R-field. The SCell Activation/Deactivation MAC CE of four octets is defined as follows (Figure 6.1.3.10-2).

-	Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [5], this field indicates the activation/deactivation status of the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to 1 to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that the SCell with SCellIndex i shall be deactivated;

-	R: Reserved bit, set to 0.

Figure 6.1.3.10-1: SCell Activation/Deactivation MAC CE of one octet

Figure 6.1.3.10-2: SCell Activation/Deactivation MAC CE of four octets

6.1.3.11	Duplication Activation/Deactivation MAC CE

The Duplication Activation/Deactivation MAC CE of one octet is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing eight D-fields. The Duplication Activation/Deactivation MAC CE is defined, for a MAC entity, as follows (Figure 6.1.3.11-1).

-	Di: This field indicates the activation/deactivation status of the PDCP duplication of DRB i where i is the ascending order of the DRB ID among the DRBs configured with PDCP duplication and with RLC entity(ies) associated with this MAC entity. The Di field is set to 1 to indicate that the PDCP duplication of DRB i shall be activated. The Di field is set to 0 to indicate that the PDCP duplication of DRB i shall be deactivated.

Figure 6.1.3.11-1: Duplication Activation/Deactivation MAC CE

6.1.3.12	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

The SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a variable size and consists of the following fields:

-	A/D: This field indicates whether to activate or deactivate indicated SP CSI-RS and CSI-IM resource set(s). The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	SP CSI-RS resource set ID: This field contains an index of NZP-CSI-RS-ResourceSet containing Semi Persistent NZP CSI-RS resources, as specified in TS 38.331 [5], indicating the Semi Persistent NZP CSI-RS resource set, which shall be activated or deactivated. The length of the field is 6 bits;

-	IM: This field indicates the presence of the octet containing SP CSI-IM resource set ID field. If the IM field is set to 1, the octet containing SP CSI-IM resource set ID field is present. If IM field is set to 0, the octet containing SP CSI-IM resource set ID field is not present;

-	SP CSI-IM resource set ID: This field contains an index of CSI-IM-ResourceSet containing Semi Persistent CSI-IM resources, as specified in TS 38.331 [5], indicating the Semi Persistent CSI-IM resource set, which shall be activated or deactivated. The length of the field is 6 bits;

-	TCI State IDi: This field contains TCI-StateId, as specified in TS 38.331 [5], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set indicated by SP CSI-RS resource set ID field. TCI State ID0 indicates TCI State for the first resource within the set, TCI State ID1 for the second one and so on. The length of the field is 7 bits. If the A/D field is set to 0, the octets containing TCI State ID field(s) are not present;

-	R: Reserved bit, set to 0.

Figure 6.1.3.12-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

6.1.3.13	Aperiodic CSI Trigger State Subselection MAC CE

The Aperiodic CSI Trigger State Subselection MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a variable size consisting of following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Ti: This field indicates the selection status of the Aperiodic Trigger States configured within CSI-aperiodicTriggerStateList, as specified in TS 38.331 [5]. T0 refers to the first trigger state within the list, T1 to the second one and so on. If the list does not contain entry with index i, MAC entity shall ignore the Ti field. The Ti field is set to 1 to indicate that the Aperiodic Trigger State i shall be mapped to the codepoint of the DCI CSI request field, as specified in TS 38.214 [7]. The codepoint to which the Aperiodic Trigger State is mapped is determined by its ordinal position among all the Aperiodic Trigger States with Ti field set to 1, i.e. the first Aperiodic Trigger State with Ti field set to 1 shall be mapped to the codepoint value 1, second Aperiodic Trigger State with Ti field set to 1 shall be mapped to the codepoint value 2 and so on. The maximum number of mapped Aperiodic Trigger States is 63;

-	R: Reserved bit, set to 0.

Figure 6.1.3.13-1: Aperiodic CSI Trigger State Subselection MAC CE

6.1.3.14	TCI States Activation/Deactivation for UE-specific PDSCH MAC CE

The TCI States Activation/Deactivation for UE-specific PDSCH MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a variable size consisting of following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Ti: If there is a TCI state with TCI-StateId i as specified in TS 38.331 [5], this field indicates the activation/deactivation status of the TCI state with TCI-StateId i, otherwise MAC entity shall ignore the Ti field. The Ti field is set to 1 to indicate that the TCI state with TCI-StateId i shall be activated and mapped to the codepoint of the DCI Transmission Configuration Indication field, as specified in TS 38.214 [7]. The Ti field is set to 0 to indicate that the TCI state with TCI-StateId i shall be deactivated and is not mapped to the codepoint of the DCI Transmission Configuration Indication field. The codepoint to which the TCI State is mapped is determined by its ordinal position among all the TCI States with Ti field set to 1, i.e. the first TCI State with Ti field set to 1 shall be mapped to the codepoint value 0, second TCI State with Ti field set to 1 shall be mapped to the codepoint value 1 and so on. The maximum number of activated TCI states is 8;

-	R: Reserved bit, set to 0.

Figure 6.1.3.14-1: TCI States Activation/Deactivation for UE-specific PDSCH MAC CE

6.1.3.15	TCI State Indication for UE-specific PDCCH MAC CE

The TCI State Indication for UE-specific PDCCH MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits;

-	TCI State ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to 0, this field indicates a TCI-StateId for a TCI state of the first 64 TCI-states configured by tci-States-ToAddModList and tci-States-ToReleaseList in the PDSCH-Config in the active BWP. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID. The length of the field is 7 bits.

Figure 6.1.3.15-1: TCI State Indication for UE-specific PDCCH MAC CE

6.1.3.16	SP CSI reporting on PUCCH Activation/Deactivation MAC CE

The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated;

-	R: Reserved bit, set to 0.

Figure 6.1.3.16-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE

Figure 6.1.3.9-2: Multiple Entry PHR MAC CE with the highest ServCellIndex of Serving Cell with configured uplink is equal to or higher than 8

6.1.3.17	SP SRS Activation/Deactivation MAC CE

The SP SRS Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a variable size with following fields:

-	A/D: This field indicates whether to activate or deactivate indicated SP SRS resource set. The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	SRS Resource Set's Cell ID: This field indicates the identity of the Serving Cell, which contains activated/deactivated SP SRS Resource Set. If the C field is set to 0, this field also indicates the identity of the Serving Cell which contains all resources indicated by the Resource IDi fields. The length of the field is 5 bits;

-	SRS Resource Set's BWP ID: This field indicates a UL BWP as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9], which contains activated/deactivated SP SRS Resource Set. If the C field is set to 0, this field also indicates the identity of the BWP which contains all resources indicated by the Resource IDi fields. The length of the field is 2 bits;

-	C: This field indicates whether the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) are present. If this field is set to 1, the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) are present, otherwise they are not present;

-	SUL: This field indicates whether the MAC CE applies to the NUL carrier or SUL carrier configuration. This field is set to 1 to indicate that it applies to the SUL carrier configuration, and it is set to 0 to indicate that it applies to the NUL carrier configuration;

-	SP SRS Resource Set ID: This field indicates the SP SRS Resource Set ID identified by SRS-ResourceSetId as specified in TS 38.331 [5], which is to be activated or deactivated. The length of the field is 4 bits;

-	Fi: This field indicates the type of a resource used as a spatial relationship for SRS resource within SP SRS Resource Set indicated with SP SRS Resource Set ID field. F0 refers to the first SRS resource within the resource set, F1 to the second one and so on. The field is set to 1 to indicate NZP CSI-RS resource index is used, and it is set to 0 to indicate either SSB index or SRS resource index is used. The length of the field is 1 bit. This field is only present if MAC CE is used for activation, i.e. the A/D field is set to 1;

-	Resource IDi: This field contains an identifier of the resource used for spatial relationship derivation for SRS resource i. Resource ID0 refers to the first SRS resource within the resource set, Resource ID1 to the second one and so on. If Fi is set to 0, and the first bit of this field is set to 1, the remainder of this field contains SSB-Index as specified in TS 38.331 [5]. If Fi is set to 0, and the first bit of this field is set to 0, the remainder of this field contains SRS-ResourceId as specified in TS 38.331 [5]. The length of the field is 7 bits. This field is only present if MAC CE is used for activation, i.e. the A/D field is set to 1;

-	Resource Serving Cell IDi: This field indicates the identity of the Serving Cell on which the resource used for spatial relationship derivation for SRS resource i is located. The length of the field is 5 bits;

-	Resource BWP IDi: This field indicates a UL BWP as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9], on which the resource used for spatial relationship derivation for SRS resource i is located. The length of the field is 2 bits;

-	R: Reserved bit, set to 0.

Figure 6.1.3.17-1: SP SRS Activation/Deactivation MAC CE

6.1.3.18	PUCCH spatial relation Activation/Deactivation MAC CE

The PUCCH spatial relation Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 24 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	PUCCH Resource ID: This field contains an identifier of the PUCCH resource ID identified by PUCCH-ResourceId as specified in TS 38.331 [5]. The length of the field is 7 bits;

-	Si: If there is a PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i as specified in TS 38.331 [5], configured for the uplink bandwidth part indicated by BWP ID field, Si indicates the activation status of PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i, otherwise MAC entity shall ignore this field. The Si field is set to 1 to indicate PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i shall be activated. The Si field is set to 0 to indicate PUCCH Spatial Relation Info with PUCCH-SpatialRelationInfoId i shall be deactivated. Only a single PUCCH Spatial Relation Info can be active for a PUCCH Resource at a time;

-	R: Reserved bit, set to 0.

Figure 6.1.3.18-1: PUCCH spatial relation Activation/Deactivation MAC CE

6.1.3.19	SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE

The SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	A/D: This field indicates whether to activate or deactivate indicated SP ZP CSI-RS resource set. The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint value of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	SP ZP CSI-RS resource set ID: This field contains an index of sp-ZP-CSI-RS-ResourceSetsToAddModList, as specified in TS 38.331 [5], indicating the Semi Persistent ZP CSI-RS resource set, which shall be activated or deactivated. The length of the field is 4 bits;

-	R: Reserved bit, set to 0.

Figure 6.1.3.19-1: SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE

6.1.3.20	Recommended bit rate MAC CE

The Recommended bit rate MAC CE is identified by a MAC subheader with LCID as specified in Tables 6.2.1-1 and 6.2.1-2 for bit rate recommendation message from the gNB to the UE and bit rate recommendation query message from the UE to the gNB, respectively. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.20-1):

-	LCID: This field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits;

-	Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink;

-	Bit Rate: This field indicates an index to Table 6.1.3.20-1. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate;

-	R: reserved bit, set to 0.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field

| Index | NR Recommended Bit Rate value [kbit/s] | Index | NR Recommended Bit Rate value [kbit/s] |
| --- | --- | --- | --- |
| 0 | Note 1 | 32 | 700 |
| 1 | 0 | 33 | 800 |
| 2 | 9 | 34 | 900 |
| 3 | 11 | 35 | 1000 |
| 4 | 13 | 36 | 1100 |
| 5 | 17 | 37 | 1200 |
| 6 | 21 | 38 | 1300 |
| 7 | 25 | 39 | 1400 |
| 8 | 29 | 40 | 1500 |
| 9 | 32 | 41 | 1750 |
| 10 | 36 | 42 | 2000 |
| 11 | 40 | 43 | 2250 |
| 12 | 48 | 44 | 2500 |
| 13 | 56 | 45 | 2750 |
| 14 | 72 | 46 | 3000 |
| 15 | 88 | 47 | 3500 |
| 16 | 104 | 48 | 4000 |
| 17 | 120 | 49 | 4500 |
| 18 | 140 | 50 | 5000 |
| 19 | 160 | 51 | 5500 |
| 20 | 180 | 52 | 6000 |
| 21 | 200 | 53 | 6500 |
| 22 | 220 | 54 | 7000 |
| 23 | 240 | 55 | 7500 |
| 24 | 260 | 56 | 8000 |
| 25 | 280 | 57 | Reserved |
| 26 | 300 | 58 | Reserved |
| 27 | 350 | 59 | Reserved |
| 28 | 400 | 60 | Reserved |
| 29 | 450 | 61 | Reserved |
| 30 | 500 | 62 | Reserved |
| 31 | 600 | 63 | Reserved |
| Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given. |  |  |  |

Next change

6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel |
| 33-46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) |
| 1–32 | Identity of the logical channel |
| 33–51 | Reserved |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |