3GPP TSG-RAN WG2 Meeting #105bis	R2-1903647

# Xi’an, China, 8th  – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | NNN | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NR RSRP/RSRQ/SINR value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 04/08/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The IEs RSRP-RangeNR, RSRQ-RangeNR are used for both measurements and thresholds. Currently, also for thresholds, a value range is defined. This should be a single value for threshold. For SINR (IE RS-SINR-RangeNR), a single value is defined. But it is unclear the single value is used for threshold only. We propose to reword the sentences as in 38.331 to make the definition clearly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NR RSRP, RSRQ, and SINR measurement metrics are modified according to the counterpart in 38.331 (to align with TS 38.331). Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: NR measurement Inter-operability: The CR only impact UE behavior and the network does not have to implement it. If the UE implements the CR but the network does not, there is no Inter-operability issue. It may result in (slightly) different timing of sending measurement report but it does not cause problem to legacy network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear which value should be used as a NR threshold value. It may result in (slightly) different timing of sending measurement report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on draft 36.331-f50 |  |  |  |  |  |  |  |  |

### 6.3.5	Measurement information elements

<Skip unrelated Parts>

#### –	RSRP-RangeNR

The IE RSRP-RangeNR specifies the value range used in RSRP measurements and thresholds. For measurements, integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 [84]. For thresholds, the actual value is (IE value – 156) dBm, except for the IE value 127, in which case the actual value is infinity.

RSRP-RangeNR information element

-- ASN1START

RSRP-RangeNR-r15 ::=						INTEGER (0..127)

-- ASN1STOP

<Skip unrelated Parts>

#### –	RSRQ-RangeNR

The IE RSRQ-RangeNR specifies the value range used in RSRQ measurements and thresholds. For measurements, integer value for RSRQ measurements according to Table 10.1.11.1-1 in TS 38.133 [84]. For thresholds, the actual value is (IE value – 87) / 2 dB.

RSRQ-RangeNR information element

-- ASN1START

RSRQ-RangeNR-r15 ::=			INTEGER (0..127)

-- ASN1STOP

<Skip unrelated Parts>

#### –	RS-SINR-RangeNR

The IE RS-SINR-RangeNR specifies the value range used in RS-SINR measurements and thresholds. For measurements, integer value for RS-SINR measurements is according to Table 10.1.16.1-1 in TS 38.133 [84].  For thresholds, the actual value is (IE value – 46) / 2 dB.

RS-SINR-RangeNR information element

-- ASN1START

RS-SINR-RangeNR-r15 ::=					INTEGER (0..127)

-- ASN1STOP